3GPP TSG-SA5 Meeting #163	S5-254536

Wuhan, China, 13. - 17. October 2025

Source:	Ericsson Telecom S.A. de C.V.

Title:	PCR TR 28.822 Rel-20 5GA Sustainability use case and requirements

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

Current AI/ML management specifications in SA5 do not include any way to represent or access sustainability-related energy information or the resources used in AI/ML model training and inference.

Although TS 28.310 already specifies energy information allowing operators to configure energy attributes for managed entities, there is no explicit use case or requirements that connect this information to AI/ML training and inference functions.

This contribution proposes to add use case and requirements for Sustainable aspects of ML model training and inference.

Proposed Changes

* * * First Change * * * *

# X Use Cases

## X.1 Sustainable aspects of ML model training and inference

### X.1.1 Description

AI/ML model training and inference can require substantial computing power and therefore significant energy. 
 For sustainability goals and environmental reporting in AI/ML management, it is valuable for 3GPP Management System to reflect the type of energy powering the managed entities and resources used for ML model training and AI/ML inference (for example, renewable and/or carbon related information).The operator is responsible for handling this energy related information in the 3GPP management system, based on knowledge of the infrastructure it manages. In particular:

- the operator obtains energy related information typically based on data from external energy suppliers or infrastructure owner; the operator sets, maintains and updates the association of energy related information to managed entities that support ML model training and inference, using the configuration mechanisms defined in TS 28.310 [1].

- the operator defines access control rules/policies to determine what Managed entities (e.g. NFs, AI/ML functions) can gain access to this information.

In the context of ML model training and AI/ML inference, the MnS producer can read the operator-configured energy related information from the 3GPP management system, subjected to operator-defined access control. By becoming an authorized consumer of this information, the MnS producer can support interpreting training/inference requests containing energy related information. In case of consumer-initiated requests, the MnS consumer is the operator.

In the context of ML model training, the MnS producer can also expose the operator-configured energy related information to other training functions, for the case these training functions need to know this information for selection criteria, as it happens in Federated Learning and Distributed Training.

It is the operator who must guarantee the reliability and trustworthiness of this information. The MnS producer merely relies on operator-configured information and uses it when selecting functions for model training or inference. The MnS producer has no technical means to verify whether the information is correct or up to date.

### X.1.2 Potential Requirements

| REQ-ML_SUST -01 | The 3GPP Management System should allow the operator to configure and maintain the energy related information for the managed entities representing a ML model training function. | Sustainable aspects of ML model training and Inference (clause X.1.1) |
| --- | --- | --- |
| REQ-ML_SUST -02 | The 3GPP Management System should expose the operator-configured energy information for the managed entities used by ML model training function to authorised MnS consumers. | Sustainable aspects of ML model training and Inference (clause X.1.1) |
| REQ-ML_SUST -03 | The 3GPP Management System should allow the operator to configure and maintain the energy information for the managed entities representing an AI/ML model inference function. | Sustainable aspects of ML model training and Inference (clause X.1.1) |
| REQ-ML_SUST -04 | ML Training Function should allow for the MnS consumer request for training with consideration to energy related information criteria. | Sustainable aspects of ML model training and Inference (clause X.1.1) |

* * * End of Changes * * * *