3GPP TSG-SA5 Meeting #163	S5-254597

# Wuhan, China, 13. - 17. October 2025

Source:	Ericsson

Title:	Pseudo-CR on TR 28.881 Add  description and requirements for intent guarantee UC#8

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR defined guaranteed requirements as a new feature to be added to the TS 28.312 specification. It explained how guaranteed requirements can be used by MnS Consumers to obtain agreed fulfilment of intents and intents aspects with forward-looking predictability. It also introduces key concepts that are necessary for the modelling of guaranteed requirements, such as confidence level, guaranteed period and guaranteed validity.

The pCR also proposes a solution to the basic modelling of guaranteed requirements.

This pCR is related to WT-3.1.

Proposed Changes

* * * First Change * * * *

## 4.8 Use case #8: Support to express guarantee requirements in an intent

### 4.8.1 Description

An intent defines one or more requirements, goals and constraints that should be ensured for the intent to be considered fulfilled. The requirements are evaluated against the state of the network during the intent fulfilment feasibility check. Even if the result is feasible, no guarantees are explicitly provided by the MnS producer to ensure that the requirements will be continuously fulfilled over time. While the MnS producer may find a solution that meets the requirements when the intent is accepted, it may later degrade due to over-allocation, faults, or other issues.

When the IDMS is used for communication service assurance in a specific time window, the MnS consumer should be allowed to express the request for guaranteed requirements as part of the intents. Such requirements should be considered for forward-looking sustained compliance. Then, the MnS Producer can utilize different methods (e.g., statistical modelling, digital twin simulation, resource partitioning, redundancy, prioritization, resource reservation, etc.) to guarantee the intent fulfilment in the future.

Additionally, after an MnS Producer has classified an intent as “FEASIBLE” following the initial feasibility check, subsequent changes in network resource can render the intent “INFEASIBLE” before intent fulfilment is requested without actions taken for guarantee. This temporal inconsistency, caused by the absence of continuous feasibility validation, will lead to fulfilment failures, undermining the reliability of intent-based operations and degrading overall network performance.

Guaranteed requirements are associated with a confidence level that expresses the probability of failing in fulfilling the requirements. The confidence level is specified by the MnS consumer as a probability threshold. How to express confidence may depend on the context of the intent, e.g. for a connectivity service-related intent, the confidence may be expressed as service availability and service reliability. How the MnS producer will provide the confidence level to the requirements is implementation specific, which may include methods such as prioritization, resource reservation, resource partitioning, redundancy, etc.

Since guarantees may not extend indefinitely, the MnS consumer may specify a guarantee period to define the planning horizon. The guarantee period begins when an intent is created or updated, and it is implicitly renewed and re-evaluated at the end of each period.

The guarantee may have its own validity which can be different from the validity of the associated requirements. The guarantee validity can be used to ensure requirements are met (with a certain confidence level) during specific periods, such as business hours, weekdays, etc., or to enable/disable the guarantee over time according to different criteria. A guarantee is only considered when it is valid. And a guarantee is not considered when the associated requirement is not valid.

* * * Next Change * * * *

### 4.8.2 Potential requirements

REQ-IDMS_Guarantee-1: The intent driven MnS producer should have the capability to allow MnS consumer to express guarantee requirements in an intent.

REQ-IDMS_Guarantee-2: The intent driven MnS producer should have the capability to allow MnS consumer to specify which requirements in an intent should be guaranteed.

NOTE: The guaranteed requirements can be a subset of or all requirements in an intent.

REQ-IDMS_Guarantee-3: The intent driven MnS producer should have the capability to allow MnS consumer to specify the confidence level of the guaranteed requirements.

REQ-IDMS_Guarantee-4: The intent driven MnS producer should have the capability to allow MnS consumer to specify the guarantee period.

REQ-IDMS_Guarantee-5: The intent driven MnS producer should have the capability to allow MnS consumer to specify the guarantee validity.

REQ-IDMS_Guarantee-6: The intent driven MnS producer should have the capability to report to the MnS consumer about the compliance with guaranteed requirements.

### * * * End of Changes * * * *