3GPP TSG-SA WG6 Meeting #68	S6- 253377

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Samsung

Title:	New key issue on traffic identification and differentiated QoS for multiplexed media flows

Spec:	3GPP TR 23.700-51 v0.2.0

Agenda item:	9.5

Document for:	Approval

Contact:	jerry.bae@samsung.com

1. Introduction

This pCR is proposed to introduce a New key issue on traffic identification and differentiated QoS for multiplexed media flows

2. Reason for Change

This key issue is mainly related to the objective c) in FS_XRM_Ph3_APP.

c.	End to end support for multiplexed data flows, including multiplexed media flows identification and information provisioning in application enabler layer based on (S)RTP Multiplexed Media Identification Information or additional packet filter, providing application enabler layer differentiated QoS service for multiplexed data flows,etc.

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 and enables differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection.

In order to request differentiated QoS for multiplexed media flows, the AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information in the Flow Description and the specific QoS requirements.

(S)RTP Multiplexed Media Identification Information may include a combination of at least one of Synchronization Source (SSRC), Payload Type (PT), RTCP SDES item Media Identification (MID) and RTP SDES header extension for MID and RTCP packet type as defined in 5.7.6.2 of TS 23.501.

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter.

If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE.

Therefore, if the VAL server does not provide (S)RTP Multiplexed Media Identification Information and specific QoS requirements, the SEALDD server should be able to provide those informations.

This pCR proposes new key issue and the related open issues for study.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-51 v0.2.0.

* * * First Change * * * *

4.x	Key Issue #X: traffic identification and differentiated QoS for multiplexed media flows

4.x.1	Description

Traffic identification and differentiated QoS for multiplexed media flows is as defined in clause 5.37.11 of TS 23.501 and enables differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection.

In order to request differentiated QoS for multiplexed media flows, the AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information in the Flow Description and the specific QoS requirements.

(S)RTP Multiplexed Media Identification Information may include a combination of at least one of Synchronization Source (SSRC), Payload Type (PT), RTCP SDES item Media Identification (MID) and RTP SDES header extension for MID and RTCP packet type as defined in 5.7.6.2 of TS 23.501.

If a UE indicates support of (S)RTP Multiplexed Media Identification Information in the IP packet filter, it shall indicate this to the SMF at PDU Session establishment and the SMF provides this UE capability information to the PCF. The PCF considers such UE capability information when providing the PCC rules to the SMF.

If (S)RTP Multiplexed Media Identification Information is received in the PCC rule, the SMF provides (S)RTP Multiplexed Media Identification Information to the UPF in the corresponding PDR(s) and also to the UE in the QoS Rule if the UE has indicated that it supports (S)RTP Multiplexed Media Identification Information in IP packet filter.

If the UE has not indicated so, the SMF shall not provide the (S)RTP Multiplexed Media Identification information in the QoS Rule to the UE.

Therefore, if the VAL server does not provide (S)RTP Multiplexed Media Identification Information and specific QoS requirements, the SEALDD server should be able to provide those informations.

4.x.2	Open Issue

Based on the above description, the following aspects need to be studied:

- Whether and how the SEALDD service can provide (S)RTP Multiplexed Media Identification Information and specific QoS requirements.

- Whether and how the application enabling layer support for differentiated QoS in multiplexed media flows scenarios.