3GPP SA WG6 Meeting #69	S6-254115

# Oct. 13 – 17th, 2025; Wuhan, China.

Source: 	China Mobile

Title: 	KI#3 Solution: Provision and monitor AIoT device presence

Document for: 	Approval

Agenda Item: 	9.8

Work Item / Spec.:	FS_AmbientIoT_Ph2_APP / TR 23.700-26 v0.1.0

Abstract of the contribution: This document proposes a solution to address the KI#3 (TR 23.700-026, i.e., provision and monitor AIoT device presence.

## 1	Introduction

This pCR proposes a solution to address the KI#3, i.e., provision and monitor AIoT device presence. The solution is based on both the enhanced architecture of the application enabler layer for the support of the AIoT App service as proposed in the pCR S6-254112 and the latest SA2 Ambient IoT architecture, as well as the agreement of supporting the exposure of AIoT device location information to AF as in 3GPP TS 23.369 [3].

Thanks to the exclusive support of the application enablement layer at server side for the Ambient IoT App service, any provisioning and monitoring operations will be handled by the application enabler AIoTApp that is newly defined in the pCR S6-254112. The VAL server communicates with the App enabler AIoTApp by providing the parameter ‘location information requested’. Then, AIoTApp interacts with the 5GS NF AIOTF, as a trusted or untrust AF, to retrieve the location information of an AIoT device. AIoTApp can thus track the presence of the device as well as its trajectory.

## 2	Reason for Change

For the AIoT service operation requests defined in 3GPP TS 23.369 [3], i.e., Inventory and Command, there exist certain scenarios for which the AIoT application enabler can enhance the operations.

For example, a 3rd-party application server (e.g., VAL server) may collect the AIoT device information (e.g., location information) via inventory service request for an AIoT device originally located in a first target area. This AIoT device may be moved to a second target area for any reason. When the 3rd-party application server issues inventory service request for the same AIoT device to the first target area, the failure to retrieve the AIoT device information will be generated. For this scenario, the AIoTApp application enabler may provide a remediation mechanism by searching within alternate targeted area(s) so as to pinpoint the up-to-date location information of the AIoT device. The 5GC does not provide such remediation mechanisms.

In another scenario, some application servers (e.g. a logistics company) may need to regularly track the locations of AIoT devices (e.g. the goods in the truck) and identify the (moving) trajectory of the AIoT devices, based on which warnings could be sent to the AIoT devices in case they deviate from the right/expected trajectory. Recently, the SA2 WG has reached agreement to support the exposure of AIoT device location information to AF as in 3GPP TS 23.369 [3].

In summary, it would be of great advantage if the Ambient IoT App enabler could help monitor the presence as well as track the trajectory of AIoT devices.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-26 v0.1.0.

# * * * * First change * * * *

## 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| …… |  |  |  |  |
| Sol #X |  |  | X |  |

# * * * * next change * * * *

## 7.x	Solution #x: Provision and monitor AIoT device presence

### 7.x.1	Solution Architecture

This solution is based on an independent architecture that is newly defined in the pCR S6-254112. In the architecture, an AIoTApp application enabler is defined and acts as a SEAL layer function entity. This function entity can be deployed and implemented by integrating with other related service enabler functions (e.g., SEALDD server, NSCE server) for the support of the Ambient IoT application services. The following Fig. 7.x.1-1 is included for completeness. Please reference the associated pCR S6-254112 for more detailed explanation of the solution architecture.

Figure 7.x.1-1: On-network functional model for AIoTApp service

### 7.x.2	Solution description

For the AIoT service operation requests defined in 3GPP TS 23.369 [3], i.e., Inventory and Command, there exist certain scenarios for which the AIoT application enabler can enhance the operations.

For example, a 3rd-party application server (e.g., VAL server) may collect the AIoT device information (e.g., location information) via inventory service request for an AIoT device originally located in a first target area. This AIoT device may be moved to a second target area for any reason. When the 3rd-party application server issues inventory service request for the same AIoT device to the first target area, the failure to retrieve the AIoT device information will be generated. For this scenario, the AIoTApp application enabler may provide a remediation mechanism by searching within alternate targeted area(s) so as to pinpoint the up-to-date information of the AIoT device. The 5GC does not provide such remediation mechanisms.

In another scenario, some application servers (e.g. a logistics company) may need to regularly track the locations of AIoT devices (e.g. the goods in the truck) and identify the (moving) trajectory of the AIoT devices, based on which warnings could be sent to the AIoT devices in case they deviate the right/expected trajectory. Recently, the SA2 WG has reached agreement to support the exposure of AIoT device location information to AF as in 3GPP TS 23.369 [3].

Therefore, it would be of great advantage if the Ambient IoT App enabler could help monitor the presence as well as track the trajectory of AIoT devices.

Based on the functional model for AIoTApp service as in the Fig. 7.x.1-1, the application enablement architecture layer is proposed to enhance at the server side. The VAL server communicates with the AIoTApp enabler via the AIoTApp-S reference point, which further communicates with the underlying 3GPP network systems via either the N33 (via NEF for an untrusted AF), or the interface off the 5GC AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF. The enhanced provisioning and monitoring operations will be handled by the AIoTApp enabler.

A VAL server provides to the App enabler AIoTApp the parameter ‘location information requested’, along with other parameters. Then, AIoTApp interacts with the 5GS NF AIOTF to retrieve the location information of an AIoT device. Accordingly, AIoTApp can track the presence of the device as well as its trajectory. AIoTApp enabler may also request location information based on its local configuration.

AIoTApp can send concurrent service requests to multiple AIOTFs in 5GC to optimize the operations.

### 7.x.3	Procedure

### 7.x.3.1	Provision and monitor AIoT device presence

Figure 7.x.3.1-1 illustrates the procedure for the AIoT application enabler to interact with the 5GS for the support of the Ambient IoT App service that leverages the corresponding 5GC NF services and APIs as specified in the TS 23.369 [3]. The VAL server sends the ‘location information request’ indication to AIoTApp, along with the potential monitoring schedule. The AIoTApp sends the received request, or based on local configuration, to the 5GC. Once the requested AIoT device information (e.g., location information) is received from the 5GC, the AIoTApp enabler can do more enhanced information processing (e.g., presence verification, trajectory tracking) before sending the information to the VAL server (i.e., AIoT service consumer).

Pre-condition:

-	The VAL server discovers and selects the AIoTApp enabler by CAPIF functions.

-	Multiple AIOTFs may be discovered and selected in the 5GS.

-	AIOTFs have determined to provide the AIoT device location information to AF based on operator policy.

Figure 7.x.3.1-1: Provision and monitor AIoT device presence

### The VAL server decides to use the AIoT application enabler (AIoTApp) to collect the AIoT device information and allocates address/port as AIoTApp-S Data transmission connection information for receiving the AIoT data packets from the AIoTApp enabler. The VAL server sends the AIoT_app_service request to the AIoTApp enabler. The types of AIoT requests can be Inventory and Command. The service request includes VAL server ID, VAL service ID, AIoTApp-S Data transmission connection information of the VAL server side, and the AIoT service related parameters (e.g., AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], location information requested). The VAL server may also send AIoT device provisioning and monitoring parameters (e.g., device trajectory tracking request, monitoring schedule (e.g., every hour for 24 hours), device-id/a group of device-IDs). The details of AIoT service related parameters are described in TS 23.369 [3].

As specified in the clause 5.8 of TS 23.369 [3], Information about the target AIoT Device(s) may include Filtering Information that might be constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information). These components can be used by the AIoTApp enabler to identify a particular AIoT device or a group of devices.

### 2.	Upon receiving the request, the AIoTApp enabler performs an authorization check. If authorization is successful, the AIoTApp enabler allocates the AIoTApp-S data transmission connection information (e.g., address/port) of AIoTApp enabler side to receive the AIoT data packets from the VAL server to be delivered to the 5GC. The AIoTApp enabler responds with the AIoT_app_service response.

### NOTE 1:	The AIoTApp-S data transmission connection information of the AIoTApp enabler side is optional, if the AIoTApp enabler uses the downlink pull mode to fetch the AIoT data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the AIoT data to the address provided by VAL server.

### After the AIoTApp enabler receives from the VAL server both the AIoT service related parameters and AIoT device provisioning and monitoring parameters, depending on the local configuration, the AIoTApp enabler sends AIoT service requests to 5GC, with the indication ‘location information requested’.

### The AIoTApp may send concurrently the requests for a group of AIoT device-IDs to multiple discovered and selected AIOTF NFs in 5GC.

### Depending on the trustiness of the AF (i.e., AIoTApp enabler), the AIoTApp invokes either Nnef_AIoT_Inventory/Command (being an untrusted AF) or Naiotf_AIoT_Inventory/Command (being a trusted AF) service operations towards the (selected) AIOTF(s), as in TS 23.369 clause 6.2.

### There could be multiple AIOTF NFs selected from the step-3. for concurrent AIoT device monitoring operations.

### NOTE 2:	All the invoked service operations, together with the service parameters, conform to what have been specified in TS 23.369. E.g., depending on the trustiness of the AF (i.e., AIoTApp), AIoTApp sends either External Target Area information (to NEF as an untrusted AF) or Target Area information (to AIOTF as a trusted AF).

### NOTE 3:	The AF (i.e., AIoTApp) can send the parameter ‘location information requested’ to AIOTF in 5GC to request the AIOTF to report the location information of AIoT devices.

### The 5GS processes the AIoT service request(s) and collects the AIoT device information based on the provisioned service parameters, or local configuration. The AIOTF can report the AIoT device location information to the AF (i.e., AIoTApp) as described in the TS 23.369 [3].

### (Multiple) AIOTF(s) send collected AIoT device information to AIoTApp (or via NEF). This information may include the location information of an AIoT device, depending on operator policy and the requested parameter from the AIoTApp.

### 7.	The AIoTApp enabler processes the received AIoT device information, e.g.

### If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp receives the device information for the AIoT device, then AIoTApp reports the presence to the VAL server.

### If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp does not receive the device information for the AIoT device from 5GC, then AIoTApp may expand the service request in more Target Areas that have been received in step-1 from the VAL server. The steps 4-6 of the procedure will be repeated to monitor the presence of the AIoT device.

### If the VAL server has requested the AIoTApp to track the trajectory of an AIoT device, then AIoTApp will record the received location information of the AIoT device. Based on the monitoring scheduled received in step-1, the AIoTApp enabler may repeat the steps 4-6 of the procedure to continue tracking the location of the device. When the monitoring schedule is fulfilled, the AIoTApp can send the tracked trajectory of the AIoT device to the VAL server.

### The AIoTApp enabler sends the processed AIoT device information via the AIoT_app_service notification to the VAL server.

### 7.x.3.2	Information flows

### 7.x.3.2.1	AIoT App Service request

Table 7.x.3.2.1-1 describes the information flow from the VAL server to the AIoTApp enabler for the support of the AIoT App service.

Table 7.x.3.2.1-1: AIoT App service request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| AIoTApp-S Data transmission connection information | M | Address/port and/or URL of the VAL server to receive the AIoT data packets from the AIoTApp. |
| AIoT service parameters | M (Note 1) | AIoT service parameters (e.g., AF ID, (external) target area info, location information requested, AIoT device filtering info., etc.). |
| AIoT monitoring schedule | O | AIoT service request schedule based on which AIoT service requests are sent to 5GC. |
| DL AIoT service request delivery status subscription indication | O | Indicates the VAL server expected to receive the DL AIoT service request delivery status notification |
| NOTE 1: The AIoT service parameters are specified in the 3GPP TS 23.369 [3]. |  |  |

### 7.x.3.2.2	AIoT App service response

Table 7.x.3.2.2-1 describes the information flow from the AIoTApp enabler to the VAL server to respond to the AIoT App service request.

Table 7.x.3.2.2-1: AIoT App service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| AIoTApp-S information Data transmission connection information | O | Address/port and/or URL of the AIoTApp enabler to receive the AIoT data packets from the VAL server to be delivered to 5GC. |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g., AF ID not supported. |
| NOTE: The IE is only present if the Result is failure. |  |  |

### 7.x.3.2.3	AIoT App service notification

Table 7.x.3.2.3-1 describes the information flow from the AIoTApp enabler to the VAL server to notify the events related to the AIoT App service.

Table 7.x.3.2.3-1: AIoT App service notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event of the AIoTApp enabler interaction status with 5GC for AIoT App services, e.g., device present, location & trajectory tracking, no collected AIoT device info., etc. |
| VAL service ID | O | Identity of the VAL service. |
| DL AIoT service request delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL AIoT App service request |
| > Collected AIoT device info. | O | Indicates the collected AIoT device information (raw, location, etc.) |
| > AIoT device tracking info. | O | Indicates tracked trajectory, presence monitoring results of AIoT device info |
| NOTE: Either Event ID IE or the DL AIoT service request delivery instruction ID is present. |  |  |

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *