3GPP SA WG6 Meeting #69	S6-254138

# Oct. 13 – 17th, 2025; Wuhan, China.

Source: 	China Mobile

Title: 	KI#5 Solution: traffic identification and differentiated QoS for multiplexed media flows

Document for: 	Approval

Agenda Item: 	9.5

Work Item / Spec.:	FS_XRM_Ph3_APP / TR 23.700-51 v1.0.0

Abstract of the contribution: This document proposes a solution to address the KI#5 (TR 23.700-51), i.e., traffic identification and differentiated QoS for multiplexed media flow.

## 1	Introduction

This pCR proposes a solution to address the KI#5, i.e., traffic identification and differentiated QoS for multiplexed media flow. The solution targets at enhancing the SEALDD layer to identify the multiplexed media flow with provisioned QoS differentiation for the uplink (UL) direction of the XR application.

## 2	Reason for Change

The 3GPP TS 23.501 clause 5.37.11 describes how to achieve the traffic identification and enable differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection. In the TS 23.503 clause 6.1.3.27.7, it states that, in order for PCF to determine and provide differentiated QoS for multiplexed media flows, AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information in the Flow Description and the specific QoS requirements.

The AF requests for the multiplexed media flows are for either uplink or downlink direction. While the request for the downlink direction does not have UE dependency, the request on the uplink direction depends on the UE capability information that is indicated via the support for (S)RTP Multiplexed Media Identification Information in the IP Packet Filters sent to SMF, and then to PCF, at PDU sessions establishment, as in TS 23.501 clause 5.37.11.

The TS 23.503 clause 6.1.3.27.7 elucidates that, when the AF request is for uplink direction and the UE has indicated its support for (S)RTP Multiplexed Media Identification, the PCF will generate PCC rules and send to SMF, which then sends the corresponding PDRs to UPF and also QoS rules to UE. In this (uplink) scenario, while the AF (i.e., SEALDD server) knows the information related to multiplexed media flows and their corresponding QoS requirements, the SEALDD client on the UE side does not have the information for the potential optimization to the end-to-end multiplexed media-flow service (for uplink processing). This solution proposes to enhance the SEALDD layer such that the SEALDD server can coordinate with the SEALDD client to apply the per media flow traffic identification and specific QoS requirements for the uplink direction.

Note that the solution assumes that the VAL server will send to the SEALDD server the (S)RTP Multiplexed Media Identification Information and the specific QoS requirements. In the case the VAL server does not provide the information to the SEALDD server, the SEALDD server may determine the information according to VAL service ID for different service types of XR application traffic, like the step 2 of the clause 9.2.2.2 in TS 23.433.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-51 v1.0.0.

# * * * * First change * * * *

## 5	Solutions

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |  |
| Sol #2 |  | x |  |  |  |
| Sol #3 |  |  | x |  |  |
| Sol #4 |  |  |  | x |  |
| Sol #5 | x |  |  |  |  |
| Sol #6 | x |  |  |  |  |
| Sol #X |  |  |  |  | X |

# * * * * next change * * * *

## 5.x	Solution #x: Traffic identification and differentiated QoS for multiplexed media flows

### 5.x.1	Architecture

### This solution is based on the SEALDD architecture as in 3GPP TS 23.433 [6].

### 5.x.2	Solution description

The 3GPP TS 23.501 clause 5.37.11 describes that, to achieve the traffic identification and enable differentiated QoS handling for (S)RTP media flows that are multiplexed on the same end-to-end transport layer connection, AF has to provide per media flow the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement for either uplink or downlink direction. The request for the uplink direction depends on the UE capability information that is indicated via the support for (S)RTP Multiplexed Media Identification Information in the IP Packet Filters sent to SMF, and then to PCF, at PDU sessions establishment, as in TS 23.501 clause 5.37.11.

The TS 23.503 clause 6.1.3.27.7 elucidates that, when the AF request is for uplink direction and the UE has indicated its support for (S)RTP Multiplexed Media Identification, the PCF will generate PCC rules and send to SMF, which then sends the corresponding PDRs to UPF and also QoS rules to UE. In this (uplink) scenario, while the AF (i.e., SEALDD server) knows the information related to multiplexed media flows and their corresponding QoS requirements, the SEALDD client on the UE side does not have the information for the potential optimization to the end-to-end multiplexed media-flow service (for uplink processing).

This solution proposes to enhance the SEALDD layer such that the SEALDD server can coordinate with the SEALDD client to apply the per media flow traffic identification and specific QoS requirements for the uplink direction.

Note that the solution assumes that the VAL server will send to the SEALDD server the (S)RTP Multiplexed Media Identification Information and the specific QoS requirements. In the case the VAL server does not provide the information to the SEALDD server, the SEALDD server may determine the information according to VAL service ID for different service types of XR application traffic, like the step 2 of the clause 9.2.2.2 in TS 23.433.

### 5.x.2.1	Enhancement to existing SEALDD procedure

The SEALDD procedure and information flows for multi-modal application as defined in 3GPP TS 23.433 [6] can be enhanced as follows (all changes are in bold-italic format.)

NOTE:	During the phase of normative work, whether the changes as proposed by the solution need to merge with changes from other solution(s) enhancing the same existing procedure will be considered.

9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements, the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement as in the clause 5.37.11 of TS 23.501[3]. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may include the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description and the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE 1: If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. NOTE 2: If in step 1, the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement are provided for multiplexed media flows transported on the same end-to-end transport layer connection, then the SEALDD client repeats steps 6-8 for each multiplexed media flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

# * * * * next change * * * *

### 5.x.2.2	Enhancement to existing SEALDD information flows

9.12.3.1	SEALDD enabled multi-modal transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service.

Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| >QoS information | O | QoS information provided by VAL server |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| > Multiplexed media flow info | O | The multiplexed media flow info includes the (S)RTP Multiplexed Media Identification Information and the specific QoS requirement as specified in the clause 5.37.11 of TS 23.501[3]. |

9.12.3.3	SEALDD multi-modal transmission connection establishment request

Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal transmission connection establishment.

Table 9.12.3.3-1: SEALDD multi-modal transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flows info | M | Information of the list of SEALDD-UU flows. |
| > SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| > SEALDD traffic descriptor | M | SEALDD traffic descriptor for the flow of the SEALDD client side used to establish SEALDD multi-modal XR connection. |
| >> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >> Transport layer protocol | M | Indicates the transport layer protocol. |
| >> Multiplexed media flow | O | Indicates the (S)RTP Multiplexed Media Identification Information |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |

9.12.3.4	SEALDD multi-modal transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the multi-modal transmission connection establishment.

Table 9.12.3.4-1: SEALDD multi-modal transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow info | M | Information of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| > Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal service connection. |
| > SEALDD-UU flows list | M | Indicates the list of SEALDD-UU flows. |
| >> SEALDD-UU flow ID | M | Identity of the SEALDD-UU flow. |
| >> SEALDD traffic descriptors | M | SEALDD traffic descriptors for the flows of the SEALDD server side used to establish SEALDD multi-modal connection. |
| >>> Addressing info | M | Indicates the addressing information (e.g. address, port, URL). |
| >>> Transport layer protocol | M | Indicates the transport layer protocol. |
| >>> Multiplexed media flow | O | Indicates the (S)RTP Multiplexed Media Identification Information |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

### 5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *