3GPP TSG-SA WG6 Meeting #69	S6-254168

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	New solution for KI#1 enhancement on API service to support for energy saving

Spec:	3GPP TR 23.700-44

Agenda item:	9.9

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This contribution proposes to address the following open issues in KI#1,

-	Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

2. Reason for Change

As indicated in TS 22.261, the energy consumption and energy efficiency have the following requirements:

Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption for serving this 3rd party.

Based on operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party.

Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time.

As indicated in TS 23.501, 5GC shall support for exposing energy consumption information as following description:

The Energy Information Function (EIF) is defined to:

- expose the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF).

In SA6, CAPIF is applicable to any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves. Cosiderting that energy consumption has becomging a more and more important KPI for 5G low carbon evolution. It is benefit to involve energy consumption information in CAPIF API management.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44.

* * * First Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #x | x |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.222: " Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs;".

* * * Next Change * * * *

6.x	Solution #x: Enhancement on API service to support for energy saving

6.x.1	Solution Description

This solution addresses the KI#1, enhancing CAPIF to support for API service registration and selection based on energy consumption information. CAPIF is applicable to any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves. Cosiderting that energy consumption has becomging a more and more important KPI for 5G low carbon evolution. It is benefit to involve energy consumption information in CAPIF API management.

6.x.2	Architecture impacts

This solution is based on architecture of CAPIF as described in 3GPP TS 23.222 [x].

6.x.2.1	Impact to existing publish service API procedures

The publish service API procedures and information flows in 3GPP TS 23.222 [x] can be enhanced (highlighted in bold italics) as follows.

8.3 Publish service APIs 8.3.1 General The CAPIF supports publishing service APIs by the API provider. The API publishing function can be within PLMN trust domain or within 3rd party trust domain. 8.3.2 Information flows 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 2); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods, energy efficiency information (Energy consumption information, energy saving information).. Shareable information; O (see NOTE 1); Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained.. NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs of 3GPP AEFs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19].. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use.. 8.3.2.2 Service API publish response Table 8.3.2.2-1 describes the information flow service API publish response from the CAPIF core function to the API publishing function. Table 8.3.2.2-1: Service API publish response Information element; Status; Description. Result; M; Indicates the success or failure of publishing the service API information. Service API published information reference; O (see NOTE); The information which can be used for referencing the information (set) about the published service API by the API publishing function.. Service API information; O (see NOTE); The authorized service API information, which may be a subset or the full set, of the Service API information as specified in Table 8.3.2.1-1.. NOTE: This information element is included when the Result indicates success.. 8.3.3 Procedure Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function. Pre-conditions: 1. Authorization details of the APF are available with the CAPIF core function. 2. API invokers may have subscribed with the CAPIF core function to obtain new service API information. Figure 8.3.3-1: Publish service APIs 1. The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API, including energy efficiency information. If the service API is to be shared to other CAPIF core functions, the shareable information and the CAPIF provider domain information are included. 2. Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information provided by the API publishing function is stored at the CAPIF core function (API registry). 3. The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

6.x.2.2	Impact to existing retrive service API procedures

The retrieve service API procedures and information flows in 3GPP TS 23.222 [x] can be enhanced (highlighted in bold italics) as follows.

8.5 Retrieve service APIs 8.5.1 General The CAPIF supports retrieving the published service APIs information by the API provider. The API publishing function can be within PLMN trust domain or within 3rd party trust domain. 8.5.2 Information flows 8.5.2.1 Service API get request Table 8.5.2.1-1 describes the information flow service API get request from the API publishing function to the CAPIF core function. Table 8.5.2.1-1: Service API get request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API published information reference; M; The information provided by the CAPIF core function which can be for referencing the information (set) about the published service API by the API publishing function.. 8.5.2.2 Service API get response Table 8.5.2.2-1 describes the information flow service API get response from the CAPIF core function to the API publishing function. Table 8.5.2.2-1: Service API get response Information element; Status; Description. Result; M; Indicates the success or failure of retrieving the service API information. Cause; O; The cause for the request failure.. Service API information; O (see NOTE); The service API information as specified in Table 8.3.2.1-1.. NOTE: Shall be present if the Result information element indicates that the service API get request is successful. Otherwise service API information shall not be present.. 8.5.3 Procedure Figure 8.5.3-1 illustrates the procedure for retrieving the service APIs. The service API retrieval mechanism is supported by the CAPIF core function. Pre-condition: 1. Authorization details of the APF are available with the CAPIF core function. Figure 8.5.3-1: Retrieve service APIs 1. The API publishing function sends a service API get request to the CAPIF core function, with service API published information reference provided by the CAPIF core function when the service API was published, including energy efficiency information. 2. Upon receiving the service API get request, the CAPIF core function checks whether the API publishing function is authorized to get published service APIs information. If the check is successful, the CAPIF core function prioritizes service API with low energy consumption, and the corresponding service API information is retrieved from the CAPIF core function (API registry). 3. The CAPIF core function provides a service API get response to the API publishing function which includes the service API information.

6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End Change * * * *