3GPP TSG-SA WG6 Meeting #69	S6-254169

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	New Solution of edge application server discovery considering energy consumption

Spec:	3GPP 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liutangqing (liutangqing@chinamobile.com)

lvhuazhang (lvhuazhang@chinamobile.com)

1. Introduction

This pCR proposes new solution for FS_EnergySys_Ph2_APP.

2. Reason for Change

This paper proposes to give the solution to solve the key issue #1 and key issue #3 to enhance the edge application service discovery procedure.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #x | x |  | x |  |  |  |
| Sol #2 |  |  |  |  |  |  |

* * * Second Change, all new text * * * *

6.x	Solution #x: Edge Application Server Discovery by Considering Energy Consumption and Energy Efficiency

6.x.1	Solution Description

The proposed solution solves the problem listed in KI#1 and KI#3 and proposes to consider energy efficiency and energy consumption in edge computing.

6.x.1.1	Definition of Energy Efficiency (EE), Energy Consumption (EC) and Renewable energy

As indicated in TS 28.310[4], the energy consumption and energy efficiency have the following definitions:

Energy Efficiency (EE): ratio between performance and energy consumption.

NOTE 8: the performance may be measured based on e.g. data volume, latency, number of active users, etc.

Energy Consumption (EC): integral of power consumption over time.

NOTE 9: see ETSI ES 202 706-1 [23].

As indicated in TS 28.310[4], the renewable energy and renewable energy factor have the following definitions:

Renewable energy: energy from renewable non-fossil sources.

NOTE 15:	This definition is taken from 3GPP TS 22.261 [31].

NOTE 16:	Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 17:	A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [33].

NOTE 18:	Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [33].

Renewable energy factor: ratio of the renewable energy to the total energy.

NOTE 19:	This definition is taken from ISO/IEC 30134-3:2016 [33].

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

6.x.1.2	Solution description

In Edge Computing deployment, an application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host service may use a single IP address (anycast address) or different IP addresses. To start such a service, the UE needs to know the IP address(es) of the Application Server(s) serving the service. The UE may do a discovery to get the IP address(es) of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server and service latency, traffic routing path and user service experience can be optimized.

But, in the current design of EDGEAPP in SA6, there exists the following issues:

Firstly, during the EAS discovery procedure, that the Energy Efficiency or Energy Consumption of EAS is not considered. If one EAS has the higher Energy Efficiency or lower Energy Consumption, this EAS has the following benefits:

-	Lower energy consumption or higher energy efficiency can improve the response speed and processing capacity of applications, allowing users to enjoy a better experience.

-	The low power consumption and lower heat generation feature of servers enhance the stability and reliability of server operation, effectively reducing service interruptions caused by hardware failures.

-	Lower energy consumption or higher energy efficiency in application server leaves sufficient space for future service expansion and maintaining a high-quality service level, for example, if user performs high energy consumption service like XRM rendering.

Secondly, during the EAS discovery procedure, that the renewable energy of EAS is not considered. In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. For example, some servers can be powered by electricity converted from renewable energy to reduce the carbon emission. Considering the renewable energy can be recycled and regenerated in nature, if the UE has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

Also, the EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

6.x.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of enabling edge applications defined in section 6.2 of TS 23.558[y]. But the following enhancements are added:

-	The EES subscribes the Energy Efficiency and Energy Consumption of EAS. And the EES selects the lower Energy Consumption or higher Energy Efficiency related EAS for EEC.

-	The EES can subscribe to EAS whether renewable is consumed by EAS. If the EAS consumes the renewable energy, for example in the morning time that solar energy can be consumed, the EES may select this EAS to save the energy.

-	The EEC can indicate to the EES to select the lower Energy Consumption or higher Energy Efficiency EAS, or indicate to select the EAS that consumes the renewable energy.

-	The EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

6.x.3	Procedure

6.x.3.1	Procedure of subscription Energy Consumption information from EAS

Figure 6.x.3.1-1: EAS energy consumption information subscription

1.	The EES sends an EAS energy consumption information subscription request to the EAS. The EAS energy consumption information subscription request includes the EES ID along with the security credentials, Event ID, time period and energy threshold to subscribe to information about energy consumption information of EAS.

The energy consumption information includes the Energy Efficiency (EE), Energy Consumption (EC) and Renewable energy of EAS. The Renewable energy of EAS includes that either the indication of the EAS consumes renewable energy or not, or the Renewable energy factor of EAS.

The time period indicates the EAS to notify the Energy Consumption information of EAS in the certain time period in the future.

The energy threshold indicates the value of energy consumption information. The energy threshold can be the value of Energy Efficiency (EE), the value of Energy Consumption (EC), the value of Renewable energy factor and etc.

NOTE: For example, the energy consumption of EAS on standby status is 150W. And if the EAS reaches the energy threshold, the EAS notifies the EES.

2.	Upon receiving the request from the EES, the EAS checks if the EEC is authorized to subscribe for information of the requested EAS(s).

3.	If the processing of the request was successful, the EAS sends an EAS energy consumption information subscription response to the EES, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire.

4.	The EAS sends an EAS energy consumption information notification to the EES with the EAS energy consumption information determined in step 1.

If the energy threshold is provided in step 1, the EAS sends the notification to EES according to the comparison between the value of energy threshold and the value of energy consumption information of EAS.

6.x.3.2	Procedure of EAS discovery considering energy consumption information

The procedure listed here are based on the procedure defined in section 8.5.2.2 of TS 23.558[y].

Figure 6.x.3.2-1: EAS Discovery procedure considering energy consumption information

1.	The same as step 1 in section 8.5.2.2 but with the enhancement that the EAS discovery request includes also the indication and energy threshold.

The indication indicates the EES to discover or select the most suitable EAS that has lower energy consumption or higher energy efficiency, or support renewable energy.

If the indication indicates to select the EAS that support renewable energy, it means that either the EAS supports the renewable energy supply or the Renewable energy factor of the EAS exceeds the energy threshold.

If the indication indicates to select the EAS that lower energy consumption or higher energy efficient, it means that the value of EE exceeds the energy threshold or the EC lower than the energy threshold.

NOTE: For example, if EAS 1 has higher energy efficient compared to EAS 2, that EAS 1 is selected by EES. Or if the energy efficient exceeds energy threshold, this EAS can be selected as candidate EAS.

2.	The same as step 2 in section 8.5.2.2.

3.	The same as step 1 in section 8.5.2.2 but with the enhancement that the EAS discovery response includes the discovered EASs that may satisfy the conditions below:

The energy consumption of the EAS is lower than the energy threshold in step 1, and this EAS is selected.

The energy efficiency of the EAS is higher than the energy threshold in step 1, and this EAS is selected.

The Renewable energy factor of the EAS is higher than the energy threshold in step 1, and this EAS is selected.

The EAS starts to consume the renewable energy as energy supply, and this EAS is selected.

6.x.3.3	Procedure of QoS update related to renewable energy consumption

Figure 6.x.3.3-1: Procedure of QoS update related to renewable energy consumption

0.	The same as the procedure defined in section 8.6.6.2.2 of TS 23.558[y].

1-3.	The same as the procedures defined in section 4.29.1 and 4.29.2 of TS 23.502[x]. The EIF notifies to EAS that the subscribed energy consumption information of certain UE and QoS flow contains renewable energy consumption.

4-6.	The same as step 1-3 of section 8.6.6.2.3 of TS 23.558[y], with the following changes:

The EAS trigger the session with QoS update request to 5GC via EES if either of the following conditions are satisfied:

- Renewable energy consumption information is included in the energy consumption information of certain UE or QoS flow from EIF. This may indicate that renewable energy is consumed for certain UE or QoS flow.

- Another one is the value of renewable energy consumption, for example the total renewable energy consumption or the ratio of renewable energy consumption in the energy consumption information of certain UE or QoS flow exceeded the energy threshold configured in EAS.

The QoS parameters in step 4 which are defined as described in clause 6.1.3.22 of TS 23.503 [z], should be improved compared with the QoS parameters in step 0.

NOTE x: For example, the UL-maximum bitrate and DL-maximum bitrate in step 4 can be larger than in step 1 to improve the user experience when consuming renewable energy.

6.x.4	Solution evaluation

Editor’s note: This part is for further update.

* * * Third Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Energy Consumption (EC): As defined by the TS 28.310 [4].

Energy Efficiency (EE): As defined by the TS 28.310 [4].

Renewable Energy: As defined by the TS 28.310 [4].

Renewable energy factor: As defined by the TS 28.310 [4].

* * * Fourth Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[x]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[y]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[z]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS)".

* * * End of Changes * * * *