3GPP TSG-SA WG6 Meeting #69	S6-254190

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New solution on architecture and functional model for AIoT services

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution for architecture/functional model and related functionalities for AIoT services.

2. Reason for Change

As the AIoT devices are the new type of IoT devices which are different from the previous ones (e.g., MTC or PIN) and need the different functionalities for the 5G system, the previous application enablers (like PINAPP) couldn’t support the new type and new functionalities of AIoT devices. So it’s necessary to propose a new architecture and functional model to support the AIoT devices/services.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.x	Solution #x: New architecture and functional model for AIoT services

7.x.1	Solution description

This solution addresses the KI#1. The following procedures introduce the on-network and off-network functional model for AIoT services, also mention the functional entities and reference points in the new architecture.

This solution aligns with the network architecture for AIoT services which specified in clause 4.2 of 3GPP TS 23.369 [3].

7.x.1.1	On-network Functional model for AIoT services

Figure 9.2.2-1 illustrates the generic on-network functional model for AIoT services.

Figure 7.x.1.1-1: On-network functional model for AIoT services

The AIoT management client provides the support for AIoT management functions to the VAL client(s) over AM-C reference point. The VAL server(s) communicate with the AIoT management server over AM-S reference point. The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

The AIoT management server, acting as an AF, may obtain AIoT data information from the 3GPP core network via NEF (over N33 reference point) which is defined in clause 4.5.4 of 3GPP TS 23.369 [3]. If the AIoT management server is a trusted AF, it can interact with AIOTF directly for AIoT related service as defined in clause 4.5.5 of 3GPP TS 23.369 [3].

In this release, the AIoT devices can interact with 3GPP network system (via NG-RAN reader) which is out of scope of present document.

7.x.1.2	Off-network Functional model for AIoT services

Figure 7.x.1.2-1 illustrates the off-network functional model for AIoT services.

Figure 7.x.1.2-1: Off-network functional model for AIoT services

The AIoT management client of the UE1 communicates with the AIoT management client of the UE2 over the AM-PC5 reference point.

7.x.1.3	Functional entities description

AMS (AIoT Management Server):

The AMS performs the following functionalities to support AIoT services:

- Interact with 3GPP core network via NEF to obtain the AIoT device data, acting as an untrusted AF;

- Interact with AIOTF in 3GPP core network directly to obtain the AIoT device data, acting as a trusted AF;

- Enrich the processing of AIoT service requests and the collected Ambient IoT related data;

- Provide the value-added information of AIoT devices to the VAL servers or 3rd parties;

- Provision the AIoT devices monitoring information;

- Expose the AIoT device monitoring information to the the VAL servers or 3rd parties;

- Acting as CAPIF's API exposing function as specified in 3GPP TS 23.222 [23222].

NOTE 1: The functionalities for the AMS may update according to the approved solution and final conclusions.

AMC (AIoT Management Client):

The AMC performs the following functionalities to support AIoT services:

- Acting as the application client for AIoT services;

- Interact with the corresponding AMC between the two UEs.

NOTE 1:	The functionalities for the AMC may update according to the approved solutions and final conclusions.

7.x.1.4	Reference points description

AM-C:

The interactions related to AIoT management functions between the VAL client(s) and the AIoT management clients within a VAL UE are supported by AM-C reference point.

AM-S

The interactions related to AIoT management functions between the VAL server(s) and the AIoT management servers are supported by AM-S reference point.

AM-S reference point is used by the VAL server to request and receive the AIoT device related information from AIoT management server.

AM-S reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [23222].

N33

The interactions between the AIoT management server and the NEF in the 3GPP core network are supported by N33 reference point which is defined in clause 4.5.4 of 3GPP TS 23.369 [3].

N1

The interactions between the UE and the AMF in the 3GPP core network are supported by N1 reference point which is defined in clause 4.2.7 of 3GPP TS 23.501 [23501].

AM-PC5

The interactions related to AIoT management functions between the AIoT management clients located in different VAL UEs are supported by AM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [23303].

7.x.2	Architecture Impacts

This solution proposes a new architecture and functional mode for AIoT services.

7.x.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for ambient power-enabled IoT, Stage 1".

[3]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things, Stage 2".

[4]	3GPP TR 23.700-30: "Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”.

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[23303]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[23501]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

* * * Next Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

AIoT Device: An Ambient IoT device is an IoT device powered by energy harvesting, with limited energy storage capability.

* * * Next Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

AIoT	Ambient IoT

AIOTF                Ambient IoT Function

* * * End of Change * * * *