3GPP TSG-SA WG6 Meeting #69	S6-254212

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0403 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Include power saving configuration within the NRM services for IoT devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph4-APP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution #11 addressed the technical gap#1 in TR 23.700-35 on new SEAL service for power saving in IoT devices is agreed to be considered in normative phase. This paper proposes to include procedures which allow VAL server/VAL client to influence the power saving configuration. However, it is up to the 3GPP network to decide the final configurations. to add the content in Solution #11 to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the information elements and related procedures in which the VAL server/ VAL client indicates the desired power saving configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEAL service for power saving in IoT devices is missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, and the following new clauses:14.3.2.a, 14.3.2.b, 14.3.2.c, 14.3.2.d, 14.3.x, 14.3.x.1, 14.3.x.2, and 14.3.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[40]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[41]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[42]		3GPP TS 28.533: "Management and orchestration; Architecture framework".

[43]		3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[44]		3GPP TS 28.532: "Management and orchestration; Generic management services".

[45]		3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[46]		3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[47]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[48]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[49]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[50]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[51]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[52]	3GPP TS 23.256 "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[53]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[54]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[55]	Void

[56]	3GPP TS 23.482 "Functional architecture and information flows for AIML Enablement Service".

[57]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[58]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial map and Spatial anchors".

[59]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital Assets".

[60]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS) ".

[61]	3GPP TS 22.156: "Mobile Metaverse Services".

[38.331]	3GPP TS 38.331: " NR; Radio Resource Control (RRC) protocol specification".

# * * * * Second change * * * *

#### 14.3.2.a	Power saving assist request

Table 14.3.2.a-1 describes the information flow for power saving assist request from the VAL server to the NRM server, or from the NRM client to the NRM server.

Table 14.3.2.a-1: Power saving assist request

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided |
| VAL Service ID | M | The identity of the VAL service |
| Power saving requirements | M | The desired power saving requirements for the VAL UE(s), e.g., power saving mode (e.g., power-saving, balanced, performance) to be enforced for the target user(s) |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

#### 14.3.2.b	Power saving configuration request

Table 14.3.2.b-1 describes the information flow for power saving configuration request sent from the NRM server to the NRM client

Table 14.3.2.b-1: Power saving configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Requested power saving configuration | M | The desired power saving configuration to be updated for the target user, e.g., DRX and eDRX configurations in 3GPP TS 38.331 [38.331] clause 6.3, clause 6.2, respectively. |

#### 14.3.2.c	Power saving configuration response

Table 14.3.2.c-1 describes the information flow for power saving configuration response sent from the NRM client to NRM server.

Table 14.3.2.c-1: Power saving configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE which is a target for power savings configuration |
| Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| Accepted power saving configuration | O (NOTE) | Indicates the accepted power saving configuration |
| NOTE: This information element is present if the accepted power saving configurations are different from the desired one. |  |  |

#### 14.3.2.d	Power saving assist response

Table 14.3.2.d-1 describes power saving assist response sent from the NRM server to the VAL server and from NRM sever to the NRM client.

Table 14.3.2.d-1: Power saving assist response

| Information element | Status | Description |
| --- | --- | --- |
| VAL User or VAL UE(s) ID(s) (NOTE) | M | The identity of the VAL user whose UE(s) are target for power savings configuration. Alternatively, is the UE ID on which the NRM client resided. |
| >Result | M | Indicates whether the result of the power saving configuration request is successful or not |
| >Accepted power saving configuration | O | Indicates the accepted power saving configuration |
| NOTE: If the consumer is the NRM client, this IE shall be the UE ID. If the consumer is VAL server, the IE may be a list of UE IDs or identity of VAL user. |  |  |

# * * * * Third change * * * *

### 14.3.x	Power saving configuration

#### 14.3.x.1	General

This clause discusses the possibility for the VAL server or VAL client (via the NRM server or NRM client, respectively) to influence the power saving configurations of a VAL UE(s), e.g., by configuring their DRX and eDRX parameters, through e.g., CpProvisioning, NpConfiguration and/ or MonitorEvent APIs to meet the application requirement, e.g., IoT application requirements.

For this purpose, the NRM server may be configured with DRX and eDRX parameters that influences different power saving modes, which for example extend the battery lifetime.

NOTE 1:	It is up to the PLMN operator to determine which parameters the NRM server may request to reconfigure the VAL UEs.

NOTE 2:	It is up to the PLMN operator to accepted, modify, or reject the requested configurations by the NRM server.

The power saving configurations may either be triggered by the VAL server as described in clause 14.3.x.2, or by the VAL UE as described in clause 14.3.x.3.

#### 14.3.x.2	VAL server triggers power saving configuration

Figure 14.3.x.2-1 describes the procedure when the VAL server triggers power saving assist service for power saving in IoT devices.

Figure .14.3.x.2-1: Procedure for VAL server triggers power saving assist service

1.	VAL server requests the NRM Server to assist power saving of the VAL user(s) that are part of VAL Service. The request contains the list of VAL UEs and VAL Users that are targets for the power saving, VAL service ID, power saving requirements, e.g., mode (economy, balanced, performance).

2.	NRM server determines, based on the VAL service ID, UE(s) access type and power saving mode, the power saving configuration, network parameters and communication pattern parameters for each requested VAL UE(s). Furthermore, the SEAL Server may determine the VAL UE ID(s) associated with the VAL user(s) included in step 1.

3.	NRM Server interacts with 3GPP CN (e.g., CpProvisioning, NpConfiguration and/or MonitorEvent APIs) to provision the VAL UE(s) network parameters and communication pattern parameters determined in step 2.

4.	NRM Server interacts with NRM Client with recommended power saving configuration for the UE (e.g., the determined power saving configuration in step 2).

5.	The target UE interacts with 3GPP network as per clause 4.2.2.2.2 of 3GPP TS 23.502 [11], to indicate the desired power saving configuration, e.g., DRX or eDRX configuration. The UE receives from 3GPP RAN the accepted power saving configuration.

NOTE:	It is up to the 3GPP system to determine the final DRX configurations to be reconfigured in the VAL UE.

6.	NRM Client responds to the NRM Server with power saving configuration response and indicates the accepted power saving configuration.

7.	NRM Server responds to the VAL Server with power saving assist response.

#### 14.3.x.3	VAL client triggers power saving configuration

Figure 14.3.x.3-1 describes the procedure when the VAL client triggers power saving assist service for power saving in IoT devices.

Figure 14.3.x.3-1: Procedure for VAL client triggers power saving assist service

0.	VAL Server may interact with VAL UE(s) that are part of VAL Service for power saving request.

1.	NRM Client requests the NRM Server to assist power saving of the VAL UE(s). The NRM client indicates the desired power saving configuration to the NRM Server.

2.	Steps 2-6 as in Figure 14.3.x.2-1.

3.	The NRM Server responds to the NRM Client with power saving assist response.

4.	 The VAL UE may interact with VAL Server for power saving response.

# * * * * End of changes * * * *