3GPP TSG-SA WG6 Meeting #69	S6-254244

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0325 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Functional model description to support 3RD party API providers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 6.2.1 defines the functional model description to support multiple API Provider domains. However, it is unclear whether an onboarded API Invoker (with the received onboarding credetntials) is able to enroll to service APIs offered by API Provider 1 and API Provider 2, as indicated by the SLA between the CAPIF Provider domain and the API Provider domains, or requires separate onboarding procedures per API Provider domain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification in clause 6.2.1 of API invoker enrolling services from API Provider domain 1 and API Provider domain 2 with the same API Onbarding procedure, that is, with the same API Invoker. Correction of API Invoker Onboarding procedure, to indicate that the onboarding can contain enrolled APIs from multiple API Provider domains. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 8.1.3, 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.2.1	Functional model description to support 3rd party API providers

Figure 6.2.1-1 shows the functional model for the CAPIF to support 3rd party API providers.

Figure 6.2.1-1: Functional model for the CAPIF to support 3rd party API providers

The CAPIF core function in the PLMN trust domain supports service APIs from both the PLMN trust domain and the 3rd party trust domain having business relationship with PLMN. The API invokers may exist within the PLMN trust domain, or within the 3rd party trust domain or outside of both the PLMN trust domain and the 3rd party trust domain. The API provider domain 1 offers the service APIs from the PLMN operator. The API provider domain 2 offers the service APIs from the 3rd party. When the 3rd party API provider is a trusted 3rd party of the PLMN, the API provider domain 1 may also offer the service APIs from the 3rd party (i.e., trusted 3rd party APIs are exposed in same way as own API provider domain APIs, so the API provider domain 2 in the figure above would not apply).

The API invoker 2 within the PLMN trust domain onboards and interacts with the CAPIF core function via CAPIF-1, and invokes the service APIs in the PLMN trust domain via CAPIF-2 and invokes the service APIs in the 3rd party trust domain via CAPIF-2e. The API invoker 3 within the 3rd party trust domain onboards and interacts with the CAPIF core function via CAPIF-1e, and invokes the service APIs in the PLMN trust domain via CAPIF-2e and invokes the service APIs in 3rd party trust domain via CAPIF-2. The API invoker 1 from outside the PLMN trust domain and 3rd party trust domain, onboards and interacts with the CAPIF core function via CAPIF-1e and invokes the service APIs in the PLMN trust domain and the service APIs in the 3rd party trust domain via CAPIF-2e. The CAPIF core function consolidates in the API invoker profile the service API enrollment information for the service APIs in the PLMN trust domain and the service APIs in the 3rd party trust domain, as specified in clause 8.1.3.

The API exposing function, the API publishing function and the API management function of the API provider domain 1 within the PLMN trust domain interacts with the CAPIF core function via CAPIF-3, CAPIF-4 and CAPIF-5 respectively. The API exposing function, the API publishing function and the API management function of the API provider domain 2 within the 3rd party trust domain interacts with the CAPIF core function in the PLMN trust domain via CAPIF-3e, CAPIF-4e and CAPIF-5e respectively. The API exposing function within the PLMN trust domain and the 3rd party trust domain provides the service APIs to the API invoker, offered by the respective trust domains.

The interactions between the API exposing functions within the PLMN trust domain is via CAPIF-7 (not shown in the figure 6.2.1-1 for simplicity). The API exposing function within the PLMN trust domain interacts with the API exposing function in the 3rd party trust domain via CAPIF-7e.

NOTE 1:	The communication between the API exposing function and the CAPIF core function, between the API publishing function and the CAPIF core function and between the API management function and the CAPIF core function over CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e respectively can be API based.

The detailed information of the APIs provided by the CAPIF core function is specified in clause 10.

NOTE 2:	The security aspects of CAPIF reference points are under SA3 responsibility and out of scope of the present document.

* * * Second Change * * * *

### 8.1.3	Procedure

Figure 8.1.3-1 illustrates the procedure for onboarding the API invoker to the CAPIF. The security aspects of this procedure are specified in subclause 6.1 of 3GPP TS 33.122 [12].

Pre-conditions:

1.	The API invoker is not a recognized user of the CAPIF.

2.	The API invoker has visibility to APIs information (e.g., API catalogue or dashboard - central place for the API provider to manage which APIs are displayed, giving API invokers the ability to enroll for).

Figure 8.1.3-1: Procedure for onboarding the API invoker to the CAPIF

1.	For enrolment of the API invoker to be a recognized user of the CAPIF, the API invoker triggers onboard API invoker request towards the CAPIF core function, providing the information as required for the API management and criteria information as required to onboard the API invoker.

2.	The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker, and further initiates a grant process. If the onboard API invoker request message includes onboard criteria information, then the CCF verifies whether the CCF can support the onboard criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the onboard criteria information from the API Invoker. Successful onboarding results in provisioning API invoker profile which includes identity for the API invoker. The authorization information and the list of APIs, for the applicable API Provider domains, and the categories of APIs, for the applicable API Provider domains, that the API invoker can access subsequent to successful onboarding may also be created, considering the network slice information, if applicable. The CAPIF core function may create access control policy (see Table E-1) for the onboarded API invoker considering the network slice information.

NOTE 1:	Completion of onboarding process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

NOTE 2:	The API invoker profile consists of at least the identity information for the API invoker, information required for the authentication and authorization by the CAPIF (e.g., application service information such as application service provider and application identifier, enrolled service APIs) and the CAPIF identity information.

3.	If the API invoker has triggered the onboard API invoker request and is granted permission, the onboard API invoker response provides success indication including information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

4.	As a result of successful onboarding process, the CAPIF core function is able to authenticate and authorize the API invoker.

* * * Fourth Change * * * *

### 8.2.3	Procedure

Figure 8.2.3-1 illustrates the procedure for offboarding the API invoker from the CAPIF, triggered by the API invoker. The security aspects of this procedure are specified in subclause 6.8 of 3GPP TS 33.122 [12].

Pre-conditions:

1.	The API invoker has been onboarded as a recognized user of the CAPIF.

Figure 8.2.3-1: Procedure for offboarding the API invoker from the CAPIF

1.	The API invoker triggers offboard API invoker request to the CAPIF core function, providing the information as required for the API management.

2.	The CAPIF core function cancels the enrollment of the service APIs for the API invoker from CAPIF. The API invoker ceases to be a recognized user of the CAPIF. All the authorizations corresponding to the API invoker are revoked from CAPIF. Optionally, the information of the API invoker may be retained at the CAPIF core function as per the operator policy.

NOTE:	Completion of offboarding process can require explicit notification to the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the de-provisioning process internally without the need of explicit grant by the CAPIF administrator.

3.	The CAPIF core function returns the offboard API invoker response providing successful offboarding indication.

* * * End of Change * * * *