3GPP TSG-SA WG6 Meeting #69	S6-254246

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0327 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Open Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.38.3 specifies that the CAPIF core function is aware of the service API information that can be disseminated when a requestor invokes the Open Discovery service API call. However, it is unspecified how the CAPIF core function is aware of it, i.e., it is missing the CAPIF Administration task that allows to restrict the service APIs available for Open Discovery or the service API information available for Open Discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of CAPIF Administrator provided Open Discovery policies to the CCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to identify how the CCF can obtain awareness of the service API information available for Open Discovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.38.1, 8.38.3, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.38.1	General

The following procedure in this subclause corresponds to supporting open discover service APIs. Such open discovery operation allows requestor to discover service API information (e.g., API name, API provider name, API category) about the available set of APIs offered by CCF before onboarding.

This procedure may be invoked by different types of API invokers (i.e., application management client, hosted application) at different phases (e.g., hosted application development).

* * * Second Change * * * *

### 8.38.3	Procedure

Figure 8.38.3-1 illustrates the procedure for open discover service APIs.

The open service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The requestor is not a recognized user of the CAPIF.

2.	The CAPIF core function is configured with the Service API information that can be disseminated (e.g. Open Discovery policy to restrict discovery to category of APIs) according to the requestor who is not a recognized user of the CAPIF.

Figure 8.38.3-1: Open Discover service APIs

1.	The requestor sends an open service API discover request to the CAPIF core function. It may include the query information from the requestor.

NOTE:	The security aspects of this procedure are unspecified in this release of the specification.

2.	Upon receiving the open service API discover request, if query information is included, the CAPIF core function retrieves the stored service API(s) information as per the query information in the open service API discover request, otherwise all stored service API(s) information is retrieved. Further, since the requestor is not a recognized user of CAPIF, the CAPIF core function applies the Open Discovery policy and performs filtering of service APIs information.

3.	The CAPIF core function sends an open service API discover response to the requestor with the filtered list of service API information.

* * * Third Change * * * *

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
|  | > Network slice identification |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| Subclause 8.38 | List of service API Open Discovery restrictions |
|  | > Service API identification |
|  | > Service API category |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. |  |

* * * End of Change * * * *