3GPP TSG-SA WG6 Meeting #69	S6-254250

Wuhan, China 13th – 17th October 2025	(revision of S6-254xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#1, API invoker enrolled Service API notification

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#1 for TR 23.700-43.

2. Reason for Change

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain, based e.g. on the termination of the service agreement between an API Invoker and an API Provider.

The API invoker status can change: onboarding/offboarding. Within the onboarding status, it may change the enrollment information for the list of enrolled APIs. However, the CAPIF API events API is not able to convey these changes of the API invoker status, which could be useful for the API Invoker, in case of revocation of API Invoker’s enrolled service APIs by CAPIF Administrator, and for the API Provider domain, in case of API Invoker is modifying the list of enrolled APIs.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.1.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | … |
| --- | --- | --- | --- |
| Sol #1 |  |  |  |
| Sol #2 |  |  |  |
| Sol #X. | X |  |  |

* * * Second Change * * * *

## 6.x	Solution #x: API invoker enrolled Service API notification

### 6.x.1	General

This solution relates to KI #1.

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain. E.g., the API invoker decides to terminate the service agreement with the API provider, or the API Provider decides to terminate the service agreement with the API Invoker because e.g., unpaid bills.

With the current level of specification, the API invoker status can change between onboarded/offboarded/onboarding info update. When the API invoker is in the onboarded status, the enrollment information for the list of enrolled APIs may change. However, the CAPIF API events API is not able to convey these changes that may occur while the API invoker is onboarded, which could be useful for the API Invoker, in case of revocation of API Invoker’s enrolled service APIs by CAPIF Administrator, and for the API Provider domain, in case of API Invoker is modifying the list of enrolled service APIs.

This solution proposes a new trigger for the notification of the API invoker’s onbarding update event, that is the update of the list of the API Invoker’s enrolled service APIs. In addition, this solution also proposes to add the list of the enrolled service APIs when the CCF triggers the notification about onboarding update event.

### 6.x.2	Architecture Impacts

This solution proposes an enhancement to the CAPIF_Events API, with the definition of a new trigger for the notification of the API invokers onbarding update event, that is the update of the list of the API Invoker’s enrolled service APIs. In addition, this solution also proposes to be able to add the list of the enrolled service APIs when the CCF triggers the notification about onboarding event. There are no architecture impacts.

### 6.x.3	Solution description

#### 6.x.3.0	General

Figure 6.x.3.0-1 illustrates the procedure for the API provider domain subscribing with the CCF to notification of API Invoker onboarding/offboarding changes, and the notification from the CCF to API provider domain about API Invoker changes.

Figure 6.x.3.0-1: Procedure for API invoker enrolled Service API notification

1.	The API Provider domain subscribes with the CCF to notifications about API Invoker onboarding changes. The API Provider domain may provide the API Invoker information to identify the API invokers to be notified of and may provide as notification filter the service API(s) the API Invoker may be including for enrollment within the onboarding operation.

2.	The CCF determines the already onboarded API invokers.

3.	The CCF returns the response to the API provider subscription request to API Invoker onboarding events. If the API Provider requested immediate notification, the CCF returns the information available for the currently onboarded API Invokers, including the list of enrolled service APIs per API Invoker.

4.	API Invoker X requests onboarding in the CCF and includes a set of service APIs for enrollment.

5.	The CCF returns a successful API Invoker response, including the successfully enrolled APIs.

6.	The CCF notifies the API Provider domain about a new API invoker onboarding, including the successfully enrolled Service APIs for the API Invoker.

7.	The API Provider domain acknowledges the notification.

8.	API Invoker X requests to update the set of enrolled service APIs (disenroll and/or enroll).

9.	The CCF acknowledges the request and returns the successfully enrolled service APIs.

10.	The CCF triggers a notification to the API Provider domain to the API Provider domain indicating the change of the onboarding status and including the updated set of successfully enrolled service APIs.

11.	The API Provider domain returns a notification request acknowledgement to the CCF.

#### 6.x.3.1	Information flows

### 8.8.6	List of CAPIF events

Table 8.8.6-1 provides a non-exhaustive list of CAPIF events.

Table 8.8.6-1: List of CAPIF events

| Events | Events Description |
| --- | --- |
| Availability of service APIs | Availability events of service APIs (e.g. active, inactive) |
| Service API updated | Events related to change in service API information |
| Monitoring service API invocations | Events corresponding to service API invocations |
| API invoker status | Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met, change in the list of successfully enrolled service APIs.) |
| API topology hiding status | Events related to API topology hiding status in CAPIF (created, revoked) |
| System related events | Alarm events providing fault information |
| Performance related events | Events related to system load conditions |
| CAPIF-1/1e interaction events | Events related to CAPIF-1/1e interactions for a service API (e.g. onboarded API invokers) |

### 6.x.4	Solution evaluation

This solution proposes an enhancement to the CAPIF_Events API, with the definition of a new trigger for the notification of the API invokers onbarding update event, that is the update of the list of the API Invoker’s successfully enrolled service APIs, and the definition as filter criteria of the at least one service API that needs to be included in the list of successfully enrolled service APIs. In addition, this solution also proposes to be able to add the list of the successfully enrolled service APIs when the CCF triggers the notification about onboarding update event.

* * * End of Change * * * *

<Proposed change in revision marks>