3GPP TSG-SA WG6 Meeting #69	S6-254252

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0163 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provide the quality of MC client connection to authorized MC users |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Netherlands Police |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MultiHop-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces a procedure for authorised MC service users to request the quality of the connection an MC service UE is utilizing. The MC service UE can either be connected to the 3GPP network directly over the Uu interface or indirectly via an MC service capable 5G ProSe (multihop) UE-to-Network relay over the PC5 interface, as figure 4.2.7.1-1 or figure 4.2.7.1A-1 in 3GPP TS 23.304: The procedure addresses requirements which are captured in 3GPP TS 22.261 in clause 6.9.2.2: The 5G system shall be able to provide indication to a remote UE (alternatively, an authorized user) on the quality of currently available indirect network connection paths. And in 3GPP TS 22.261 clause 6.10.2 The 5G system shall provide a means by which an MNO can inform authorised 3rd parties of changes in the - RAT type that is serving a UE; - cell ID; - RAN quality of signal information; - assigned frequency band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - updating the reference with TS 38.300 - adding a definition of the indirect path information in clause 3.1 - updating the list of abbreviations with the newly introduced ones in this CR - adding a new clause and a related general clause to describe the feature - adding new information elements related to the Quality of MC service client connection request/response - adding a procedure dedicated for direct connection scenario - adding a procedure dedicated for indirect connection scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements in clause 6.9.2.2 and clause 6.10.2 of 3GPP TS 22.261 are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.3, and the following new clauses: 7.x, 7.x.1, 7.x.2, 7.x.3 and 7.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Service (ProSe) in the 5G System (5GS); Stage 2".

[18]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[19]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[20]	3GPP TS 23.479: "UE MBMS APIs for Mission Critical Services".

[21]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[22]	3GPP TS 22.289: "Mobile communication system for railways".

[23]	3GPP TS 23.283: "Mission Critical Communication Interworking with Land Mobile Radio Systems".

[24]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[25]	Void

[26]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[27]	3GPP TS 23.180: "Mission critical services support in the Isolated Operation for Public Safety (IOPS) mode of operation"

[38300]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2."

# * * * * Second change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Indirect path information: Consists of the path information and root relay in the form of User Info IDs of the MC capable 5G ProSe intermediate UE-to-Network relay(s) and the MC capable 5G ProSe multihop UE-to-Network relay, respectively, as defined in 3GPP TS 23.304 [17]. In case the remote MC service UE is directly connected to an MC capable 5G ProSe UE-to-Network relay, the indirect path information consists of the User Info ID of the MC capable 5G ProSe UE-to-Network relay under consideration.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply:

MC service

MC service user

MC service UE

MC system

MC user

MC gateway UE

MC client

For the purposes of the present document, the following terms given in 3GPP TS 23.247 [15] apply:

MBS session

Broadcast MBS session

Multicast communication service

Multicast MBS session

Broadcast communication service

MBS service area

MB-SMF service area

# * * * * Third change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ECN	Explicit Congestion Notification

L4S	Low Latency, Low Loss and Scalable Throughput

NPN	Non-Public Network

NTN	Non-Terrestrial Network

PNI-NPN	Public Network Integrated Non-Public Network

RSC	Relay Service Code

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SNPN	Stand-alone Non-Public Network

SD-RSRP	Sidelink Discovery RSRP

SL-RSRP	Sidelink RSRP

NID	Network Identifier

# * * * * Fourth change * * * *

## 7.x	Reporting the quality of MC service client connection

### 7.x.1	General

This clause describes the procedure to inform authorised MC service user(s) about the connection quality of a target MC service client. As a result, the target MC service client obtains the signal strength measurement, and any necessary information associated with the current connection utilized by the MC service UE and report it back to the MC service server, where the latter provide it to the requested authorised MC service client.

In case the target MC service UE is directly connected to the 5GS over the Uu interface, the MC service client obtains the Uu signal strength measurements from the lower layers of the MC service UE.

NOTE 1:	The interaction between the MC service client and the lower layers of the MC service UE are out of scope of 3GPP.

NOTE 2:	The reported signal strength measurements are RSRP and RSRQ as indicated in 3GPP TS 38.300 [38.300].

In case the target MC service UE is remote and is indirectly connected to the 5GS via an MC service capable 5G ProSe (multihop) UE-to-Network relay over the PC5 interface, the MC service client obtains from the ProSe lower layer of the remote MC service UE the sidelink signal strength measurements along with the indirect path information which comprises of the MC capable 5G ProSe (multihop) UE-to-Network relay and MC capable 5G ProSe intermediate UE-to-Network relay(s) if available. Upon receiving the information, the MC service server requests MC service client(s) comprising the indirect path information to report the quality of their own connection to the MC service server to be provided to the requesting authorised MC service client.

NOTE 3:	The interaction between the MC service client and the ProSe lower layers of the MC service UE are out of scope of 3GPP.

NOTE 4:	The reported sidelink signal strength measurements are SL-RSRP and SD-RSRP as defined in 3GPP TS 38.300 [38.300].

### 7.x.2	Information flows

#### 7.x.2.1	Quality of MC service client connection request

Table 7.x.2.1-1 describes the information flow of the quality of MC service client connection request sent from the MC service client to the MC service server, and from the MC service server to the MC service client.

Table 7.x.2.1-1: Quality of MC service client connection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service identity of the MC service client |
| MC service ID(s) (NOTE 1) | O | The MC service identity of the MC service client(s) to report the quality of connection |
| NOTE 1: This information element is mandatory if the request is sent from an authorised MC service client. |  |  |

#### 7.x.2.2	Quality of MC service client connection response

Table 7.x.2.2-1 describes the information flow of the quality of MC service client connection response sent from the MC service client to the MC service server.

Table 7.x.2.2-1: Quality of MC service client connection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service identity of the MC service client sending the response |
| Sidelink signal strength measurements | O | The measurements of the sidelink signal strength towards the immediate (adjacent) relay the MC service client has obtained from ProSe lower layers. The immediate (adjacent) relay is either the MC service capable 5G ProSe UE-to-Network relay, or the MC service capable 5G ProSe intermediate UE-to-Network relay if available |
| Uu signal strength measurements | O | The Uu signal strength measurements obtained from the MC service if available |
| Measurement time stamp | O | The time stamp associated with the reported measurements. |
| Indirect path information (NOTE 1) | O | (An ordered list of) the User Info ID(s) of the MC service client(s) associated with the current indirect connection, as defined in 3GPP TS 23.304 [17]. |
| Number of hops | O | The current number of hops towards the MC service capable 5G ProSe multihop UE-to-Network relay forming the current indirect path in case of multihop connectivity |
| The maximum number of hops | O | The maximum number of hops supported by the MC service capable 5G ProSe multihop UE-to-Network relay |
| Result (NOTE 2) | O | It indicates the success or failure of the request. |
| NOTE 1: This information element is mandatory for reporting quality of connection of remote MC service UE. NOTE 2: This information element is mandatory when the MC service server sends the quality of MC service client connection response to an authorised MC service client. |  |  |

### 7.x.3	Provide quality of direct connection of an MC service UE

Figure 7.x.3-1 illustrates the procedure to provide an authorised MC service client the quality of direct connection of a certain MC service UE.

Figure 7.x.3-1: Providing the quality of MC service client connection

- The MC service user 1 would like to know the direct connection quality of the MC service UE 2. Therefore, the MC service client 1 sends a quality of MC service client connection request including the MC service ID of MC service client 2.

- The MC service server performs an authorisation check. If the MC service client 1 is not authorised to request the connection quality of MC service client 2, the MC service server provides the MC service client 1 a failure response and the procedure terminates.

- If the authorisation check is successful, the MC service server sends a quality of MC service client connection request to the MC service client 2 at the MC service UE 2.

- The MC service client 2 obtains the signal strength measurements from the lower layers of MC service UE 2.

- MC service client 2 sends the MC service server a quality of MC service client connection response, which includes the obtained measurements from the previous step.

6.	The MC service server stores the signal strength measurements that corresponds to the direct connection of the MC service UE 2.

7.	The MC service server sends the MC service client1 a quality of MC service client connection response including the obtained measurements that corresponds to the direct connection of MC service client 2.

NOTE:	Aspects related to re-sending the quality of MC service client connection request/response to update the signal strength measurements is left to implementation, e.g., after a certain period of time.

### 7.x.4	Provide quality of indirect connection of a remote MC service UE

Figure 7.x.4-1 illustrates the procedure to provide an authorised MC service client the quality of the indirect connection of a certain remote MC service UE.

Precondition:

-	MC service UE 2 is connected to the 5GS via an MC capable 5G ProSe (multihop) UE-to-Network relay.

Figure 7.x.4-1: Providing the quality of a remote MC service client connection

- The MC service user 1 would like to know the connection quality of the indirect path of the MC service UE 2. Therefore, the MC service client 1 sends a quality of MC service client connection request.

- The MC service server performs an authorisation check. If the MC service client 1 is not authorised to request the connection quality of MC service client 2, the MC service server provides the MC service client 1 a failure response and the procedure terminates.

- If the authorisation check is successful, the MC service server sends a quality of MC service client connection request to the MC service client 2 at the MC service UE 2.

- The MC service client 2 obtains the necessary information from the remote MC service UE 2. The information is the sidelink signal strength towards the MC service capable 5G ProSe UE-to-Network relay and the indirect path information associated with the remote MC service UE 2. In case of multihop connectivity, the MC service client 2 obtains the sidelink signal strength towards the immediate hop MC service capable 5G ProSe intermediate UE-to-Network relay, and the indirect path information associated with the remote MC service UE 2. The indirect path information is the (ordered list of) User Info ID the MC service capable 5G ProSe (multihop) UE-to-Network relay and the User Info ID(s) of the MCservice capable 5G ProSe intermediate UE-to-Network relay(s) if available, as described in 3GPP TS23.304 [17]. Furthermore, in case of multihop connectivity, the MC service client 2 may provide the number of hops associated with the indirect path information and the maximum number of hops associated with the MC service capable 5G ProSe multihop UE-to-Network relay under consideration.

- MC service client 2 provides the MC service server the quality of MC service client response, which includes the obtained information in the previous step.

6.	The MC service server stores the information received in the previous step. Furthermore, the MC service server retrieves the (ordered list of) MC service ID(s) that corresponds to the (list of) User Info ID(s) of the indirect path.

7.	The MC service server sends the MC service client at the MC service capable 5G ProSe (multihop) UE-to-Network relay a quality of MC service client connection request. In case of multihop connectivity, the MC service server additionally sends the MC service capable 5G ProSe intermediate UE-to-Network relay(s) a quality of MC service server connection request.

8.	The MC service client(s) obtains the respective sidelink signal strength measurements and the Uu signal strength measurements from the MC service capable 5G ProSe (multihop) UE-to-Network relay and the MC service capable 5G ProSe intermediate UE-to-Network relay(s) if available, respectively.

9.	The MC service client(s) sends the MC service server a quality of MC service client connection response including the respective measurements obtained in the previous step.

10.	The MC service server stores the measurements that correspond to the indirect connection utilized by the remote MC service UE 2.

11.	The MC service server sends the MC service client 1 a quality of MC service client connection response including the obtained indirect path information and the respective measurements.

NOTE:	Aspects related to re-sending the quality of MC service client connection request/response to update the signal strength measurements is left to implementation, e.g., after a certain period of time.

# * * * * End of changes * * * *