3GPP TSG-SA WG6 Meeting #69	S6-254279

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Lenovo

Title:	Solution #12 update

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This contribution provides an update of solution #12 to complete it.

2. Reason for Change

This contribution provides an update of solution #12 to complete it with corresponding APIs and evaluation.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.12	Solution #12: Support for ML model split learning using relays

### 7.12.1	Solution description

This solution addresses Key Issue #4. The solution provides a mechanism for identifying and determining an intermediate entity (UE, edge function) to serve as relay for a split learning operation, and particularly a split AI/ML operation.

Such intermediate entity can be an application layer relay UE which either doesn’t provide any processing and is not aware of the data to relay (encrypted), or 2) intermediate node which undertakes some layers of the split operations (e.g. layers 5-15) instead of the source VAL UE. For both cases, the intermediate entity is expected to be associated with the split operation pipeline as processing entity or just relaying entity.

This solution has two procedures:

- Intermediate node registration procedures to allow the UE (as in clause 7.12.1.1.1) or an edge node (as in clause 7.12.1.1.2) to register its capabilities to ML repository to serve as candidate intermediate node.

- An intermediate node discovery procedure (clause 7.12.1.2) to allow discovering relay node for the split learning operation.

#### 7.12.1.1 Intermediate node registration procedures

##### 7.12.1.1.1 AIMLE client registration

The registration procedure provides enhancement to AIMLE client registration procedure as described in 3GPP TS 23.482 clause 8.7.2.2 (bold italics is new).

----------------------- text from 3GPP TS 23.482 clause 8.7.2.2 (bold italics is new)---------------

8.7.2.2	AIMLE client registration

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection.

For the case that the service is about an AI/ML split learning operation, the AIMLE client requests to register as relay/intermediate node for the service ID. Such indication can be part of the AIMLE client profile as in Table 8.7.3.2-2.

The AIMLE client may also indicate the policies / criteria in which the VAL UE can act as candidate relay/intermediate node. This includes the time and area of validity, a list of supported VAL services/servers or vendors which are part of the Split AIML operation, compute and energy requirements and constraints for the utilization of the VAL UE (e.g. only use as relay if battery is higher than 50% or in low load scenarios or in good channel conditions (e.g. cell centre)). This may also include criteria for the UE mobility or relative mobility, i.e., UEs that move towards the same direction with a speed that allows connectivity could be considered as candidate relay nodes.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository.

The ML repository also stores the association of the AIMLE server with a certain candidate role for a split learning operation. The storage at the repository maps the AIMLE client ID with an ML task/service type (or list of services which support split learning), a role indication and a role type (relay, intermediate processing node).

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| Information on the model split option or topology | O | Information on how the ML model can be split, given that different option may depend on the model type. ML model split option can be equivalent to model split topology. This refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches. Data parallelism involves splitting the training data, while model parallelism involves splitting the model itself. Hybrid approaches combine both data and model parallelism to optimize performance. |
| Split Operation/Learning related information supported role/capability | M | The VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay (A&F, S&F, handle some layers/cuts). |
| > supported role(s) | M | The role of the AIMLE client as relay/intermediate node and the type of relay (A&F, S&F, handle some layers/cuts). |
| > permissions/restrictions | M | The permissions or restrictions (vendor compatibility, access type/RAT, compute limitations) for the use of the AIMLE client as relay/intermediate. |
| > VAL policies for split operation | O | The policies related to the selection of the VAL UE as relay/intermediate node (compute, load, energy constraints and requirements). |
| > time and area of validity | M | The time and geographical/topological area in which the AIMLE client can be used as relay/intermediate node. |
| > List of DNs/VAL servers | M | The list of servers/DNs for which the VAL UE can be utilized in the split operation as relay. |
| >> preference on utilizing the same or different pipelines | O | Indicates a preference from VAL UE on utilizing the same split ML model pipeline or different for the multi-user split operation. |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

--------------------------------------------------------------------------------------------------------------------------------------------

##### 7.12.1.1.2 Split operation node registration

The registration procedure provides enhancement to the Split Operation node registration procedure as described in TS 23.482 clause 8.14.2.4.2 (bold italics is new).

--------------------- text from 3GPP TS 23.482 clause 8.14.2.4.2 (bold italics is new) -----------

8.14.2.4.2	Split operation node registration

Figure 8.14.2.4.2-1 illustrates the procedure for VAL server to register with the AIMLE server to indicate split operation capabilities.

Pre-conditions:

1.	The VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.14.2.4.2-1: Split operation node registration

1.	The VAL server sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server (or from another entity connected with the VAL server indirectly, such as an EAS). The request includes information defined in Table 8.14.3.6-1.

Table 8.14.3.6-1: Split operation node register request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| Node information | M | Information about the VAL server node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | M | Split operation capabilities of the VAL server. |
| > Model information | M | Information about ML model capabilities of the VAL server for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Registrar node information | M | The information of the node to register which can be a server node (EAS, VAL server). |
| >The node of supported role/capability | M | The node capability and availability information for acting as intermediate or relay node for the Split Operation. |
| >>supported role(s) | M | The role of the node as relay or intermediate node and the type of relay (A&F, S&F, handle some layers/cuts). |
| >>permissions/restrictions | O | The permissions or restrictions (vendor compatibility, access type/RAT, compute limitations) for the use as relay/intermediate node. |
| >> VAL policies for split operation | O | The policies related to the selection of the node as relay/intermediate node (compute, load, energy constraints and requirements). |
| >> time and area of validity | M | The time and geographical/topological area in which the node can be used as relay/intermediate node. |
| >> preference on utilizing the same or different pipelines | O | Indicates a preference from the registrar on utilizing the same split ML model pipeline or different for the multi-user split operation. |
| Expiration time | O | The proposed expiration time of the registration. |

2.	Upon receiving the request from the VAL server, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information (including the role of the registered node (VAL server or EAS with respect to the split model operation).

3.	The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

----------------------------------------------------------------------------------------------------------------------------------------------

#### 7.12.1.2 Intermediate node discovery procedure

Pre-conditions:

- A split AIML pipeline is created, and the UE and VAL server has an ongoing connection session for undertaking jointly the split learning operation.

- One or more VAL UEs or edge nodes (e.g. EAS) have registered their capabilities in the ML repository / AIMLE server.

Figure 7.12.1.2-1: Support for split relay operation discovery

1.	A VAL UE detects that a ML model performance is degraded or expected to degrade. The detection can have different flavours either based on UE mobility and channel conditions or due to load/energy status and degradation of the ongoing ML task. More specifically, the detections can be due to:

- VAL client notifies the AIMLE client that the task is delaying or expected to delay.

- AIMLE client utilizes ADAE analytics on VAL session performance (based on TS 23.436) and predicts the downgrade of the link between the VAL UE and the VAL server

- High load or energy consumption at the VAL UE side, due to high app load or lower layer processing

- UE mobility prediction to an area with no coverage or poor coverage or bad channel conditions.

2.	The AIMLE client determines a requirement for updating the pipeline to include a relay/intermediate node and sends a discovery request to the AIMLE server.

The discovery request includes discovery criteria for the selection of the relay, including capability of the VAL UE (apps onboard, workloads supported, processing power and capacity, energy consumption limitations/restrictions), the status of the radio capability of the UE (RATs/spectrum supported, channel conditions, interference limitations), whether the UE is static/low or high mobile UE, whether it supports dual-connectivity (or multi-connectivity), whether it can be connected to the server via non-3GPP means (Wifi, NTN,..), the mobility of the UE or relative mobility to the UE of interest (source UE), etc.

The requirement can include also requested topology/option for the split model operation for which the relay/intermediate nodes need to be discovered. Alternatively, the model type may be provided in the request if there is an association between the type and the topology at the server side (as pre-configuration).

NOTE: Split model topology refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches.

3.	The AIMLE server requests from ML repository and receives the registered nodes (available AIMLE clients, EASs to serve as candidate relays) with their roles as requested at the discovery criteria.

4.	The AIMLE server sends a discovery response to the AIMLE client including the discovered nodes and their capabilities. Such response includes the split learning operation related information such as VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay, the supported split model topologies, VAL policies on the split operation etc.

5.	The AIMLE client after receiving the list of discovered AIMLE clients, it checks whether the other VAL UEs in the list are active and in close proximity. Hence it sends a groupcast/broadcast or unicast message via AIMLE-PC5 to all listed AIMLE clients, to receive reporting on the availability and conditions related to the sidelink interface. The other AIMLE clients can provide response if available and the source AIMLE client will establish a connection/session with them if they fulfil the application layer KPI (e.g. latency for the handshake is less than a pre-defined threshold).

Additionally, the sidelink status check can be in form of requesting from the target UEs additional /supplementary information on the channel status, load, energy and mobility information locally. The status check can be also using ADAE analytics for VAL UE-to-UE session performance stats/predictions as specified in TS 23.436.

6.	The AIMLE client of the VAL UE of interest (source UE) selects one of the candidate intermediate/relay nodes (edge or UE based relays) given criteria which can be equivalent to the discovery criteria and the performance KPI for the split learning operation. This may be based on local or VAL UE or server policies on the selection, e.g. to select UEs to meet KPIs with higher energy /load remaining capacity or prefer static UEs or edge nodes vs UEs.

Then the AIMLE client sends the selected intermediate node (AIMLE client or EAS) to the AIMLE server, including the identifier of the selected node and the time to activate (time of validity) for the addition to the pipeline, as well as to a pipeline ID (of the pipeline to be modified).

7.	AIMLE server associates the selected intermediate node in the AIML pipeline. This can be performed in different ways based on whether the intermediate node is a UE or an edge node and whether the intermediate node is expected to undertake part of the processing:

- If the relay is a UE (AIMLE or VAL client), the AIML pipeline update adds the relay UE as another entity in the existing pipeline with the associated flag as relay/intermediate and the type of relay and the layers that it is going to process (or this can be an indication of the type of processing that the relay will undertake if this is pre-configured, e.g. “relay_split_X corresponds to relay node handling layers 5-15”)

- If the relay is an edge node (EAS/edge AIMLE) then the AIMLE pipeline update add the edge node as another entity in the pipeline or creates two further pipelines (VAL UE-edge node, edge node to VAL server) which are coupled/associated with the first pipeline for the give AIML task/ split learning operation.

8.	After step 7 (or this can be before previous steps given the scenario) the AIMLE server notifies the VAL server and the intermediate AIMLE client on the addition of the intermediate / relay node to the pipeline (or to the creation of dependent/partner pipelines to the existing pipeline).

### 7.12.2	Architecture Impacts

There is no foreseen architecture impact.

### 7.12.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #12. For the procedures in 7.12.1.1, the enhancements relate to existing APIs as in TS 23.482:

- AIMLE client registration (Aimles_AIMLEClientRegistration)

- Split operation node register (Aimles_SplitOpNodeRegistration)

This subclause provides also a summary on the corresponding API for the discovery procedure.

-	Intermediate Node Discovery API (request / response model; API provider: AIMLE server or ML repository; known consumer: AIMLE client or server; corresponding to step 2-4 of clause 7.12.1).

-	Intermediate Node Selection Confirm API (request / response model; API provider: AIMLE client; known consumer: AIMLE server; corresponding to step 6 of clause 7.5.1).

- 	Intermediate Node Notify API (subscribe / notify model; API provider: AIMLE server; known consumer: AIMLE clients, VAL server; corresponding to step 8 of clause 7.5.1).

### 7.12.4	Solution evaluation

This solution addresses Key Issue #4 and provides mechanisms for identifying and determining an intermediate entity (UE, edge function) to serve as relay for a split learning operation, and particularly a split AI/ML operation. Such mechanisms include the registration of the relay/intermediate node (UE or edge node) and the discovery of the node for split learming operation. Such mechanisms propose enhancements to AIMLE client registration, Split Operation node registration as in TS 23.482 to support registering nodes as intermediate nodes in such split operations.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

* * * Next Change * * * *

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |  |
| Sol #3 |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  |  |  |  |
| Sol #6 |  |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  |  |
| Sol #8 |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |
| Sol #11 |  |  |  |  |  |  |  |  |
| Sol #12 |  |  |  | x |  |  |  |  |
| Sol #13 |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  |  |
| Sol #16 |  |  |  |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |
| Sol #20 |  |  |  |  |  |  |  |  |
| Sol #21 |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  |  |  |  |  |  |