3GPP TSG-SA WG6 Meeting #69	S6-254285

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0077 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial Anchor Group Management APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | APIs related to Spatial anchor group management are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the APIs related to the spatial anchor group management operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (All new) 8.6, 8.6.1, 8.6.2, 8.6.2.1, 8.6.2.2, 8.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 8.6	SEAL APIs for spatial anchor group management

### 8.6.1	General

Table 8.6.1-1 illustrates the SEAL APIs for spatial anchor group management.

Table 8.6.1-1: List of SEAL APIs for spatial anchor group management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SAnGroupManagement | Create | VAL server, SAn client | Request/Response |
|  | Update | VAL server, SAn client |  |
|  | Delete | VAL server, SAn client |  |

### 8.6.2	SS_SAnGroupManagement API

#### 8.6.2.1	General

Service description: This API enables a VAL server (or SAn client) to manage spatial anchors groups.

#### 8.6.2.2	Create operation

API operation name: SS_SAnGroupManagement_Create

Description: The consumer requests to create spatial anchor groups.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.7.3.1-1.

Outputs: See Table 8.3.7.3.2-1.

See clause 8.3.7.2.1 for details of usage of this operation.

#### 8.6.2.3	Update operation

API operation name: SS_SAnGroupManagement_Update

Description: The consumer requests to update spatial anchor groups.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.2.3.5-1.

Outputs: See Table 8.3.2.3.6-1.

See clause 8.3.7.2.3 for details of usage of this operation.

#### 8.6.2.4	Delete operation

API operation name: SS_SAnGroupManagement_Delete

Description: The consumer requests to delete spatial anchor groups.

Known Consumers: VAL server, SAn client.

Inputs: See Table 8.3.7.3.3-1.

Outputs: See Table 8.3.7.3.4-1.

See clause 8.3.7.2.2 for details of usage of this operation.

# * * * * End change * * * *