3GPP TSG-SA WG6 Meeting #69	S6-254293

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0239 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Application Client request to update the stored message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update the stored message is introduced to MSGin5G service in Rel-19 in SP-241221. However, the request sent from Application Client to MSGin5G Client (step 0 in clause 8.3.6.1) is not specified. When the Application Client residing in a different UE with the MSGin5G Client, this request is needed to be specified in stage 3. An Editor's note is introduced in clause 6.4.2.3.4 of TS 24.538 by CP-252163, as below: Editor's note: The structure of this request message depends on SA6 definition. Therefore, this CR is proposed to specify the request sent from Application Client to MSGin5G Client to update the stored message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to specify the request sent from Application Client to MSGin5G Client to update the stored message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The request sent from Application Client to MSGin5G Client to update the stored message is not specified. This issue will impact the work of stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.1, 8.11.4, 8.11.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.6.1	Procedure for MSGin5G Client to update the stored message

Figure 8.3.6.1-1 shows the procedure for MSGin5G Client to update the stored message. This procedure enables user to update store and forward parameters of the stored message or discard to stored message. It is up to the end user or application client to decide if update the stored message is required or not.

Pre-conditions:

1.	The MSGin5G Client has originated a message to a recipient UE and it is stored to the MSGin5G Server as the recipient UE is not available (as specified in clause 8.3.6).

Figure 8.3.6.1-1: MSGin5G update stored message parameters

0.	The user or the Application Client decides to update the stored message. In order to update the stored message, the user or the Application Client decides to either update store and forward parameters or discard the stored message. The Application Client informs MSGin5G Client to update the stored message as specified in clause 8.11.x.

1.	The MSGin5G Client sends a request message to update the store message to the MSGin5G Server. The message includes the Message ID for which store and forward parameters needs to be updated, and new or updated store and forward parameters (e.g. Message expiration time) as specified in Table 8.3.2-2.

Upon receiving the request, if the MSGin5G Client is not authorized to update the stored message (e.g. to update store and forward parameters or to discard the message), the MSGin5G Server sends failure response in step-2.

Table 8.3.6.1-1: MSGin5G update stored message request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| MSGin5G Server address | M | The targeted server address |
| Message ID | M | Unique identifier of the stored message which requires update (i.e. for which parameters need to be updated or needs to be discarded) |
| Store and forward parameters (NOTE) | O | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. |
| NOTE: This IE is only present if the request is to update stored and forward parameters. |  |  |

2.	If the MSGin5G Client is authorized, the MSGin5G Server checks whether the message is stored based on received Message ID. If message is stored, and if "Store and forward parameters" IE is present, then the MSGin5G Server updates the store and forward parameters. if "Store and forward parameters" IE is not present, the MSGin5G server discards the stored message. The MSGin5G Server sends a response back to the MSGin5G Client. If the message does not exists, then the response message includes a failure cause.

Table 8.3.6.1-2: MSGin5G update stored message response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the result is Failure. |  |  |

* * * Next Change * * * *

8.11.4	Application Client sending a message using MSGin5G Client

The signalling flow for the Application Client to send a message using the MSGin5G Client on MSGin5G UE-1 is illustrated in figure 8.11.4-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The Application Client is successfully registered with MSGin5G UE-1 to use MSGin5G service.

Figure 8.11.4-1: Application Client sends message using MSGin5G Client on MSGin5G UE-1

1)	An Application Client sends a request to send MSGin5G message to the MSGin5G Client. The information elements defined in Table 8.11.4-1 are included in the message.

Table 8.11.4-1: Information elements for Request to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Recipient UE Service ID or AS Service ID (see NOTE) | O | The service identity of the recipient. This IE is mandatory to initiate Point-to-Point messaging and Point-to-AS messaging. |
| Group Service ID (see NOTE) | O | The service identifier of the target MSGin5G Group. This IE is mandatory to initiate Group messaging. |
| Broadcast Area ID (see NOTE) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message. |
| Messaging Topic (see NOTE) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic. |
| Application ID | O | Identifies the application(s) for which the payload is intended. |
| Request ID | M | Unique identifier of this request. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not. |
| Priority type | O | Application priority level requested for this message as specified in Table 8.3.2-1. |
| NOTE: Only one occurrence shall be present of any of these IEs. |  |  |

2)	Upon receiving the request from the Application Client, the MSGin5G Client constructs an MSGin5G message with the related IEs specified in table 8.3.2-1 and sends the MSGin5G message.

a)	if the size of the received message exceeds the maximum allowed packet size, the MSGin5G Client generate Message IDs for the multiple MSGin5G messages that sent this request, maintain the mapping between the multiple Message IDs and Request ID, and then sends the message as specified in clause 8.5; or

b)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client generate Message ID for the MSGin5G message that sent this request, maintain the mapping between the Message ID and Request ID, and then sends the message as specified in clause 8.7.; or

c)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client may apply message aggregation as specified in clause 8.4 before sending the message as specified in clause 8.7. In this case, the MSGin5G Client generate Message ID for the MSGin5G message that sent the multiple requests, maintain the mapping between the Message ID and multiple Request IDs, and then sends the message as specified in clause 8.7.

NOTE 1:	The MSGin5G Client may also reject the request to send the MSGin5G message based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client sends a response to Application Client. The response message includes the information elements as specified in Table 8.11.4-2.

Table 8.11.4-2: Information elements for Response to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID or AS Service ID or Group Service ID or Broadcast Area ID or Message Topic | M | The service identifier that the message is sent |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If the MSGin5G Client has decided to reject the request to send the message or the MSGin5G Client received a reject response from the MSGin5G Server in step 2, the MSGin5G Client sends a failure response to the Application Client and stops performing further steps.

4)	If delivery status is requested in the message in step 1, the MSGin5G Client may receive MSGin5G message delivery status report from the MSGin5G Server.

5)	Upon receiving the MSGin5G message delivery status report, the MSGin5G Client sends the message delivery status report to the Application Client. The message delivery status report includes information elements as specified in Table 8.11.4-3.

Table 8.11.4-3: Information elements for MSGin5G message delivery status

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID or AS Service ID | M | The service identifier that sends the message delivery status |
| Delivery status | M | Indicates delivery status |

* * * Next Change * * * *

8.11.x	Application Client updating a stored MSGin5G message via MSGin5G client

The signaling flow for Application Client tp update a stored MSGin5G message via MSGin5G Client on MSGin5G UE-1 is illustrated in figure 8.11.x-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The Application Client is successfully registered with MSGin5G UE-1 to use MSGin5G service.

Figure 8.11.x-1: Application Client tp update a stored MSGin5G message via MSGin5G Client

1)	An Application Client requests the MSGin5G Client to update store and forward parameters. The request message includes information elements as specified in Table 8.11.x-1.

Table 8.11.x-1: Information elements for Inform to update store and forward parameters request

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID (see NOTE) | O | Layer-2 identity of UE-2 |
| Application ID | M | Application ID of the application client |
| Request ID | M | Unique identifier of this request. |
| Store and forward parameters (NOTE) | O | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. |
| NOTE: Only included when the application client resides on UE-2. |  |  |

2)	Upon receiving the request from the Application Client, the MSGin5G Client constructs MSGin5G update stored message request and send it to the MSGin5G Server as specified in clause 8.3.6.1. The MSGin5G Client includes the Message ID in the MSGin5G update stored message request based on the mapping between Message ID and Request ID:

a)	If multiple Message IDs map to this Request ID, the MSGin5G Client should construct multiple MSGin5G update stored message requests. Each request contains one Message ID associated with this Request ID.

b)	If one Message ID maps to this Request ID, the MSGin5G Client should construct one MSGin5G update stored message request. This request contains the Message ID associated with this Request ID.

c)	If one Message ID maps to multiple Request IDs, the MSGin5G Client should construct one MSGin5G update stored message request. This request contains the Message ID associated with this Request ID.

3)	The MSGin5G Client sends a response to the Application Client. The response message includes information elements as specified in Table 8.11.x-2.

Table 8.11.2-2: Information elements for update store and forward parameters response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the result is Failure. |  |  |