3GPP TSG-SA WG6 Meeting #69	S6-254300

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution on Alignment 8.4.2 and 8.4.3 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

Spec:	3GPP TR 23.700-57 v0.3.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of KI#1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework. This solution addresses the  procedures in clause 8.4.2 and 8.4.3 of 3GPP TS 23.392.

This pCR proposes this new solution for TS23.700-57.

2. Reason for Change

This pCR provides the solution of KI#1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.3.0.

* * * First Change * * * *

6.x	Solution #x: Alignment 8.4.2 and 8.4.3 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

6.x.1	Description

This solution resolves KI#1: Alignment eMMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework. This solution addresses the  procedures in clause 8.4.2 and 8.4.3 of 3GPP TS 23.392 [2] in Rel-20.

6.x.2	Procedures

6.x.2.1	Update to existing procedures in 8.4.2

The following changes are proposed to clause 8.4.2. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

8.4.2	Application calling service API with Data Channel capability

8.4.2.1	General

The MMTel Enabler Server exposes the Application calling service API with Data Channel capability to the Applications in order to support establishing a call between Application and DCMTSI client.

8.4.2.2	Procedure

8.4.2.2.1	Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client

Figure 8.4.2.2.1-1 illustrates the operation to establish an Application Call with Data Channel capability between non-IMS application and DCMTSI client.

Pre-conditions:

1.	The Application is authorized to use Application calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

3.	The MMTel Enabler Server is authorized to use OMA ThirdPartyCall API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.2.2.1-1: Establishing a Application Call with Data Channel capability between non-IMS applications and DCMTSI client

1.	The Application sends an Application Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, if the requester is authorized then the MMTel Enabler Server can utilize the OMA Third Party Call API [7] to establish a Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third-Party Call establishment. The MMTel Enabler Server may also obtains the IMS Session ID associated with the Third Party Call as specified in RFC 7989 [10], and handles the mapping between the OMA call session ID and the IMS session ID.

NOTE 1:	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session ID.

3,	The MMTel Enabler Server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of  3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12]DC update capability of the IMS core network to add DC media to the existing Application Call Session as specified in Annex AG.2.1AC of 3GPP TS 23.228 [3]. The IMS core network returns the update result.

3.	The MMTel Enabler Server sends an Application Call establishment response to the Application. The response message includes information elements as specified in clause 8.4.2.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

8.4.2.3	Information flows

8.4.2.3.1	Application Call establishment request

The information elements of Application Call establishment request are as same as the elements in Table 8.4.3.3.1-1.

8.4.2.3.2	Application Call establishment response

The information elements of Application Call establishment response are as same as the elements in Table 8.4.3.3.2-1.

6.x.2.2	Update to existing procedures in 8.4.3

The following changes are proposed to clause 8.4.3. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

8.4.3	Third-Party Call service API with Data Channel capability

8.4.3.1	General

The MMTel Enabler Server exposes the Third-Party Call service API with Data Channel capability to the Vertical service provider in order to support establishing a call between two UEs.

8.4.3.2	Procedure

8.4.3.2.1	Establishing a Third-Party Call with Data Channel capability

Figure 8.4.3.2.1-1 illustrates the operation to establish a Third-Party Call with Data Channel capability.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

3.	The MMTel Enabler Server is authorized to use OMA THIRD PARTY CALL API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.3.2.1-1: Establishing a Third-Party Call with Data Channel capability

1.	The Vertical service provider sends a Third-Party Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server utilizes the OMA Third Party Call API [7] to establish a Third-Party Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third Party Call establishment. The MMTel Enabler Server obtains IMS Session ID associated with the Third Party Call as specified in RFC 7989 [10],the OMA API call Session ID and handles the mapping between OMA call session ID and the IMS session ID

NOTE 1:	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session Id.

3.	the MMTel Enabler Server utilizes the IMS session management capabilities provided by IMS AS as specified in Annex AA.2.4 of 3GPP TS 23.228 [4] or IMS session management capabilities provided by NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12]DC update capability of the IMS core network to add DC media to the existing Third-Party Call Session as specified in Annex AG.2.1 of 3GPP TS 23.228 [3], and the IMS core network returns the update result.

4.	The MMTel Enabler Server sends a Third-Party Call with DC capability establishment response to the Vertical service provider. The response message includes information elements as specified in clause 8.4.3.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

8.4.3.3	Information flows

8.4.3.3.1	Third-Party Call with DC capability establishment request

Table 8.4.3.3.1-1 describes information elements for the Third-Party Call with DC capability establishment request from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.1-1: Third-Party Call with DC capability establishment request

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A |
| Notification information | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. | The callbackReference of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

8.4.3.3.2	Third-Party Call with DC capability establishment response

Table 8.4.3.3.2-1 describes information elements for the Third-Party Call with DC capability establishment response from the Application Server to the MMTel Enabler Server.

Table 8.4.3.3.2-1: Third-Party Call with DC capability establishment response

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Call establishment result | M | Indication if the Call establishment is success or failure. | The HTTP response codes of the created call session as specified in OMA REST_NetAPI_Common [11]. |
| Session ID (see NOTE 1) | O | The identifier of the call session. | The callSessionId as specified in OMA Third Party Call API [7]. |
| Failure Cause (see NOTE 2) | O | The reason for failure | The Policy Exception and Service Exception fault as specified in OMA Third Party Call API [7]. |
| Originating ID (see NOTE 3) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7]. |
| Terminating ID (see NOTE 3) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7]. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of MMTel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7]. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call. | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call. | N/A |
| NOTE 1: This IE shall only be present when the Call establishment result is Success. NOTE 2: This IE shall only be present when the Call establishment result is Failure. NOTE 3: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

6.x.3	Impacts on Existing Nodes and Functionality

In this solution, the MMTel Enabler server needs to utilize new IMS session management capabilities provided by the NEF as specified in clause 5.2.6.40 of 3GPP TS 23.502 [12], and also needs to utilize the procedures specified in Annex AG.2.1 of 3GPP TS 23.228 [3] instead of Annex AC of 3GPP TS 23.228 [3].