3GPP TSG-SA WG6 Meeting #69	S6-254348

Wuhan, China 13th – 17th Oktober 2025	(revision of S6-254xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on DM for MCVideo push and pull and MCData file upload and download

Spec:	3GPP TR 23700-37-030

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR introduces the specific key issues for DM for MCVideo push and pull and MCData file upload and download. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

This key issue should be addressed in the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.3.0.

* * * First Change * * * *

## 5.x	Key issue x:	Discreet monitoring for MCVideo push and pull and MCData file upload and download

The MCVideo video push and video pull services each start with a negotiation phase, where information related to the video content to be pushed or pulled such as URL, media characteristics etc is provided and agreed. The MC user can be notified and asked to accept incoming video within a video push.

Similarly, the MCData file upload and file download services each start with a negotiation phase where information related to the file transfer such as URL is provided and agreed. The MC user can be notified and asked whether to accept an incoming file transfer.

The discreet monitoring service could either allow the authorized MC user who is the recipient of the information to accept video or data before that video or data is sent to the authorized MC user or could be provided with the video or data sent to or from the target MC user of the discreet monitoring service automatically.

Issues:

-	Whether the authorized MC user should be notified about the start of video push and pull or file upload or download transactions involving the MC user who is the target of discreet monitoring and given the choice to accept and receive the information or reject receiving the information; or whether the authorized MC user should always receive the information without any choice being needed.

-	Whether the authorized MC user should receive notification both when the file upload or download negotiation phase takes place, and a second notification when the file upload or download takes place.

-	Whether the authorized MC user should automatically be presented with the file date if a mandatory download takes place to the target MC user

-	Whether the file content should be saved by the MCData server (or another suitable entity) until the authorized MC user decides to retrieve the file, and if so, whether there should be a time limit on this storage.

* * * End of Changes * * * *