3GPP TSG RAN Meeting #109												RP-251935

Beijing, China, September 15-18th, 2025

Title:	Text proposal for TR 38.914 for NTN deployment scenarios

Source:	Thales, ESA, Airbus DS, Fraunhofer IIS, Fraunhofer HHI, Eutelsat, Iridium, Gatehouse Satcom, TNO, Novamint, ST Engineering iDirect

Agenda Item:	8.2.1.1 Deployment scenarios

Document for:	Discussion/Decision

# Introduction

This contribution presents a Text proposal for clause 4.10 on NTN deployment scenarios for the TR 38.914. It is based on RP-250880 and RP-251558

======================Start of the changes========================

# 4.	Deployment scenarios

# ..

## 4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

This deployment scenario category is defined to allow for the provision of services for those areas where the terrestrial network service is not available (on permanent or temporary basis) and also for those services that can be more efficiently supported by the non-terrestrial networks such as one to many services. Non Terrestrial networks act as a fill-in especially over low density populated areas and maritime areas. As such NTN will mainly help to:

- Achieve ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improve overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables by supporting in priority critical communications and emergency communications (messaging, voice calls).

Non Terrestrial Networks may support messaging, voice, positioning, delay tolerant, machine type communications and data services.

Two types of deployment scenarios can be considered

- Direct connectivity to smartphone or Direct to IoT- devices operating in bands less than 7.125 GHz

- Broadband connectivity to VSAT terminals operating in bands above 10.7 GHz

The characteristics of the NTN scenario are listed in Table 4.10

Table 4.10: Attributes for Non-Terrestrial Network Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz (L band) Around 2 GHz (S band) Around 4 GHz (C band) Around 10/12 GHz (Ku band) Around 20/30 GHz (Ka band) Around 40/50 GHz (Q/V band) |
| System Bandwidth | Below 7.125 GHz: Up to [40] MHz (DL+UL) Above 10.0 GHz: Up to [500] MHz (DL+UL) |
| Orbit | LEO: From 300 to 1200 km altitude, 20° min elevation angle MEO: From 7000 to 25000 km altitude, 20° min elevation angle GSO: at 35 786 km, 10° min elevation angle |
| Duplex mode | In paired bands and all orbits: FDD (Frequency Division Duplexing) or Half Duplex Frequency Division Duplexing (HD-FDD) mode In unpaired bands and with LEO only: TDD (Time Division Duplexing) mode |
| Payload type | Transparent (a): RF repeater on board Transparent (b): Radio Unit (RU) on board Regenerative (a): at least Full base station (BS) on board Regenerative (b): Part of a base station on board |
| Beam foot print | Earth fixed or Earth moving, Up to 250 km diameter |
| User distribution | Below 7.125 GHz: - Nominal conditions: [99.99%] Outdoor and [0.01%] indoor (1st wall) - Disaster conditions: [99.0%] Outdoor and [1.0%] indoor (1st wall) Above 10.7 GHz: Outdoor only |

The NTN capable UE characteristics are described in clause 5.3.2

======================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 4.10 of the TR 38.914.