3GPP TSG RAN Meeting #109				RP-252000

Beijing, China, September 15-18, 2025

Source:	OPPO

Title:	Discussion on 6G Key performance indicators

Agenda Item:	8.2.1.2

Document for:	Discussion

Introduction

The study item on 6G scenarios and requirements was approved in RAN#107 (SID in [1]). In RAN#108, the 6G KPIs were discussed, and the first TP was agreed in [3]. But some values were left TBD for further study. In this paper, our views on the remaining issues on 6G deployment scenarios are presented.

Discussions on IMT-2030 (6G) KPIs

KPIs for 6G communication

In this section, our views and TPs are provided for the 6G communication KPI values.

Table 1: TP for TR 38.914 on 6G communication KPI values

| ITU IMT-2030 TPR | IMT-2020 Value | Proposed TP for TR 38.914 Section 5 | OPPO’s view |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20 UL: 10 | The minimum requirements for peak data rate are as follows: – Downlink peak data rate is TBD20 Gbit/s. – Uplink peak data rate is TBD10 Gbit/s.] | - Peak data rate is not a requirement from market, but only a theoretical calculation. Peak data rate in NR has been 200x of user experienced data rate). - We suggest to keep the same value as IMT-2020. |
| Peak spectral efficiency (bit/s/Hz) | DL: 30 UL: 15 | The minimum requirements for peak spectral efficiencies are as follows: – Downlink peak spectral efficiency is TBD60 bit/s/Hz. – Uplink peak spectral efficiency is TBD22.5 bit/s/Hz. These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink. | - From the requirement perspective, the Peak spectral efficiency should not be theoretically calculated based on number of MIMO layer and modulation order. - Moderately improved, e.g., - DL: 2x of IMT-2020 - UL: 1.5x of IMT-2020. |
| User experienced data rate (Mbit/s) | DL: 100 UL: 50 | The minimum requirements for the user experienced data rate are as follows in the Dense Urban – IC [test environment]: – Downlink user experienced data rate is TBD200 Mbit/s. – Uplink user experienced data rate is TBD100 Mbit/s. | - Considering the potential performance improvement, the following values are . - DL: 2x of IMT-2020 - UL: 2x of IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | The minimum requirements for 5th percentile user spectral efficiency for various [test environments] are summarized in Table 5.1.4-1. Table 5.1.4-1: 5th percentile user spectral efficiency [Test Environment]; Downlink (bit/s/Hz); Uplink (bit/s/Hz). Indoor Hotspot – IC; 0.9; TBD0.4. Dense Urban – IC; 0.675; TBD0.3. Rural – IC; TBD0.24; TBD0.09. | - Prefer 2x of IMT-2020. - But agree with 3x of IMT-2020 for DL. Antenna assumption for 3x of IMT-2020 in DL: Up to CSI-RS 256T4R@7GHz |
| Average spectral efficiency (bit/s/Hz) | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1. Table 5.1.5-1: Average spectral efficiency [Test Environment]; Downlink (bit/s/Hz/TRxP); Uplink (bit/s/Hz/TRxP). Indoor Hotspot – IC; 27; TBD13.5. Dense Urban – IC; 23.4; TBD10.8. Rural – IC; TBD6.6; TBD3.2. | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. (feasible with AI/ML-based enhancements). |
| Area traffic capacity (Mbit/s/m²) | DL: 10 | The minimum requirement for area traffic capacity in downlink is TBD30 Mbit/s/m² in the Indoor Hotspot – IC [test environment]. | Prefer 3x of IMT-2020. But agree with 5x of IMT-2020. (Needs to be justified.) |

It should be noted that these KPIs are defined to evaluate the 6G capability to provide traditional communication services, e.g., eMBB, industrial IoT, low-end IoT. The question is that to what extent the KPIs need to be improved over IMT-2020 values. Considering that 6G targets on a more successful technology on market, the performance improvements should be helpful for the success of the services. In this sense, we can categorize the KPIs into three groups:

- Group 1: The KPIs are not the key to make the 6G communication services successful.

- Group 2: The KPIs are the key to make the 6G communication services successful, but have been met by 5G system. No substantial enhancement is needed.

- Group 3: The KPIs are the key to make the 6G communication services successful, and have not well met by 5G system. Enhancement is needed.

From our perspective, the Group 1 include Peak data rate and Peak spectral efficiency. The two KPIs are only theoretical, and are not directly related to service provision capability, even for Immersive/eMBB services. In IMT-2020 (5G) TPR, DL Peak data rate = 20Gbps, UL Peak data rate = 10Gbps. The data rate is not actually needed by any 5G service. It will not be needed by any 6G service either, including XR or AIaaS application. It may be required by holographic display. But we do not see the need to provide holographic display services in mobile communications system in near future. Another consideration is that IMT-2030 may not have new spectrum with much wider bandwidth than IMT-2020. Only 800MHz spectrum are newly provided to IMT services below 7GHz in WRC-23. There is no need to significantly expand the bandwidth. Hence Peak data rate should not be the focus for 6G improvement. We see no need to increase DL/UL Peak data rate to 50~100/25~50Gbps.

We have similar observation on Peak spectral efficiency. In IMT-2020 (5G) TPR, DL Peak spectral efficiency = 30bit/s/Hz, UL Peak spectral efficiency = 15bit/s/Hz, which has been a very high value. From the requirement perspective, the Peak spectral efficiency should not be theoretically calculated based on number of MIMO layer and modulation order. We think this KPI can be moderately improved, e.g., 2X of 5G NR for DL and 1.5X of 5G NR for UL.

Group 2 may include Control plane latency, Reliability and Connection density. Latency and reliability are the essential performance requirements for URLLC/IIoT applications. Connection density is the essential performance requirement for massive IoT. The performance has been supported by 5G/5G-A system, e.g., 1ms air interface latency, 10-5~10-6 reliability and 106 devices/km2 connection density. Compared to the actual commercial deployment of URLLC/IIoT and massive IoT, the 5G system performance is far beyond the market needs. No actual justification that 1ms latency and 10-6 reliability cannot support HRLLC/IIoT applications in market. No actual justification that 106 devices/km2 connection density cannot support massive IoT applications in market. We suggest to firstly deploy the system capabilities in market, and consider the further KPI improvements after the first-wave commercial deployment. We suggest to design 6G low-end IoT in 1st release of 6G. But the motivation is to design a low-end IoT which share the single RAT with 6G eMBB/immersive, not to pursue a higher KPI, e.g., connection density. 6G connection density does not need to be improved over 5G.

Group 3 include User experienced data rate, Average spectral efficiency, 5%th percentile user spectral efficiency and Area traffic capacity. The KPIs are related to actual user experience in eMBB/Immersive services. And the values are below the actual market expectation, at least for 5G smartphone market. Among them the most essential performance is the 5th percentile user spectral efficiency. In 5G system, the difference between user experience in cell center and at cell edge became larger. Smartphone users feel that 5G coverage degrades over 4G coverage. However, the feasibility of applicable coverage enhancement techniques needs to be considered. The preliminary evaluation of 6G average spectrum efficiency and 5%-tile spectrum efficiency in 7GHz band is shown in Figure 1.

Figure 1: Preliminary evaluation of 6G average/5%-tile spectrum efficiency gain @7GHz

2 performance over IMT-2020 can be considered for 5%th percentile user spectral efficiency. 3 performance over IMT-2020 can be achieved with specific MIMO configuration, e.g., CSI-RS ports 256T4R@7GHz. For Average spectral efficiency, 2 performance over IMT-2020 can be considered for 7GHz band. Beyond that, the performance gain should be practically estimated considering the improvement of MIMO and AI for air interface techniques. Considering 6G new spectrum is limited, we should not expect an obvious wider bandwidth to significantly improve User experienced data rate. So, the gain mainly relies on the improvement of spectral efficiency. We suggest to consider 2 performance over 5G for them.

Proposal 1: For 6G communication KPIs, consider the TPs in 3rd column of Table 1 for TS 38.914 Section 5.

KPIs for 6G Energy efficiency

Energy efficiency was in principle defined in IMT-2020 (5G) TPR under capability of “sustainability”, without defining numerical KPIs values. It can be considered to define the KPIs for IMT-2030 (6G) TPR.

One way is to define the comparative KPI from 5G energy consumption. This approach has been employed in 5G enhancement for UE power saving and NW energy saving from Rel-16 to Rel-19. We think this approach can be used as a baseline, i.e., how many percent energy 6G NW and UE can save over 5G NW and UE.

A further way is to define the energy efficiency, e.g., consumed energy per transmitted bit (in Joule/bit). However, it needs to be further studied what service/traffic model is used for the definition. In principle, a high-rate data service has a higher energy efficiency than low-rate service, although the energy consumption of high-rate service is much higher.

Proposal 2: For 6G Energy efficiency, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

Composite requirement

For some KPI items are defined only for extreme use cases. For example, the latency and reliability in 5G are defined assuming zero payload in packet. But the extreme latency (e.g. 1ms) and reliability (e.g. 99.999%) does not make much sense for service provision in real market. Even for URLLC/IIoT services, a moderate size of packet needs to be transferred.

Hence, to evaluate the performance and the real use experience of a service requiring other KPIs than data rate, a joint KPI combining latency, reliability and capacity should be adopted. For example, in Study on XR (Extended Reality) Evaluations for NR (TR 38.838 [4]), the joint KPI is used to evaluate performance of XR system. For example, 11 XR users/cell can be supported with 30Mbps data rate, 10ms PDB (Packet Delay Budget) and 1% PER (Packet Error Rate). 16 XR users/cell can be supported with 30Mbps data rate, 10ms PDB and 5% PER.

We suggest to adopt the evaluation methodology in TR 38.838 [4] as reference, and define a joint KPI combining data rate, PDB, PER and capacity for typical services, as shown in Table 1.

Table 1: Example of Composite requirements for different services

|  | Data rate (Mbps) | PDB (ms) | PER (%) | Capacity (UEs/cell) |
| --- | --- | --- | --- | --- |
| Service 1 |  |  |  |  |
| Service 2 |  |  |  |  |
| Service 3 |  |  |  |  |
| …… |  |  |  |  |

For the first release of 6G, the primarily essential application scenarios are Immersive and Massive communication. For massive communication, the joint KPI can be with low data rate, large PDB, basic PER and large number of UEs/cell. We can also define the joint KPI for HRLLC. But from our perspective, the HRLLC can be supported for 6G Day1 by reusing 5G URLLC standards.

Proposal 3: Consider composite requirement combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

KPIs for 6G Sensing

Integrated Sensing and Communication is another area requiring new KPI system. One discussion point is which level KPIs are defined for 6G sensing. Generally, three options can be considered.

Option 1: KPIs for basic physical parameters, such as distance, velocity and angle. Similar as defined in TR22.837, accuracy and resolution of distance/velocity/angle estimation and related general requirement, Confidence level, latency, refreshing rate, missed detection under constant false alarm. Option 1 could cover some fundamental use cases but it is still restricted within these fundamental use cases. Some popular use cases recent years cannot be covered.

Option 2: KPIs for both basic and advanced physical parameters, such as accuracy of model parameters. Comparing with option 1, option 2 extends to some advanced parameters to cover some popular use cases recent years, such as motion monitoring. For motion monitoring, accuracy of model parameter, i.e. repetition rate can be defined as KPI.

Option 3: KPIs for service parameters, such as probability of rainfall. Comparing with option 1 and option 2, option 3 means that BS/UE will not only perform signal processing in physical layer but also provide service-oriented data analysis. For each service, service-oriented data analysis is very professional, which is not our advantage currently but it gives us opportunity to extend our business.

Comparing with option 1/2/3, Option 2 is a good tradeoff between extensiveness and complexity. So, option 2 can be a starting point at current stage. As sensing function and application develops, option 3 can be considered in later stage.

Proposal 4: For 6G Sensing, KPIs for both basic and advanced physical parameters can be studied.

Conclusion

The proposals for TR 38.914 are listed below:

P Proposal 1: For 6G communication KPIs, consider the TPs in 3rd column of Table 1 for TS 38.914 Section 5.

Proposal 2: For 6G Energy efficiency, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

Proposal 3: Consider composite requirement combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

Proposal 4: For 6G Sensing, KPIs for both basic and advanced physical parameters can be defined.

References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- Recommendation ITU-R M.2160-0 (11/2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

- RP-251880, Text proposal for TR 38.914 on KPIs, CMCC, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17), V17.0.0 (2021-12)

- TS 22.137, Integrated Sensing and Communication (Release 19), V19.1.0 (2024-03)