3GPP TSG RAN Meeting #109								RP-252065

Beijing, China, September 15-18, 2025

## Status Report to TSG

Agenda item:			9.6.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242394 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 40% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#122, the following agreements were made.

UE-initiated/event-driven beam management

Agreement:

Adopt the following changes in TS38.213 Section 9.2.5.1:

- Reason for change: In the last meeting, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR, the multiplexing rule that was agreed in RAN1#121.  However, in the current specification, only the priority rule of LRR, UEIRI and normal SR is captured when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR.

- Summary of change: In TS38.213 section 9.2.5.1, add the case of no positive LRR and at least one positive UEIBRs to the multiplexing rule of PUCCH with UEIBR.

- Consequences if not approved: UE behavior is unclear for the case of no positive LRR and at least one positive UEIBRs, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR.

9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH < Unchanged parts are omitted > If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+1 bits representing a negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs. If a UE would transmit a PUCCH with CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, log2K+1 bits representing corresponding negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are prepended to the CSI information bits as described in clause 9.2.5.2 and the UE transmits a PUCCH with the combined OUCI=log2K+1+OCSI UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs.

Agreement:

Adopt the following changes in TS38.213 Section 7.5:

- Reason for change: When the transmission powers of multiple uplink transmissions exceed the UE’s maximum transmit power, TS 38.213 specifies a prioritization mechanism to determine which transmissions should be reduced or dropped. This prioritization covers all conventional UCI types, such as SR, CSI reports, and HARQ-ACK, ensuring consistent behavior across implementations when power reduction is needed. However, the UE-initiated CSI reporting indicator (UEIRI), introduced as a new UCI type to support UE-initiated CSI reporting, is not currently included in the priority order defined in TS 38.213.

- Summary of change: Capture the priority rule for UEIRI, i.e., the same priority as LRR/SR.

- Consequences if not approved: Without explicit prioritization, UE behavior may vary between implementations, potentially causing inconsistent reporting of UE-initiated CSI under power-limited conditions. This could result in the unintended suppression of UEIRI or, conversely, excessive reduction of other UCI types, impacting system performance and interoperability

## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation or for operation with a candidate cell configured by LTM-Config or for operation with a single cell configured with sTx-2Panel, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cell(s) or on a candidate cell, if any, in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cell(s) or on a candidate cell, if any, in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according to Clause 6.2.1.4 of [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power. For the purpose of power allocation in this clause, if a UE is provided uci-MuxWithDiffPrio and the UE multiplexes HARQ-ACK information in a PUSCH, a priority index of the PUSCH is the larger of (a) the priority index of the PUSCH according to clause 9 and (b) the larger priority index of the HARQ-ACK information. When determining a total transmit power for serving cells or a candidate cell, if any, as described in Clause 21 in a frequency range in a symbol of transmission occasion i, the UE does not include power for transmissions starting after the symbol of transmission occasion i. The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. - PRACH transmission on a candidate cell, if any, as described in Clause 21 - PRACH transmission on the PCell - PUCCH or PUSCH transmissions with larger priority index - For PUCCH or PUSCH transmissions with same priority index - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, and/or UEIRI, or PUSCH transmission with HARQ-ACK information of the priority index - PUCCH transmission with CSI or PUSCH transmission with CSI - PUSCH transmission without HARQ-ACK information of the priority index or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell - If the UE is configured with prioSCellPRACH-OverSP-PeriodicSRS-r17 - Aperiodic SRS transmission or PRACH transmission on a serving cell other than the PCell - Semi-persistent and/or periodic SRS transmission - otherwise, - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell =========unchanged omitted===========

Agreement:

Adopt the following changes in Clause 5.2.5 in TS38.214.

- Reason for change: Reusing the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for PUSCH for UEI-BR for Mode B was agreed. However, it is not captured in the current specification on the priority rules of CSI reports.

- Summary of change: In TS38.214 section 5.2.5, clarify the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for PUSCH is reused for UEI-BR for Mode B.

- Consequences if not approved: UE behavior is not clear when PUSCH carrying UEI-BR for Mode B overlaps with other PUSCHs.

5.2.5 Priority rules for CSI reports <Unchanged parts are omitted> If a UE would transmit a first PUSCH that includes semi-persistent CSI reports or UE initiated CSI report for mode B and a second PUSCH that includes an UL-SCH on the same carrier, and the first PUSCH transmission would overlap in time with the second PUSCH transmission, the UE does not transmit the first PUSCH and transmits the second PUSCH. The UE expects that the first and second PUSCH transmissions satisfy the above timing conditions for PUSCH transmissions that overlap in time when at least one of the first or second PUSCH transmissions is in response to a DCI format detection by the UE.

Agreement:

Adopt the following changes in TS38.214 Section 5.2.1.5.4.1d:

- Reason for change: In TS38.214 [3], the CSI report identifier in the beam report format is captured as CSI-ReportConfigID. However, according to the related agreement, the CSI report identifier is determined based on the order of the triggered CSI report in the CSI report configurations associated with the same PUCCH resource. Thus, the specification description is aligned with the agreement only when all CSI report configurations in a CC are associated with the same PUCCH resource.

- Summary of change: In TS38.214 section 5.2.1.5.4.1d, clarify the determination of a CSI report identifier in the report format when multiple CSI report configurations are associated with one PUCCH resource.

- Consequences if not approved: The spec is not aligned with the agreement.

5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations For a UE configured with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, the UE expects - the multiple CSI-ReportConfig to be configured in the same CC, - the multiple CSI-ReportConfig to be associated with a same CSI trigger state if reportTransmissionMode is configured as ‘ModeA’, else - the same configuredPUSCHResourceOfModeB if reportTransmissionMode is configured as ‘ModeB’. The UE reports in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet in a CSI-ReportConfig that satisfies the event. The CSI report includes the corresponding CSI-ReportConfigId CSI report configuration indicator as defined in Table 6.3.2.1.2-3I in [5, TS38.212] corresponding to the CSI-ReportConfig the and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig.

Agreement:

Adopt the following changes in Section 5.4 in TS 38.214.

- Reason for change: Legacy CSI reference definition of aperiodic CSI reporting is reused for mode-A and definition of Z' for mode-B follows the legacy definition, hence we should capture the agreement by declaring that Z3 and Z3' defined for legacy beam report are used for UEI-CSI report as well.

- Summary of change: Clarifying that Z3 and Z3' defined for legacy beam report are used for UEI-CSI report as well.

- Consequences if not approved: Z/Z' definition for UEI-CSI report is unclear.

# 5.4 UE CSI computation time <Irrelevant part is omitted> - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index' or 'ssb-Index-RSRP- Index ', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or if the CSI report is configured with ltm-CSI-ReportConfig for L1-RSRP measurement, or if the CSI report is configured with eventType, or

Agreement:

Adopt the following changes in TS38.214 Section 5.2.1.5.4.1:

- Reason for change: Besides for correcting typos, when “current” beam RS for event evaluation is SSB QCLed with RS in an activated TCI state, the SSB having the Q-th best quality among all SSBs QCLed with the RSs in all the activated TCI states should be clarified:

- Summary of change:

  - In TS38.214 section 5.2.1.5.4.1, add “state” after “indicated TCI”.

  - In TS38.214 section 5.2.1.5.4.1c, clarify the current beam RS determination when enabledCurrentBeamReport is configured for event-7.

- Consequences if not approved: It can cause inconsistency in the specification description.

5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. < Unchanged parts are omitted > 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 < Unchanged parts are omitted > After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ-th highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block with the valueOfQ-th highest L1-RSRP out of the SS/PBCH blocks QCLed with the activated TCI state reference signals which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. < Unchanged parts are omitted >

Agreement

Update the RRC candidate values for a threshold value eventThreshold-Event1-r19 for trigger event detection regarding Event-1.

- only values 16, …,113 in RSRP-Range are valid.

Agreement:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, occupation time of occupied CPU(s) starts when CSI report configuration for the UEI beam report is configured, and ends when it is released.

CSI

Agreement:

Adopt the following changes to TS38.214 V19.0.0 Clause 6.2.1.3 on priority rules for UE sounding procedure as follows:

Reason for change: The newly agreed CSI reporting quantities in Rel-19 MIMO CSI has not been reflected in the current description of priority rules in TS38.214

Summary of the change: Added the new CSI reporting quantities for CJT calibration reporting

Consequences if not approved: Incomplete description in TS38.214

#### 6.2.1.3 UE sounding procedure between component carriers ##### < Unchanged part omitted > - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. ##### < Unchanged part omitted >

Agreement:

For the Rel-19 Type-I SP and Type-II codebook refinement, refine the following agreement in RAN1#121 as follows:

Agreements

For the Rel-19 Type-I SP and Type-II codebook refinement for P=48, 64, and 128 CSI-RS ports with K>1 aggregated NZP aperiodic CSI-RS resources for CMR, to implement the previous agreements on the mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation

- In TS38.211, extend the enumeration of antenna port p=3000+p’ with p’=0 …P–1

- In TS38.214, remove the term ‘port index for CSI/PMI calculation’

Consequently, based on the endorsed version of TS38.214 V19.0.0, the current description for aperiodic CSI-RS resources should also be applied to periodic and semi-persistent CSI-RS resources.

TPs in Section 3.1 and 3.2 of R1-2505335 for TS 38.214 are endorsed.

Conclusion:

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, joint configuration of the Rel-18 SD NES Type-I, the Rel-19 Type-I SP codebook, and the Rel-19 soft scaling for the Rel-19 Type-I SP codebook is not supported.

Conclusion:

For Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports based on the Rel-18 Type-II Doppler codebook, following the legacy (Rel-18) principle, a UE shall assume that CSI-RS ports mapped to the same port index p' across the K aperiodic CSI-RS resources, p'=0,1,…,PCSI-RS-1, as described in Clause 7.4.1.5.3 of [4, TS 38.211], share the same antenna port.

Conclusion:

For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the N1⋅N2X1⋅X2-bit group-based bitmap is identical for all the O1O2 groups

Conclusion:

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, when one NZP CSI-RS resource for interference measurement is configured, it is associated with all the KS NZP CSI-RS resources for channel measurement

Conclusion:

For Rel-19 SRS port grouping, whether resources across multiple resource sets can be used follows the legacy principle, i.e. for periodic and semi-persistent SRS resource sets, it is not possible to have the grouped SRS ports resources across multiple SRS resource sets; but for aperiodic SRS resource sets, the grouped SRS ports can come from multiple SRS resource sets.

3Tx uplink

Agreement:

The TPs in Proposal 1 in Section 4.1 of R1-2505237 for TS38.214 are endorsed.

Agreement:

The TP in Proposal 2 in Section 4.1 of R1-2505237 for TS38.214 is endorsed.

Agreement:

The TP in Proposal 4 in Section 4.1 of R1-2505237 for TS38.214 is endorsed.

Agreement:

Adopt the following TP for TS 38.214 section 6.2.1.2.

Reason for change: Reflect the agreement in UE feature.

Summary of change: Include the cases of 3T3R and 3T6R antenna switching.

Consequence if not changed: The AS feature 3T3R and 3T6R antenna switching feature will not be supported.

#### 6.2.1.2 UE sounding procedure for DL CSI acquisition -------------------------------------------Unchanged parts are omitted------------------------------------------ When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R, or the UE may be configured with the following configurations depending on the indicated UE capability srs-AntennaSwitching8T8R ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R, 'noTdm' or 'tdmAndNoTdm' for 8T8R) or the UE may be configured with ‘t3r3’ for 3T=3R depending on the indicated UE capability [FG 59-3-3a: srs-AntennaSwitching3T3R] or the UE may be configured with ‘t3r6’ for 3T6R depending on the indicated UE capability [FG 59-3-3: srs-AntennaSwitching3T6R]: -------------------------------------------Unchanged parts are omitted------------------------------------------

Agreement:

Adopt the following TP for TS 38.214 section 6.2.3.1.

Reason for change: Correct the PTRS-DMRS association as DCI format 0_3 is not supported for a 3TX UE

Summary of change: Include a new text without referencing PUSCH port 1003 that is not supported.

Consequence if not changed: Incomplete/incorrect specification of PTRS-DMRS association.

#### 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled --------------------------------------Unchanged parts are omitted-------------------------------------- For partial-coherent and non-coherent codebook-based UL transmission with 2 or 4 antenna ports, or non-coherent codebook-based UL Transmission with 3 antenna ports, or when the higher layer parameter CodebookTypeUL is set to ‘codebook2’, ‘codebook3’, or ‘codebook4’ with 8 antenna ports, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - for PUSCH transmission with 2, 3 or 4 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 or 0_3 described in Clause 7.3.1 of [5, TS 38.212]. --------------------------------------Unchanged parts are omitted--------------------------------------

Asymmetric DL-sTRP/UL-mTRP

Agreement:

Adopt the following TP for TS 38.214 section 6.2.3.1.

Reason for change: Correct the PTRS-DMRS association as DCI format 0_3 is not supported for a 3TX UE

Summary of change: Include a new text without referencing PUSCH port 1003 that is not supported.

Consequence if not changed: Incomplete/incorrect specification of PTRS-DMRS association.

#### 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled --------------------------------------Unchanged parts are omitted-------------------------------------- For partial-coherent and non-coherent codebook-based UL transmission with 2 or 4 antenna ports, or non-coherent codebook-based UL Transmission with 3 antenna ports, or when the higher layer parameter CodebookTypeUL is set to ‘codebook2’, ‘codebook3’, or ‘codebook4’ with 8 antenna ports, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1, 0_2 or 0_3 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - for PUSCH transmission with 2, 3 or 4 ports, PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1, 0_2 or 0_3 described in Clause 7.3.1 of [5, TS 38.212]. --------------------------------------Unchanged parts are omitted--------------------------------------

Agreement:

The TP in Proposal 1.6 in Section 3 of R1-2506397 for TS 38.213 is endorsed.

Agreement:

The TP in Proposal 2.3 in Section 3 of R1-2506397 for TS 38.212 is endorsed.

Agreement:

The TP in Proposal 2.1 in Section 3 of R1-2506397 for TS 38.213 is endorsed.

Agreement:

The TP in Proposal 4 in Section 3 of R1-2506397 for TS 38.213 is endorsed.

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Maintenance on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Maintenance on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Maintenance on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Maintenance on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Maintenance on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Maintenance on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Maintenance on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - Maintenance on SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Maintenance on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Maintenance on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Maintenance on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Asymmetric DL-sTRP/UL-mTRP:

- Maintenance on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Maintenance on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Maintenance on two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#131

Agreements on the remaining open issues

If the PUCCH of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this PUCCH for the SCell is released by RRC. If the type-1 CG of a UEI report configuration is pointed to a SCell whose TAT of the single sTAG is expired, this type-1 CG for the SCell is cleared as a configured UL grant. There is no MAC specification impact Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits mode-A UE-initiated CSI reporting on PUCCH and PUSCH on the Serving Cells in the DRX group when such report is expected. UE does not transmit PUCCH and PUSCH for mode-B if either PUCCH or PUSCH (first valid type-1 CG occasion) is outside DRX Active Time, and if the PUCCH has been transmitted, UE does not transmit the PUSCH if type-1 CG occasion is outside DRX Active time. Inform RAN1 about this conclusion. For mode-A UEI report, regarding monitoring PDCCH for DG in cell DTX, no enhancement is needed. Regarding UEI report in cell DRX: - - If the PUSCH for mode-A UEI report is scheduled by NW, UE shall transmit regardless of cell DRX, no MAC spec. impact. - - UE does not transmit mode-A UEI report PUCCH outside cell DRX Active Period. - - UE does not transmit PUCCH/PUSCH for mode-B UEI report if either the PUCCH or PUSCH (first valid type-1 CG occasion) for a report is outside cell DRX Active Period. sDCI mTRP 2TA (in case of no PL offset) is supported for legacy Rel-18 LTM cell switch with no MAC specification impact.

Agreements on Asymmetric DL sTRP/UL mTRP

RAN2 clarifies that the parameter that enables Rel-19 two-TA configuration (singleDCI-MultiTRP-2TA) is applied for both Rel-19 intra-cell 2TA without mDCI mTRP (including asymmetric TRP and symmetric TRP) and inter-cell 2TA without mDCI mTRP (i.e., ICBM). The corresponding parameter name and its field description will be updated to capture this agreement, and the agreement will be informed to RAN1 by LS.

Agreements on other issues

For both mode-A and mode-B UEI beam report, When UEIBR is initiated but TAT expires, UE does not initiate RACH. If TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed. NW does not configure r15 UL skipping skipUplinkTxDynamic for a cell group if UEI-CSI reporting is configured for any cell of the cell group, this is captured in RRC skipUplinkTxDynamic field description, no MAC change for r15 UL skipping. Keep UEI report parameters within CSI-ReportConfig. No specification impact. referenceAntennaPort is introduced in RRC.

#### 2.2.2	Remaining Open issues

- Maintenance on RRC and MAC aspects.

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#116 (2025-08), the following agreements were made.

RF

Close all open issues for core part.

CR for TS 38.101-1 was agreed to introduce FR1 3Tx RF requirements (R4-2511773)

- Add 3Tx PC2 MPR requirements when the UE indicates ul-FullPwrMode-r16 for the band

CR on FR2 RF requirements for NR_MIMO_Ph5 was agreed for beam correspondence requirements for non-simultaneous transmission in multiple direction (R4-2511772)

- the IE unifiedTCI-StateType is configured as ‘separate’

- the IE pl-Offset for separate DL/UL TCI state(s)in the UL TCI state is not configured

- the IE referenceSignal-r17 associated with the UL TCI state differs from the qcl-Type set to ‘typeD’ associated with the DL TCI state

RRM

Close all open issues for core part.

R4-2509238 Big CR for RRM requirements of NR MIMO Phase 5 will be agreed in RAN4#116 post meeting.

UE-initiated/event-driven beam management

All draft CRs for UE-initiated/event-driven beam management are endorsed.

Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported?

Agreement:

For the window-based solution:

if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel

- eventInstanceCount-r19 is configured by NW

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

- TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event

The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

Send the LS capturing the RAN4 agreements to RAN1/2, and the action requested from RAN4 to RAN1/2 is to take RAN4 agreements into account.

Issue 1-2: Whether to specify new unified TCI switching delay requirements?

Agreement:

- This issue is closed.

Issue 1-3: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

Agreement:

- No need to capture this scenario in RAN4 requirements.

Issue 1-4: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

Agreement:

- For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Issue 1-6 (New): Whether RAN4 to specify minimum interval requirement between two event instances?

Agreement:

- RAN4 not to specify minimum interval requirement between two event instances.

Issue 1-8: UE-initiated/event-driven beam management for FR1

Agreement:

- Agree for FR1 support and check the update in draft CRs R4-2509236 & R4-2509485 directly.

Issue 1-9: L1-RSRP measurement period for event-7

Agreement:

- Check the update in draft CRs R4-2509236 & R4-2509485 directly, further revision in dCRs.

CJT calibration (CJTC) reporting

All draft CRs for CJT calibration (CJTC) reporting are endorsed.

Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

Define core requirements based on 1 sample only and further discuss the accuracy and SNR in the performance part.

Companies are encouraged to discuss how to derive the accuracy performance based on the simulation results.

Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- Reuse the legacy L1-RSPR scheduling restriction in FR1 as baseline for delay and frequency offset measurement.

  - There are no scheduling restrictions due to delay and frequency offset measurement performed on CSI-RS (tracking RS) with the same SCS as PDSCH/PDCCH in FR1.

  - For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to delay and frequency offset measurement based on CSI-RS (tracking RS).

  - For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to delay and frequency offset measurement based on CSI-RS (tracking RS).

    - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the CSI-RS (tracking RS) indexes configured for delay and frequency offset measurement.

Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration?

Agreement:

- Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

  - Measurement restriction does apply,

    - If UE does not support simultaneousRxDataSSB-DiffNumerology and

    - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

  - Measurement restriction does not apply,

    - If UE support simultaneousRxDataSSB-DiffNumerology or

- SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

Asymmetric DL-sTRP/UL-mTRP

Draft CRs for Asymmetric DL-sTRP/UL-mTRP are endorsed.

Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

- For DL reference point, agree on the concept of proposal P1c. For scenario 1, it is proposed that, for both TAGs, the reference point for PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DL or Joint TCI state.

Agreement:

- The same offset for both of TRPs.

Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

For scenario 1, same gradual timing adjustment applies for each TAG and shall be met for each TAG.

Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements?

Agreement:

- For scenario#1, no MRTD requirements.

- For MTTD requirements:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

Issue 2-4: For Scenario#2, how to update PRACH and PUCCH/PUSCH/SRS transmission requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

Agreement:

Confirm the previous agreement and remove the remaining brackets [].

Demod

MIMO Ph5 demod part is on schedule. We reach some agreements for test assumptions. WF approved as R4-2512527.

Issue 1-1-4: Clarify if UE demodulation requirements should be specified for UE reporting enhancement for CJT

Agreement:

- RAN4 does not introduce demodulation requirements for reporting enhancements for CJT deployments.

For Sub-topic 1-2 Test assumptions for typeI-SinglePanel-r19

Issue 1-2-2: N1 and N2 configuration for 64 CSI-RS ports

Agreement:

- (N1, N2) = (8,4)

Issue 1-2-3: portMappingMethod for 64 CSI-RS ports

Agreement:

- Method1

Issue 1-2-4: MCS level for 64 CSI-RS ports

Agreement:

- Start simulation work with both MCS20 and MCS22. Down select further based on simulation results.

Issue 1-2-7: Rule for CQI/RI/PMI delay

Agreement:

- RAN4 set the CQI/RI/PMI delay according to the number of CSI-RS ports and CSI processing Capability 2.

Issue 1-2-8: CQI/RI/PMI delay for FDD 64CSI-RS ports

Agreement:

- Apply slot n+4, PMI delay (from the PMI estimation slot to the PMI apply slot) 11ms, Report slot offset = [7], for FDD SCS=15kHz

- Interested companies are welcomed to provide results when the reported PMI is applied at slot n+7

Issue 1-2-9: CQI/RI/PMI delay for TDD 64CSI-RS ports

Agreement:

- Apply slot n+6, PMI delay (from the PMI estimation slot to the PMI apply slot) 11.5ms, Report slot offset = 17, for TDD SCS=30kHz

Issue 1-2-11: Correlation configuration

Agreement:

- XP-medium as starting point for simulation results alignment.

Issue 1-2-12: Whether to specify requirements for 128 CSI-RS ports?

Way forward:

- If introduced, 128 ports for TDD only. Introduce applicability rules as if 128 ports cases could pass, UE can skip corresponding 64 ports test.

For Issue 1-3 Test assumptions for eTypeII-r19 (if introduced)

Issue 1-3-1: Test metric

Agreement:

- Reuse the test metric for eType-II PMI reporting requirements, throughput ratio between the following PMI and random Rel-19 refined Type-I PMI Mode A.

Issue 1-3-2: Duplex mode

Agreement:

- Both FDD and TDD

Issue 1-3-3: Number of Rx antennas

Agreement:

- Both 2Rx and 4Rx

Issue 1-3-4: Propagation channel

Agreement:

- TDLA30-5

Issue 1-3-5: Correlation configuration

Agreement:

- XP-Medium

Issue 1-3-6: typeII-ri-restriction-r19

Agreement:

- Rank2

Issue 1-3-7: Number of CSI-RS ports

Agreement:

- 64 ports

Issue 1-3-8: N1 and N2 configuration for 64 CSI-RS ports

Agreement:

- (8,4)

Issue 1-3-9: (Number of aggregated NZP CSI-RS resources) K for 64 CSI-RS ports

Agreement:

- K=2

Issue 1-3-10: MCS level

Agreement:

- Use the same MCS value(s) as Type I codebook

Issue 1-3-11: numberOfPMI-SubbandsPerCQI-subband-r19

Agreement:

- Set as 1

Issue 1-3-12: paramCombination-r19

Agreement:

- Set paramCombination-r19 as 6 (L =4, pν =1/2, β=1/2) as the starting point.

Issue 1-3-13: CQI/RI/PMI delay

Agreement:

- Based on capability 2

Issue 1-3-14: Other parameters

Agreement:

- Reuse the test parameters of the existing Rel-16 eType-II PMI reporting requirements with 16 CSI-RS ports as baseline.

For Issue 2-1 Test assumptions for PUSCH demodulation requirements (if introduced)

Issue 2-2-1: Duplex mode

Agreement:

- Both FDD and TDD

Issue 2-2-2: Number of Rx antennas

Agreement:

- Both 4Rx and 8Rx

Issue 2-2-3: SCS and CHBW for FDD

Agreement:

- 15kHz SCS, 5MHz, 20MHz, 50MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

Issue 2-2-4: SCS and CHBW for TDD

Agreement:

- 30kHz SCS, 10MHz, 40MHz, 100MHz as starting point, further decision based on simulation results. If significant performance differences are observed between channel bandwidths for each SCS, typical bandwidth may be considered in the requirements to ensure comprehensive test coverage.

Issue 2-2-5: Number of layers

Agreement:

- Only 3 layers with TPMI index 0

Issue 2-2-6: Number of DMRS symbols

Agreement:

- 1+1

Issue 2-2-7: DMRS ports

Agreement:

- Rel-15 DM-RS port as {0,1,2}

Issue 2-2-8: Propagation channel and MCS level

Agreement:

- TDLB100-400, MCS 2

- TDLC300-100, MCS 16

- TDLA30-10, MCS 20

Issue 2-2-9: PUSCH mapping type

Agreement:

- Both mapping type A and type B

Issue 2-2-10: other parameters

Agreement:

- Existing simulation assumption for 4Tx requirement can be reused as reference

#### 2.4.2	Remaining Open issues

RF

- None. All open issues for core part are closed.

RRM

- FFS on Test cases for UE-initiated/event-driven beam management for performance part.

- FFS on Test cases for CJT calibration (CJTC) reporting for performance part.

- FFS on accuracy requirements for CJT calibration (CJTC) reporting for performance part.

- FFS on Test cases for Asymmetric DL-sTRP/UL-mTRP for performance part.

Demod

- FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19

- FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19. Interested companies are welcomed to provide simulation results next meeting

- Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19

- Supporters for introducing SRS port grouping related test requirements to provide more details on the test setup

- FFS on the necessary details for a new test framework of performance requirements for the CJT calibration offsets, i.e., TO, CFO and PO. Part of the work would be to determine whether random PMI can be used as viable/reliable option, or the follow PMI is required during the CJT test

- FFS on introducing new PMI reporting test for CJTC Dd and Rel-18 eType-II CJT CSI reports with delay offset compensation

- FFS on Test metric for 64 CSI-RS ports TypeI Rel-19 codebook

- FFS on CSI-RS configuration for 64 CSI-RS ports TypeI Rel-19 codebook

- FFS on Scaling factors for MIMO correlation matrices

- FFS on specify requirements for 128 CSI-RS ports.

- FFS on RAN4 should specify PUSCH demodulation requirements for 3-antenna-port non-coherence codebook-based UL transmissions. Interested companies are welcomed to provide simulation results next meeting

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template