3GPP TSG RAN Meeting #109                                 RP-252090

Beijing, China, Sep 15-18, 2025

Source: 	CMCC

Title:	Views on Migration and Interworking of 6G RAN

Agenda item:	8.2.1.3

Document for:	Discussion

- Background

The study of 6G has been initiated in 3GPP, and in the early stage of 6G construction, the co-existence of 5G network and 6G network is an essential topic. In this contribution we will provide our view on the potential issues related to the migration and interworking between 6G and 5G.

- Discussion

Considering the co-existence of 6G network and 5G network, the interworking between the two networks should be discussed. In our view, at least the following issues should be considered:

- Inter-RAT mobility between 6G and 5G

- Multi-RAT spectrum sharing for 6G

Inter-RAT mobility between 6G and 5G

The inter-RAT mobility is to ensure UE’s service continuity when it moves between a 6G cell and a 5G cell, the direct way as defined in other generation is inter-RAT handover, which involves the participant of core networks. And we think this should be inherited in 6G as the baseline of inter-RAT mobility between 6G and 5G. In this way, the UE registers in one cell, for example 6G cell and keeps the link with 6G network, and when the inter-RAT handover procedure to 5G ends, the 5G network maintains UE’s registration information and builds the link between the UE and 5G network, while the link between the UE and 6G network is released. For inter-RAT handover, the interruption of UE’s data transmission cannot be avoided.

Proposal 1: Inter-RAT handover between 6G and 5G is supported.

As mentioned above, inter-RAT handover is a relatively simple way to support the inter-RAT mobility, as we can draw on the solutions of other generations. However, the corresponding interruption during inter-RAT handover cannot be avoided. For 6G, we may aim for a solution with 0 interruption during inter-RAT mobility, or to minimize the interruption time. In 5G, one migration solution to achieve this objective is Dual connectivity (DC). Multiple DC options are discussed and specified in 5G, while only a few options are commercialized, due to the complexity caused in RAN. To avoid the impact and extra complexity in RAN and the potential heavy specification workload due to multiple DC options discussion, we prefer to not support DC in 6G, while Dual steer/dual stack mechanism is a more suitable solution to be considered as a migration option for 6G in our opinion. With Dual steer/dual stack, the UE maintains two separate links and registration contexts in the 6G and 5G core networks respectively, and the resources of both networks can be utilized flexibly. When the UE moves between the 6G and 5G cells, quick and dynamic link switching for the UE can be achieved, while the interruption can be avoided or reduced.

Proposal 2: Dual steer/Dual stacks in 6G and 5G can be considered as an interworking option.

Proposal 3: DC in RAN is not supported in 6G.

Multi-RAT spectrum sharing for 6G

In 5G, dynamic spectrum sharing is supported, which can improve the resource efficiency and satisfy different service requirements. Besides, it also helps to the smooth migration from 4G to 5G, reduce operator’s construction costs and increase the flexibility of the network. Taking these advantages into consideration, multi-RAT spectrum sharing, like spectrum sharing for 6G and 5G, should be supported. The spectrum allocation or sharing between 6G RAN and 5G RAN, should be decided by the 6G RAN, considering the traffic demand, service requirements and operator policies. MRSS resource allocation granularity in the time/frequency domain, as well as the potential guards between 6G and 5G resources are to be determined by the study on 6G. What’s more, MRSS can be supported in co-located case of 6G RAN and 5G RAN, also in the none co-located case. And MRSS are applicated for paired and unpaired spectrum.

6G RAN should study efficient rate matching mechanism to rate match around 5G signals, include 5G SSB, and 5G RS in DL. However, 5G-6G sharing of SSB or RSs add restrictions to 6GR designs. Especially on new spectrum which does not have 5G-6G MRSS. Hence, 5G-6G MRSS should focused on rate match around 5G signals.

Proposal 4: Efficient rate matching mechanism in 6GR to rate match around 5G NR signals should be studied in 5G-6G MRSS.

- Text Proposal for TR 38.914

Based on the discussion in Section 2, we propose the following text proposal for TR 38.914 for the general requirements for architecture of 6G RAN.

# 5.2	Requirements for architecture and migration

### 5.2.X	Requirements for migration of 6G RAN

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support interworking between the 6G RAT and 5G RAT.

-	Considering high performing inter-RAT mobility between 6G RAT and 5G RAT, taking inter-RAT handover as baseline, and Dual steer/stacks as an optional interworking option.

-	The 6GR support spectrum sharing with 5G. Efficient rate matching mechanism in 6GR to rate match around 5G NR signals should be considered.

- Conclusion

In this contribution we will provide our view on inter-RAT mobility and MRSS for 6G, which are related to the migration and interworking between 6G and 5G, and propose to add the text proposal in Section 3 to TR 38.914 Clause 5.2. Following are the proposals:

Proposal 1: Inter-RAT handover between 6G and 5G is supported.

Proposal 2: Dual steer/Dual stacks in 6G and 5G can be considered as an interworking option.

Proposal 3: DC in RAN is not supported in 6G.

Proposal 4: Efficient rate matching mechanism in 6GR to rate match around 5G NR signals should be studied in 5G-6G MRSS.