3GPP TSG RAN Meeting #109	RP-252096

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	CMCC

Title:	Consideration on 6G Sensing Services

Document for:	Discussion

# 1 Introduction

In RAN#108, the new study item on 6G Radio was approved [1], where the study on Sensing was agreed with the following agreed objectives. The agreed objectives are listed below.

(9) Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914]

a) PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4]

b) Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1]

c) Aspects of integration with communication services [RAN1]

d) higher layer procedures and protocol aspects [RAN2]

e) RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4]

In this contribution, we provide our initial consideration on 6G sensing services.

# 2 Discussion

Integrated Sensing and Communication (ISAC) has been identified as one of the six usage scenarios for IMT-2030, providing sensing services for low-altitude economy, traffic surveillance, intrusion detection and micro-deformation detection while also assisting communication services. In 5G-A, only TRP mono-static sensing for UAV use case is supported, the performance and use cases are both limited. 6G is expected to support all six sensing modes (i.e., TRP mono-static, TRP-TRP bi-static, TRP-UE bi-static, UE-TRP bi-static, UE mono-static and UE-UE bi-static) including both mono-static and bi-static with both TRP and UE involved to explore more use cases and to further improve sensing performance.

Proposal 1: All six sensing modes, including both mono-static and bi-static with both TRP and UE involved, should be studied in 6G.

However, to fully explore the potential of bi-static sensing, especially with UE involved, synchronization of time, frequency and beam steering must be studied. Meanwhile, to fully support these sensing modes simultaneously, interference coordination and elimination are essential. Interference is caused by not only the sensing devices nearby, but those communication devices as well.

Proposal 2: Synchronization mechanism and interference coordination and elimination should be studied in 6G in order to support multiple sensing modes.

Use cases including UAV, automotive and human are identified in TR 22.837[2], and requirements for such are defined in TS 22.137[3]. These use cases, which are directly related to low-altitude economy and traffic surveillance, are still important for 6G sensing.

Although UAV sensing is already covered in 5G-A studies, the performance are limited and does not meet the requirements of safety and low-altitude economy defined in TR 22.870[4]. Without limitations from NR reference signals and structures, sensing performances on these cases can be further improved. It enables new services like road digitalization. Meanwhile, sensing assisting communication is expected to be a landmark which differs the 6GR ISAC from NR ISAC. Functions including beam management and channel estimation are highly correlated to the physical world. By revealing surrounding environment, sensing will obviously benefit communication.

Proposal 3: At least the following use cases for sensing should be considered in 6G:

- UAV

- Automotive

- Human

- Environment awareness/reconstruction to assist communication

In RAN1#122, miss detection probability and false alarm probability are defined explicitly for NR ISAC evaluation. These two metrics are also identified in TS 22.137 and TR 22.870. A reliable sensing performance is vital for UAV supervision and low-altitude economy, which are the clearest use cases we identified in the industrial aspect. From operator’s perspective, we care most about safety. Thus, at least for UAV use case, both miss detection probability and false alarm probability should be low.

Proposal 4: At least for UAV use case, requirements on miss detection probability and false alarm probability shall be less than 1% in 6G.

According to requirements identified in TR 22.870, sensing accuracy and resolution can be very high (e.g. 0.1m accuracy for collaborative robots using digital twinning and 0.1m resolution for enhanced XR navigation). Such high requirements may be necessary for some new services. However, it also leads to high complexity of sensing algorithms, which require high-performance hardware.

Observation 1: High requirements on sensing accuracy and resolution may lead to complicated algorithms designed for the specific use case, due to the following factors:

- Motion compensation: To achieve a high SNR and estimate velocity, accumulation among several sensing symbols is required. Target’s motion among sensing symbols within a coherent processing interval (CPI) have to be compensated.  The error of motion compensation should be bounded by range resolution.

- Synchronization: To achieve coherent accumulation among sensing symbols, synchronization of time, frequency and even phase should be guaranteed during a relatively long CPI. Adaptive algorithms to deal with residuals caused by non-ideal synchronization are required.

Therefore, use case specific algorithms may be very common for future 6G sensing services. Considering performance enhancement and possible new sensing services after 6G day-1, sensing algorithms may be upgraded frequently.

Observation 2: From a deployment perspective, implementing the parts of the sensing processing that involve complex algorithms at the sensing function (SF) with high-performance hardware may have advantages in maintenance.

To support potential new sensing services and advanced algorithms in the radar domain, the type of sensing data reported to the SF should be studied in conjunction with specific sensing services and, to the greatest extent possible, incorporated into a flexible reporting framework.

Proposal 5: To support new sensing services beyond object detection and to support possible advanced sensing algorithms, a flexible reporting framework which supports multi-level sensing data feedback should be studied in 6G.

# 3 Text proposal

Based on the discussion above we propose the following text proposal to TR 38.914.

Table 1 text proposal to TR 38.914

All six sensing modes, including TRP mono-static, TRP-TRP bi-static, TRP-UE bi-static, UE-TRP bi-static, UE mono-static and UE-UE bi-static, should be studied. At least the following use cases for sensing in 6G should be included: – UAV – Automotive – Human – Environment awareness/reconstruction to assist communication

Proposal 6: To capture the text proposal in Table 1 into TR 38.914.

# 4 Conclusion

In this paper, we provide our consideration on the 6G sensing services.

Observation 1: High requirements on sensing accuracy and resolution may lead to complicated algorithms designed for the specific use case, due to the following factors:

- Motion compensation: To achieve a high SNR and estimate velocity, accumulation among several sensing symbols is required. Target’s motion among sensing symbols within a coherent processing interval (CPI) have to be compensated.  The error of motion compensation should be bounded by range resolution.

- Synchronization: To achieve coherent accumulation among sensing symbols, synchronization of time, frequency and even phase should be guaranteed during a relatively long CPI. Adaptive algorithms to deal with residuals caused by non-ideal synchronization are required.

Observation 2: From a deployment perspective, implementing the parts of the sensing processing that involve complex algorithms at the sensing function (SF) with high-performance hardware may have advantages in maintenance.

Proposal 1: All six sensing modes, including both mono-static and bi-static with both TRP and UE involved, should be studied in 6G.

Proposal 2: Synchronization mechanism and interference coordination and elimination should be studied in 6G in order to support multiple sensing modes.

Proposal 3: At least the following use cases for sensing should be considered in 6G:

- UAV

- Automotive

- Human

- Environment awareness/reconstruction to assist communication

Proposal 4: At least for UAV use case, requirements on miss detection probability and false alarm probability shall be less than 1% in 6G.

Proposal 5: To support new sensing services beyond object detection and to support possible advanced sensing algorithms, a flexible reporting framework which supports multi-level sensing data feedback should be studied in 6G.

Proposal 6: To capture the text proposal in Table 1 into TR 38.914.

# Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator).

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication".

- TR 22.870: "Study on 6G Use Cases and Service Requirements"