3GPP TSG RAN Meeting #109	RP-252166

# Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.4

Source:	Samsung

Title:	Requirements for data collection

Document for:	Discussion and Decision

1	Introduction

In this contribution, we propose requirements for data collection framework.

2	Discussion

Data collection and management becomes more important in 6G. Data collection is not only needed for new services (e.g., AI/ML, sensing) but also for legacy services. SON/MDT/Trace/QoE have been defined in 5G for data collection for legacy services. In 5G, AI/ML for NG-RAN also uses the data collected from the UE via MDT.

Even for MDT, different mechanisms are defined for data collection, e.g., logged MDT, immediate MDT, signalling-based MDT, and management-based MDT. For Trace and QoE, there are signalling-based and management-based procedures for data collection.

Diverse approaches in data gathering introduced significant complexity. For 6G, it is crucial to reduce the number of options. Data collection and management should be designed to support various types of data and for accommodating large volume data transmission and different QoS requirements for new services.

In order to reduce the number of options, SON/MDT/Trace shall be discussed in the first release. Then it can be discussed together with new services and evaluated whether a unified data collection framework can be defined not only for legacy services but also for new services e.g. AI/ML, sensing.

The unified data collection framework will improve the data collection efficiency and eliminate the need for defining different data collection mechanisms in subsequent releases, which often arises when the initial release fails to address diverse data collection needs.

3	Proposal

We propose to agree the following text proposal in TR 38.914.

=== Start Text Proposal ===

5.4	Requirements of New and Existing Services

< Proposed new section >

5.4.x data collection framework

Unified data collection framework should be designed for various types of data (e.g., AI/ML, Sensing). The unified data collection framework supports large volume data transmission and different QoS requirements. Furthermore, the unified data collection framework should also be used for legacy services e.g. MDT-based data collection if applicable.

< End new section >

=== End Text Proposal ===