3GPP TSG RAN    Meeting #109	RP-252199

Beijing, China, September 15-18, 2025

Agenda item:	8.2.2

Source:	Nokia

Title:	6G device types

Document for:	Approval

# 1	Introduction

RAN1#122 started 6G study item and made the first agreements for 6G. One of the topics discussed in RAN1#122 was diverse device types. RAN1 made the following agreements and identified need for further discussion and confirmation in RAN:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary.

In this contribution we discuss diverse device types and propose a way forward how RAN can help RAN1 and other RAN WGs in their work in natively supporting diverse device types in 6G radio while avoiding technology and market fragmentation with large number of UE capabilities.

# 2	Discussion

High market volumes typically justify sufficient investments and good market success for 3GPP technologies and features, in contrast to many incremental features and large number of UE capabilities, which fragment the market, increase complexity and delay implementations and deployments. One of the important 5G learnings is to avoid large number of UE capabilities and many incremental feature updates.

To take the learnings from 5G to 6G we see that 6G should be designed to natively support a variety of device types optimized for different use cases, form factors, hardware and processing power requirements, bit rates and cost constraints etc, and in 3GPP we need to define device types in a lightweighted manner. The definition of device types should enable enough market opportunities for variety of devices with different form factors, hardware and processing capabilities, data rates and cost constraints now and in the future. Device types should define UE’s minimum baseline characteristics and functionalities per device type instead of separate UE capabilities for all baseline functionalities. Naturally UEs should be allowed to support and indicate to a network better and higher capabilities and functionalities than defined by a given device type as the minimum UE support.

Proposal 1: 6G device types define UE’s minimum characteristics and functionalities per device type. UEs may support and indicate to a network better and higher capabilities and functionalities than defined by a given device type.

Figure 1: Example of different 6G device types

The first aspects to discuss and agree for 6G device types are how many different device types are needed, what are the key and necessary differentiating UE capabilities and characteristics, especially related to UE L1 and RF are what are other important factors that need to be considered for device types like device form factors, power consumption/physical battery size, use cases etc.

To facilitate 6G studies in RAN working groups, like RAN1 already indicated, we propose that RAN#109 will identify key UE characteristics and functionalities (especially related to L1 and RF), which should be defined as a part of device types. It would also be beneficial for RAN#109 to discuss how many device types should be defined in 6G.

Proposal 2: RAN#109 to identify key UE characteristics and functionalities related to UE L1 and RF for each device type.

Proposal 3: RAN#109 initiate discussion on how many different device types are needed for 6G.

In the following Table 1 we present an example what kind of UE baseline capabilities could be agreed and defined as part of device types.

Table 1: Example of device types and key UE characteristics and functionalities to defined as part of device type definitions

| Key UE characteristics and functionalities | Device Type 1 e.g. mIoT | Device Type 2 e.g. wearables | Device Type 3 e.g. smart phone | Device Type 4 e.g. FWA/CPE |
| --- | --- | --- | --- | --- |
| Minimum of Maximum Channel Bandwidth |  |  |  |  |
| Number of Rx antennas |  |  |  |  |
| Number of Tx antennas |  |  |  |  |
| Number of DL MIMO layers |  |  |  |  |
| Support of DL & UL 6G CA |  |  |  |  |
| UE power class |  |  |  |  |
| UE processing capabilities/times |  |  |  |  |

# 3	Conclusion

In this contribution we have discussed the need for defining device types in 6G to indicate minimum UE support for baseline UE capabilities and functionalities, especially related to L1 and RF. Based on the discussion we make the following proposals:

Proposal 1: 6G device types define UE’s minimum characteristics and functionalities per device type. UEs may support and indicate to a network better and higher capabilities and functionalities than defined by a given device type.

Proposal 2: RAN#109 to identify key UE characteristics and functionalities related to UE L1 and RF for each device type

Proposal 3: RAN#109 initiate discussion on how many different device types are needed for 6G

To allow RAN1 to progress its 6G studies we see that it would be important for RAN#109 to provide RAN1 an additional guidance on 6G device types and key UE baseline capabilities, which will be defined as part of device types.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)