3GPP TSG-RAN5 Meeting #108	R5-254922

# Bengaluru, India, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5482 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Inter-RAT cell reselection test case 6.2.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on AP#107.01, Inter-RAT cell reselection test case 6.2.3.6 needs modification to cover UEs supporting/not supporting protection against improper reselection to GERAN/UTRAN feature. In test case 6.2.3.6, TP3 checks whether the UE applies the cell reselection priorities broadcast in system information after timer t320 expiry. As per the system information following step 15, the UTRA cell is assigned a higher priority (5) than the E-UTRA cell (4). Therefore, a UE that supports this feature will not perform a high-priority reselection from E-UTRA to UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Steps 14-18 are made not applicable to UEs supporting protection against improper reselection to GERAN/UTRAN. Accordingly updated conformance requirements and specific message content table Table 6.2.3.6.3.3-7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case by not camping on UTRAN Cell 5 at step 18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254922 is the final version of R5-254495. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.2.3.6	Inter-RAT Cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle according to RAT priority provided by dedicated signalling

6.2.3.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority RAT }

then { UE reselects the cell which belongs to the higher priority RAT }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority RAT }

then { UE reselects the cell which belongs to the lower priority RAT }

}

(3)

with { UE not supporting protection against improper reselection to GERAN/UTRAN in E-UTRA RRC_IDLE state having received an RRCConnectionRelease message including a cellReselectionPriority }

ensure that {

when { T320 expires }

then { UE reselects a cell by applying the cellReselectionPriority broadcast in the system information }

}

(4)

with { UE in E-UTRA RRC_IDLE state, and reselection priorities have been provided in dedicated signalling }

ensure that {

when { T320 has not expired }

then { The UE shall ignore all the priorities provided in system information, and not perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies for which the UE doesn’t have a priority provided }

}

6.2.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.1 and 5.2.4.5 and TS 36.331, clause 5.3.8.3 and 5.3.8.4.

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

- the UE enters RRC_CONNECTED state; or

- the optional validity time of dedicated priorities (T320) expires; or

- a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfills the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low  during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

...

[Rel-8]

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

[Rel-18]

If the UE supports the protection against improper reselection to GERAN/UTRAN then:

-	When the UE evaluates serving cell conditions for the purpose of cell reselection to GERAN/UTRAN FDD/TDD by applying the reselection priorities provided in AS security protected dedicated signalling with a GERAN/UTRAN FDD/TDD frequency as lower priority compared to the current E-UTRAN frequency, or when the UE applies GERAN/UTRAN FDD/TDD priorities provided in system information, the UE shall:

-	treat GERAN and/or UTRAN FDD and/or UTRAN TDD frequencies as the lower priority compared to E-UTRAN;

-	set the value of ThreshServing, LowP to 6 dB if the value received in the system information is higher than 6 dB;

-	set the value of Q-RxLevMin to -116 dBm if the value received in SIB1 is higher than -116 dBm;

-	set the values of Pcompensation and Qoffsettemp to 0.

The UE shall not perform cell reselection to NR or UTRAN FDD cells for which the cell selection criterion S is not fulfilled.

…

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the releaseCause received in the RRCConnectionRelease message indicates 'loadBalancingTAURequired':

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

[TS 36.331, clause 5.3.8.4]

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information;

6.2.3.6.3	Test description

6.2.3.6.3.1	Pre-test conditions

System Simulator:

-	Cell 1(E-UTRA) and Cell 5(UTRA).

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

6.2.3.6.3.2	Test procedure sequence

Table 6.2.3.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", "T2", "T3", "T4" and "T5" are to be applied subsequently. The exact instants on which these values are applied are described in the texts in this clause.

Table 6.2.3.6.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are assigned to satisfy SrxlevCell 1 > 0 and SrxlevCell 5 < 0. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -90 | - | The power level values are assigned to satisfy Threshx,high < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that camping on Cell 1 is guaranteed. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -115 | - | The power level values are assigned to satisfy Srxlevcell 1 < Threshserving, low and Threshx,low < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |
| T4 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are such that camping on Cell 1 is guaranteed. |
|  | CPICH Ec | dBm/3.84 MHz | - | -90 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -92 |  |
| T5 | Cell-specific RS EPRE | dBm/15kHz | -70 | - | The power level values are assigned to satisfy Srxlevcell 1 > Threshserving, low and Threshx,high < Srxlevcell 5. |
|  | CPICH Ec | dBm/3.84 MHz | - | -65 |  |
|  | PCCPCH RSCP | dBm/1.28MHz |  | -65 |  |

Table 6.2.3.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease message with an idleModeMobilityControlInfo including a cellReselectionPriority on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 2 | The SS changes Cell 5 level according to row "T1" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 3 | Check: Does the UE transmit an RRC CONNECTION REQUEST message within the next 30s on Cell 5? | --> | RRC CONNECTION REQUEST | 4 | F |
| 4 | The SS transmits a Paging message including matched identity on Cell 1. | <-- | Paging | - | - |
| 5 | UE transmit an RRCConnectionRequest message on Cell 1. | --> | RRCConnectionRequest | - | - |
| 6 | The SS transmits an RRCConnectionSetup message on Cell 1. | <-- | RRCConnectionSetup | - | - |
| 7 | The UE transmits an RRCConnectionSetupComplete on Cell 1. | --> | RRCConnectionSetupComplete | - | - |
| 8 | The SS transmits an RRCConnectionRelease message with an idleModeMobilityControlInfo including a cellReselectionPriority on Cell 1. | <-- | RRCConnectionRelease | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | - |
| - | Exception: Steps 9Aa1 to 9Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 9Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 9Aa1a1-9Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 9Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 9Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 9Aa1b1 | The 6s timer expires | - | - | - | - |
| 10 | The SS changes Cell 1 and Cell 5 parameters according to the row "T2" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 11A-11G | Generic test procedure in TS 36.508 subclause 6.4.2.7 A-1 from steps 1-7 is performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | Exception: Steps 11Ha1 to 11Ha2b1 describe optional behaviour if UE sends optional IMS deregistration at steps 9Aa1 to 9Aa1b1 | - | - | - | - |
| 11Ha1-11Ha2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 11I | The SS releases RRC Connection |  |  |  |  |
| 12 | The SS changes Cell 1 and Cell 5 parameters according to the row "T3" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 2 | - |
| - | Exception: Steps 13Aa1 to 13Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 13Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 13Aa1a1-13Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 13Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 13Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 13Aa1b1 | The 6s timer expires | - | - | - | - |
| 14 | Void |  |  |  |  |
| - | EXCEPTION: Steps 14Aa1 to 14Aa13 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that takes place if the condition is met. | - | - | - | - |
| 14Aa1 | IF NOT pc_protection_against_improperReselection_to_GERAN_UTRAN THEN the SS changes Cell 1 and Cell 5 parameters according to the row "T4" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 14Aa2-14Aa8 | Generic test procedure in TS 36.508 subclause 6.4.2.7 A-1 from steps 1-7 are performed on Cell 1. NOTE: The UE performs a TAU procedure. | - | - | - | - |
| - | EXCEPTION: Steps 14Aa9a1 to 14Aa9a2b1 describe optional behaviour if UE sends IMS deregistration at steps 13Aa1 to 13Aa1b1 | - | - | - | - |
| 14Aa9a1-14Aa9a2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed NOTE: Timer_1 value of 6s used in Step 10a1 of procedure. | - | - | - | - |
| 14Aa10 | The SS releases RRC Connection |  |  |  |  |
| 14Aa11 | The SS changes Cell 1 and Cell 5 parameters according to the row "T5" in table 6.2.3.6.3.2-1. | - | - | - | - |
| 14Aa12 | Wait for 5 minutes from step 15 to ensure that T320 expires. | - | - | - | - |
| 14Aa13 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? | - | - | 3 | - |
| 15-18 | Void |  |  |  |  |

6.2.3.6.3.3	Specific message contents

Table 6.2.3.6.3.3-1: Void

Table 6.2.3.6.3.3-2: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.6.3.3-3: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.6.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.6.3.3-4: RRCConnectionRelease (Step 1, Table 6.2.3.6.3.2-2)

| Derivation path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| IdleModeMobilityControlInfo ::= SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD | Not present |  | UTRA-TDD |
| freqPriorityListUTRA-TDD | Not present |  | UTRA-FDD |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-5: RRCConnectionRelease (step 8, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-6: RRCConnectionRelease (step 11, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-7: RRCConnectionRelease (step 14Aa10, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo SEQUENCE { |  |  |  |
| freqPriorityListEUTRA SEQUENCE (SIZE (1.. maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| carrierFreq[1] | maxEARFCN |  | Band > 64 |
| cellReselectionPriority[1] | 4 |  |  |
| } |  |  |  |
| freqPriorityListGERAN | Not present |  |  |
| freqPriorityListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-FDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| freqPriorityListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | 1 entry |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| bandClassPriorityListHRPD | Not present |  |  |
| bandClassPriorityList1XRTT | Not present |  |  |
| t320 | min5 |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | Band > 64 |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMobilityControlInfo-v9e0 SEQUENCE { |  |  |  |
| freqPriorityListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| carrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |
| Band > 64 | If E-UTRA band > 64 is selected |

Table 6.2.3.6.3.3-8: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.6.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 5 |  |  |
| } |  |  |  |

<End of modified section>