3GPP TSG RAN meeting #109									RP-252364

Beijing, China, September 9-13, 2025

## Status Report to TSG

Agenda item:			9.6.1.2

| WI / SI Name | Evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_duplex_evo |  |  |  |  |
| Unique ID | 1020090 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251874 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 25% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xinghua Song, He (Jackson) Wang |
|  | Company | Huawei, Samsung |
|  | Email | songxinghua@huawei.com, h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

In RAN2#131, the following agreements were made.

Agreements on the remaining open issues

Not to support that a further different SSB RSRP threshold is indicated/configured for an SSB or a group of SSBs.

Not to pursue the further optimization of parameter signalling of SBFD RACH configuration.

Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

For the network indicating RO type, use 1 bit signalling (as in the current RRC running CR)

In RO type switching, for the other RO type, UE can select the set of Random Access resources associated with the same feature or feature combination, and with higher Msg1 repetition number, if the set with the same Msg1 repetition number is not available.

In RO type switching, when UE has to select a set of Random Access resources with higher Msg1 repetition number for the other RO type, if there are multiple sets with multiple higher Msg1 repetition numbers available, UE selects the set with next higher Msg1 repetition number.

Completion of the WI

From RAN2 point of view, the Rel-19 work item NR_duplex_evo-Core is completed.

Agreements on RACH

RAN2 assume that when CFRA indicates SBFD RO, the RACH resources for the same RO type is provided for CBRA. FFS if any spec changes is needed.

For RACH configuration Option 2, all the RRC configured parameters are re-initialized after RO type switching.

For RACH Configuration Option 1, sbfd-RACHSingleConfig-preambleReceivedTargetPower is re-initialized after RO type switching.

Can discuss in the RRC CR review the configuration restriction (if needed) for preambleTransMax

For RACH configuration Option 2, PREAMBLE_POWER_RAMPING_STEP and SCALING_FACTOR_BI are re-initialized after RO type switching.

For both RACH configuration Option 1 and RACH configuration Option 2,  PREAMBLE_POWER_RAMPING_COUNTER is not re-initialized after RO type switching.

Can further check the other UE variables in the CR review.

For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two values of preamble power ramping steps is added. Exact change to the MAC spec can be further discussed in the CR review.

Only for RACH configuration Option 1, reuse the rsrp-ThresholdMsg1-RepetitionNum2/4/8 to determine Msg1 repetition number in SBFD RO if sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 is not configured.

For RACH configuration Option 2, determine Msg1 repetition number only by sbfd-RSRP-ThresholdMsg1-RepetitionNum2/4/8 (i.e., not reuse rsrp-ThresholdMsg1-RepetitionNum2/4/8).

Other aspects

RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### ·2.3.1	Agreements

In RAN3#129, the following agreements were made.

Remove the Editor’s Note in section 8.4.y CLI Indication procedure about the procedure text, and update the message name in figure to CLI INDICATION.

Agree that the semantics description of the SBFD Frequency Configuration IE added to the Served Cell Information NR IE in the BLCR to TS38.423 reference the SBFD-Subband-Allocation-r19 IE defined in TS38.331.

Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is a relative index of the CSI-RS resources within the set of resources signalled.

gNB1 serving aggressor UEs will provide SRS-Resource configuration information to gNB2 serving victim UEs

Add a new indication in CLI INDICATION procedure for the gNB serving victim UEs to request for SRS resource configurations from the receiving gNBs.

Use the following IEs for gNB1 serving aggressor UEs to provide SRS-Resource configuration to gNB2 serving victim UEs:

- XnAP: Served Cell Information NR IE

- F1AP: Served Cell Information IE (DU to CU), Neighbour Cell Information List IE in GNB-CU CONFIGURATION UPDATE message (CU to DU).

#### 2.3.2	Remaining Open issues

None

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#116, the following agreements were made.

The agreements were captured in following T-docs:

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2512529 | Way Forward for UE-UE CLI | Charter Communications, Inc., Nokia, Cable Labs, Samsung | Approved |
| R4-2512665 | Way Forward for [116][306] NR_duplex_evo_General | Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE | Approved |
| R4-2512680 | Way Forward for [116][307] NR_duplex_evo_BSRF | Huawei | Approved |
| R4-2511255 | CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS | Samsung | Email Appoved |
| R4-2512673 | Draft CR to TS 38.104 on In-channel adjacent subband leakage for | ZTE Corporation, Sanechips | Endorsed |
| R4-2511256 | draftCR to TS38.104 on transient period requirements for SBFD-capable BS | Samsung | Endorsed |
| R4-2512528 | draftCR to TS38.104 on general clauses for SBFD-capable BS | Samsung, Ericsson | Endorsed |
| R4-2512674 | Draft CR on TS 38.104 to add unwanted emissions for SBFD (NR_duplex_evo-Core) | CMCC | Endorsed |
| R4-2512679 | Draft CR to 38.104 on adding receiver in-channel requirements for SBFD | Ericsson | Endorsed |
| R4-2510966 | Draft CR to 38.104 on adding OTA spatial emission requirements for SBFD | Ericsson | Endorsed |
| R4-2512675 | Draft CR to TS 38.104 on Annex EVM measurement procedure | ZTE Corporation, Sanechips | Endorsed |
| R4-2509326 | draftCR for TS 38 104 to introduce conducted and OTA out-of-band blocking for SBFD | CATT | Endorsed |
| R4-2512678 | draftCR to 38.104 for introduction of receiver spurious and intermodulation requirements | Qualcomm | Endorsed |
| R4-2512676 | Draft CR to 38.104 on Reference sensitivity and dynamic range for SBFD | Huawei, HiSilicon | Endorsed |
| R4-2512677 | Draft CR to 38.104 on in-band selectivity and blocking for SBFD | Huawei, HiSilicon | Endorsed |
| R4-2512133 | WF on RRM requirements for NR_duplex_evo | Huawei | Approved |
| R4-2512132 | Ad-hoc minutes for NR_duplex_evo | Samsung | Approved |
| R4-2512352 | Big CR for RRM requirements of Sub-band full duplex | Huawei, Samsung | Email Appoved |
| R4-2509239 | Draft CR on general L1-CLI-RSSI measurements | Samsung | Endorsed |
| R4-2512191 | Draft CR for L1-CLI-RSSI measurement period | LG Electronics | Endorsed |
| R4-2512192 | DraftCR to update L1-CLI-RSSI measurement requirements | CATT | Endorsed |
| R4-2512326 | draftCR on core requirements for UE-to-UE CLI handling | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2512194 | DraftCR on measurement period for L1-SRS-RSRP measurement | CMCC | Endorsed |
| R4-2512195 | Draft CR on scheduling applicability of R19 SBFD | ZTE Corporation, Sanechips | Endorsed |
| R4-2512196 | Draft CR to 38.133 on SBFD UE-to-UE CLI | Ericsson | Endorsed |
| R4-2512197 | Draft CR on clarification of DL slots and UL slots in SBFD | Apple | Endorsed |
| R4-2510087 | Draft CR on CSI-RS based CBD evaluation period requirements with SBFD | China Telecom | Endorsed |
| R4-2510653 | draftCR on L1-SINR measurement requirements with SBFD | Huawei, HiSilicon | Endorsed |
| R4-2511213 | Draft CR on clarification of L3 CSI-RS measurement requirement for SBFD | MediaTek | Endorsed |
| R4-2511637 | draftCR on CSI-RS based RLM evaluation period requirements with SBFD | Qualcomm | Endorsed |
| R4-2512199 | DraftCR on requirements of CSI-RS based L1-RSRP Reporting with SBFD | OPPO | Endorsed |
| R4-2512615 | Way Forward for [116][328] NR_duplex_evo_demod | Samsung | Endorsed |

WF captured in R4-2512529

With detailed observations provided in R4-2512529, the following bullets are provided in WF summary:

•	Based on simulations, RAN4 acknowledges the issue of UE-to-UE CLI in Urban Hotspot scenario at cell-edge following Rel-18 baseline assumptions given in Annex E, TR 38.858 for the following scenario

o	Urban Hotspot -> Urban Hotspot Scenario 2, Case 1 as reference.

o	Intra-operator adjacent-channel inter-cell UE-to UE CLI where the victim UE is legacy TDD, and the aggressor UE is SBFD-aware UE causing TDD DL TP degradation at cell-edge.

o	Flat ACLR and ACS model based on minimum RAN4 BS and UE ACLR and ACS values.

•	RAN4 has investigated five solutions to the above problem with the following observations

o	Solution 1: Reducing the power of the SBFD-aware UE reduces the TP degradation and can be considered as part of a solution for future releases, and the detailed observations can be found in section 4.1.

o	Solution 2, 3 and 4 could mitigate UE-to-UE CLI. and can be considered as part of a solution for future releases. The detailed observations can be found in section 4.2 to 4.4.

o	Solution-5: Scheduling victim UEs further apart in frequency domain could mitigate UE-to-UE CLI and can be considered as part of a solution for future releases. The detailed observations can be found in section 4.5.

Agreement and WF captured in R4-2512665

Topic #1: General system parameters for SBFD operation

Issue 1-1-1: Declaration and flexibility for SBFD operation

Agreement:

- For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested.

  - The threshold for Prated,c,sys is defined as:

    - X = 36 dBm.

Issue 1-1-2: How to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”

Agreement:

- For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations,

  - The configuration with the narrowest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

  - The configuration with the widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

    - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

    - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

Issue 1-2-1: Restriction on the number of UL subband sizes supported

Agreement:

- For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared

  - Maximum 3 subband configurations allowed for FR1 WA BS

    - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS.

    - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations.

  - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable.

    - The threshold for Prated,c,sys is defined as:

      - X = 36 dBm.

Issue 1-2-2: Restriction on the number of DL subband sizes supported

Agreement:

- For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

  - This restriction shall be applied to FR1 WA BS

  - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable.

    - The threshold for Prated,c,sys is defined as:

      - X = 36 dBm.

Issue 1-2-4: Restriction on DL subband size

Way forward:

- FFS the following aspect in conformance part for testing applicability:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required.

Topic #2: SBFD-aware UE related issues

Issue 2-1-1: The necessity to update for HPUE duty cycle evaluation

Agreement:

- For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

- For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

Issue 2-1-2: If the update is needed, how to introduce in RAN4 spec

No further agreement, and the WF from RAN4#114bis can be followed in the maintenance of Rel-19 SBFD WI.

Topic #4: New BS RF requirements for SBFD

Issue 4-2-1: Adding note for uniform SBFD configuration across sites

Agreement:

- The following note is added as an informative NOTE, with the understanding that such NOTE is not part of RF requirements:

The RF requirements for SBFD-capable BS have been derived based on the simulation assumption that only SBFD-capable BSs are deployed with the identical SBFD configuration under each BS class in the same geographical area.

Issue 4-2-2: In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case

Agreement:

- Absolute limit for in-channel adjacent subband leakage for DU/UD case

  - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands:

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

  - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

Issue 4-3-1: Interfering signal level for FR1

Agreement:

- On the level of in-channel adjacent subband blocking requirement:

  - for FR1 WA BS:

    - Adopt -28 dBm.

  - For FR1 MR BS:

    - -32 dBm.

  - For FR1 LA BS:

    - -34 dBm.

Issue 4-3-2: Interfering signal level for FR2-1

Agreement:

- On the level of in-channel adjacent subband blocking requirement,

  - For FR2-1 WA BS:

    - the way to define the requirement is similar to FR1, i.e., by considering coupling loss between co-site sectors:

      - Absolute value is -52dBm

Issue 4-3-3: Assumption for coupling loss

Agreement:

- Adopt the following assumption in the specification, and the detailed wording can be further refined:

  - For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

  - For FR2-1 WA BS, in-channel adjacent subband blocking requirement assumes 87 dBc coupling loss between interfering BS transmitter and SBFD BS receiver.

Issue 4-3-5: Minimum/maximum offset

Agreement:

- Adopt the following table as the requirement for both single carrier and carrier aggregation operations.

Table: In-channel adjacent subband blocking requirement for SBFD-capable BS

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (NOTE 1) | Interfering signal mean power (dBm) (NOTE 5) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) （NOTE 2, 3, 4） |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | CP-OFDM NR signal 30 kHz SCS, BW = X (in the unit of RB) |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | CP-OFDM NR signal 30 kHz SCS, BW1 = X1 and BW2 = X2 (in the unit of RB) |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. NOTE 2: For DU or UD case, interfering signal is placed at FC,SBFD,DL. For DUD case, two interfering signals are placed at FC,SBFD,DL,1 and FC,SBFD,DL,2. NOTE 3: For DU or UD case, X is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL. For DUD case, X1 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,1 and X2 is the largest transmission bandwidth configuration as specified in table 5.3.2-1 that is not larger than NRB, SBFD, DL,2. NOTE 4: In case the SBFD BS is configured with intra-band carrier aggregation, the set of interfering signals consists of the interfering signal(s) in this table and the signals from other intra-band contiguous carrier(s) operated in non-SBFD operation. The power spectral density of each interfering signal is set to be the same level. NOTE 5: For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dBc coupling loss between interfering BS transmitter and SBFD BS receiver. |  |  |  |  |  |

Topic #5: BS RF conformance requirements for SBFD

Issue 5-1-1: System parameters of conformance testing

Agreement:

- For system parameters of conformance testing:

  - The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

  - The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2.

Issue 5-1-2: BS type for SBFD conformance testing

Agreement:

- For BS type for SBFD conformance testing:

  - The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1.

Issue 5-1-3: Declaration of supported SBFD configuration

Agreement:

- Declaration of supported SBFD configuration:

  - At least the following declarations shall be captured in TS 38.141-1/-2:

    - TX power for SBFD;

    - Bands supported for SBFD;

    - Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104;

    - SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

    - Other declarations are not precluded

Issue 5-1-4: Testing applicability for supported SBFD configuration

Way forward:

- Further study on the following proposals on test configuration:

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD

  - Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

Issue 5-1-5: RF channels to be tested for SBFD operation

Way forward:

- Further study on the following proposals on RF channels to be tested for SBFD operation:

  - When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

Issue 5-2-1: Conformance testing related to transient period between SBFD and non-SBFD

Way forward:

- Further study on the following proposals on conformance test for transient period between SBFD and non-SBFD:

  - For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

Issue 5-2-2: Conformance testing related to in-channel adjacent subband leakage

Way forward:

- Further study on the following proposals on conformance test for in-channel adjacent subband leakage:

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

  - For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

Issue 5-2-3: Conformance testing related to in-channel adjacent subband blocking

Way forward:

- Further study on the following proposals on conformance test for in-channel adjacent subband blocking:

  - For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

  - For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

  - For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

Agreement and WF captured in R4-2512680

Issue 1: Align the terminology for “SBFD slot”, “SBFD symbol”, “SBFD symbol/slot” or “SBFD period” to be used in RAN4 specification

Agreement

- For the core spec, using SBFD symbol and a definition will be clarified in clause 3

- For the conformance testing, test should be based on slot level

Issue 2: In-band blocking value for FR1 WA BS

Agreement

- Adopt -25 dBm as the requirement

Issue 3: In-band blocking value for FR2-1 WA BS

Agreement

- Adopt -44 dBm as a provisional requirement. The value and the methodology to derive it shall be revisited in maintenance.

Issue 4: In-band blocking value for FR1 Micro BS

Agreement

- Adopt -32 dBm as the requirement.

Issue 5: In-band blocking requirement assumption

Agreement

- Capture the following as informative NOTE, with the understanding that they are not part of RF requirements.

  - NOTE1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements

  - NOTE2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor

Issue 6: Interference signal definition

Agreement

Table 2.2.1-1: FR1 SBFD-capable BS in-band blocking requirements

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wide Area BS: -25 Medium Range BS: -32 Local Area BS: -35 | ±30 | 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs |

Issue 7: ACS requirement

Agreement

- For WA SBFD BS type, adopt -28 dBm as the requirement.

- For MR and LA SBFD BS types, adopt the following values as requirement respectively.

  - MR: -32 dBm

  - LA: -35 dBm

Issue 8: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal

Agreement

- The RX linearity requirement is covered given following justifications

  - New IMD requirement is not needed, because the ACS requirement captures linearity in the same way as RX IMD requirement, i.e. the IMD between the ACS interfering signal and SBFD-capable BS’s own DL subband signal.

    - The size of SBFD UL subband shall be no less than 10 MHz. Note that this will be polished and captured in Clause 5.

Issue 9: In-channel selectivity

Agreement

- Existing ICS requirements can be reused with some updates.

  - The NOTE in ICS requirement should be updated to place the wanted signal and interference signal around the middle of uplink sub-band instead of Fc of the carrier bandwidth.

Issue 10: BS conformance testing - manufacturer declaration

Way forward

- FFS in next meeting

  - All the declarations in the WF is needed

  - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Tx requirements shall be updated to include the relevant clauses for SBFD capable BS.

  - It is necessary to add the definitions of output power requirements for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

  - It is necessary to add the definition of total power dynamic range requirement for SBFD capable BS to TS 38.141-1 and TS 38.141-2.

  - The definitions of EVM and averaged EVM for SBFD capable BS need to be added to TS 38.141-1 and TS 38.141-2.

  - The OTA measurement chamber setup, calibration, and measurement procedures defined for NR BS can serve as the baseline for testing Tx requirements of SBFD capable BS.

  - In both TS 38.141-1 and TS 38.141-2, references to TS 38.104 Rx requirements shall be updated to include the relevant clauses for SBFD capable BS.

  - The OTA measurement chamber setup and calibration (and measurement) procedures of Rx requirements testing for SBFD capable BS are different from those of NR systems, and these differences must be added to TS 38.141-2.

Issue 11: BS conformance testing - RF channels

Way forward

- FFS in next meeting

  - New test models need to be set for SBFD BS.

  - Guarantee SBFD carrier is configured at the left-most, right-most and middle of the operating band

Issue 12: BS conformance testing - Test model

Way forward

- FFS in next meeting

  - New parameters such as guard-band size should be described clearly in the spec.

  - The test model for SBFD DL subbband needs to be scalable.

  - New parameters, e.g., size of UL subband, UL subband center frequency or its relative offset from the DL, etc., are needed to describe the size and location of UL subband.

  - Signal statistics may change for SBFD test models and need to be investigated. The impact of DL subband on signal statistics needs to be analyzed.

  - In TS 38.141-1 and TS 38.141-2, the test models specified for NR BS can be reused for the existing Tx requirements testing of SBFD capable BS, but with the different allocated channel in tests.

  - A new test model which transmitter and the receiver operate simultaneously must be set up for Rx requirement testing of SBFD capable BS.

  - Test requirements for each Rx requirement of NR BS in TS 38.141-1 and TS 38.141-2 can serve as the baseline for SBFD capable BS.

  - In test requirements, NR channel bandwidth needs to be updated to the UL subband bandwidth of the SBFD capable BS.

  - Any test requirements for SBFD capable BS that differ from NR BS test requirements must be specified for SBFD capable BS in TS 38.141-1 and TS 38.141-2.

Issue 13: BS conformance testing - Test signal

Way forward

- FFS in next meeting

  - define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: BS vendor can decide to test with 50 MHz BS channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz BS channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Issue 14: BS conformance testing - Beam configuration

Way forward

- FFS in next meeting

  - When test Tx requirements of SBFD Capable BS, receivers can be turned off.

  - In the test of Tx requirements of SBFD Capable BS, the Tx antennas beam is test output beam.

  - When test OTA Rx requirements of SBFD capable BS, the transmitters must be turned on.

  - For OTA Rx requirements testing of SBFD capable BS, the beam configurations of Tx and Rx antennas need to consider the isolation requirement between them.

  - For OTA Rx requirements testing of SBFD capable BS, it is necessary to consider the directional correspondence between the test input antenna beam and the Rx antenna beam.

Agreement and WF captured in R4-2512133

Topic #1: UE-to-UE CLI handling

Issue 1-1-1: Requirement applicability for basic UE operation

Agreement

- For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Issue 1-1-2: L1 CLI measurement in non-SBFD symbols

Agreement (Monday online session)

- Define requirements for L1 UE-to-UE CLI measurements based on valid symbol type.

Agreement (Thursday online session)

- Re-use NR-U approach for defining measurement period extension due to invalid symbol type.

Issue 1-2-1: Collision between Method #1 and Method #2

Agreement (Tuesday AH session)

- UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

- Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

Issue 1-3-1: Measurement period

Agreement (Tuesday AH session)

- Based on 1 sample if timeRestrictionForChannelMeasurement is configured;

- Based on 3 samples if timeRestrictionForChannelMeasurement is not configured.

Issue 1-3-2: Measurement restriction

Agreement (Thursday online session)

- Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1

Topic #2: SBFD operation impacts

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

Agreements (Monday online session)

- For UE supporting simultaneous DL reception in 2 DL subbands,

  - For measurement period:

    - For UE indicating CSI processing time scaled by 2,

      - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2.

    - For UE not indicating CSI processing time scaled by 2,

      - Lower bound is not needed and existing measurement period applies

  - Further discuss the measurement accuracy requirements in Perf part of the WI

    - For UE indicating CSI processing time scaled by 2

      - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply.

      - Option 2: accuracy requirements based on 24 RB for Case 3 apply.

    - For UE not indicating CSI processing time scaled by 2

      - Option 1: accuracy requirements based on 24 RB for Case 3 apply.

- For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Issue 2-1-1d: Lmax in NR-U approach

Agreements (Thursday online session)

- Not define Lmax values, and not define the applicability condition based on upper bound for measurement period.

Issue 2-1-2: Requirements for RACH

Agreements

- Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase.

Issue 2-1-3: SCell activation

Agreements (Monday online session)

- The existing SCell activation delay requirement apply when activating a SBFD SCell. Further discuss whether any clarification is needed in the CR.

- Note: the agreement can be revised if not aligned with RAN1 existing and future agreements.

Issue 2-1-4: Condition for SSB based measurement with basic UE operation

Agreements

- For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Agreement and WF captured in R4-2512615

Topic 1 UE demoduation requirement

Issue 1-1-1 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured)

Agreement

- Not to introduce UE demodulation requirements with SBFD carrier (with DL sub-band and UL sub-band)

Topic 2 BS demoduation requirement

Issue 2-1-1 Types of interference modeling for demodulation requirement with UL muting

Agreement

- Evaluate PUSCH performance with UL muting under inter-site gNB interference modelling

  - FFS on self-interference and intra-site inter sector interference

Issue 2-1-2 how to model interference modeling for performance evaluation

Agreement

- Considering TDL+INR type of interference model for evaluation

  - Note: the RF assumption can be considered for interference modelling

- The following candidate value for INR can be considered for evaluation as starting point

  - Option 1: based on link budget analysis

| Conditions | FR1 |
| --- | --- |
| Interf1 > Interf2 | INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB |
| Interf1 = Interf2 | INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB |
| Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup. |  |

  - Other options are not concluded

- Interesting companies can provide the system level simulation to derive the INR value

Issue 2-1-3 Evaluation Metrics to choose interference model and make decision on requirement introduction

Agreement

- RAN4 can use the following metrics to choose interference model based on evaluation result as starting point

  - The peak throughput can be achieved

  - The obvious performance gain can be shown by comparing using muting Res for estimation to not using muted REs for estimation

  - The target SNR (e.g., SNR at70% Tput) should be feasible for both network deployment and tests

- Based on the evaluation, RAN4 can further discuss whether to introduce the PUSCH requirement with UL muting

Issue 2-1-4 Baseline Receiver Assumption

Agreement

- MMSE-IRC with the same assumption in Rel-19 performance enhanced WI

Issue 2-2-1 Frequency Range

Agreement

- For performance evaluation, prioritized FR-1

- Cover both FR1 and FR2-1 if agreed to introduce PUSCH demodulation requirement with UL muting

Issue 2-2-2 Waveform for wanted signal

Agreement

- For performance evaluation, prioritized CP-OFDM waveform

- Cover both CP-OFDM and DFT-s-OFDM waveforms if agreed to introduce PUSCH demodulation requirement with UL muting

Issue 2-2-3 sub-band pattern

Agreement

- For performance evaluation, considering sub-band configuration as {DU}

- Note: the UL sub-band is located in the lower band

Issue 2-2-4 Channel Bandwidth for wanted signal

Agreement

- For performance evaluation

  - For FR1: 40MHz CBW (20MHz DL sub-band+ 20MHz UL sub-band) for 30kHz SCS

  - For FR2-1: 200MHz CBW (100MHz DL sub-band+ 100MHz UL sub-band) for 120kHz SCS

Issue 2-2-5 TDD pattern

Agreement

- For performance evaluation

  - 30 kHz SCS:  XXXXXXXXUU

  - 120 kHz SCS: XXXXU

  - UL sub-band transmission could start from the first F slots

  - Skip U slots to simply simulation setup

Issue 2-2-6 Sub-band configuration in SBFD slot

Agreement

- For performance evaluation

  - Take SBFD UL sub-band spans all the symbols with a SBFD slot

Issue 2-2-7 Number of symbol muting

Agreement

- For performance evaluation

  - Consider both 1 and 2 symbols, 1 symbol is prioritized

  - Companies are also encouraged to evaluate the number of muting symbols as 2

Issue 2-2-8  Muting symbol configurations

Agreement

- For performance evaluation

  - Option 1

  - Option 2

  - Companies are encouraged to provide the evaluation results based on following muting symbol patterns in RAN4#116bis meeting

Issue 2-2-9 PUSCH mapping type for wanted signal

Agreement

- For evaluation

  - mapping type A

Issue 2-2-10 Number of DMRS symbols for wanted signal

Agreement

- For evaluation

  - Configure 2 DMRS symbols for defining SBFD PUSCH demodulation requirement in FR1 and FR2-1

Issue 2-2-11 Channel model

Agreement

- For evaluation

  - TDLC300-100 for wanted signal in FR1

  - TDLC300-0 for interference signal if considering INR based interference modelling in FR1

Issue 2-2-12 MCS

Agreement

- For evaluation

  - MCS16 for wanted signal in FR1

  - [16QAM for interference signal in FR1],

  - Rank 1 for wanted signal

  - Rank 1 for interference signal as starting point to simply the test setup

Issue 2-2-13 Antenna Configuration

Agreement

- For evaluation

  - Reuse the same antenna configuration assumption for evaluation SBFD PUSCH performance with UL muting as 1Tx/2Rx/4Rx/8Rx in FR1 and 1Tx/2Rx in FR2-1

  - 1Tx for gNB DL transmission as starting point to simply simulation setup

Issue 2-2-14 Other Assumption

Agreement

- For performance evaluation

  - Reuse the same assumption for specifying SBFD PUSCH demodulation requirement in FR1 and FR2-1 as referred in Table: 8.2.1.1-1/ Table: 8.2.2.1-1 and Table 11.2.2.1.1-1/ Table 11.2.2.2.1-1, respectively.

Issue 2-3-1 Specification Impact

Way forward

- Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured.

- Other options are not precluded

#### 2.4.2	Remaining Open issues

Remaining open issues for core part:

- None.

Remaining open issues for performance part:

- BS RF conformance requirements to support SBFD operation at gNB

- RRM performance requirements to support SBFD operation

- BS and/or UE demodulation performance requirements to support SBFD operation

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#131

R2-2505015	Reply LS on simultaneous configuration of SBFD and DC (R1-2504858; contact: Xiaomi)	RAN1

R2-2505030	LS on TP for TS38.300 on Rel-19 SBFD (R1-2505081; contact: Huawei)	RAN1

R2-2505088	Introduction of SBFD in TS 38300	CATT

R2-2505089	Leftover Issues on Random Access in SBFD	CATT

R2-2505090	Leftover Issues on other aspects in SBFD	CATT

R2-2505126	Remaining issues of RA for SBFD	NEC

R2-2505141	Discussion on RACH in SBFD	Xiaomi

R2-2505142	Other aspects of SBFD	Xiaomi

R2-2505243	Power ramping issue on the RO type fallback	OPPO, ZTE Corporation, Sharp, NEC, NTT DOCOMO INC., Qualcomm Incorporated, CATT

R2-2505244	Clarification on the CFRA for SBFD RO	OPPO

R2-2505363	Introduction of Rel-19 Evolution of NR duplex operation (SBFD)	Huawei, HiSilicon

R2-2505364	Summary of [Post130][216][SBFD] Running CR for 38.331	Huawei, HiSilicon

R2-2505365	Discussion on MAC open issues for random access in SBFD	Huawei, HiSilicon

R2-2505366	Discussion on other aspects of SBFD	Huawei, HiSilicon

R2-2505459	Remaining issues on Random Access procedure for SBFD	LG Electronics Inc.

R2-2505495	Open issues for RACH in SBFD	Apple

R2-2505549	Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec	Samsung

R2-2505559	Discussions on the open issues for Random Access	Samsung

R2-2505560	Summary of the SBFD open issues in MAC	Samsung

R2-2505575	Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec	Samsung

R2-2505590	Discussion on random access procedure in SBFD	ZTE Corporation

R2-2505591	Discussion on the co-existence of SBFD and LTM	ZTE Corporation, OPPO, Interdigital, LG, Apple, Charter, Nokia

R2-2505592	Discussion on multi-carrier and measurements in SBFD	ZTE Corporation

R2-2505661	Remaining issues for Random Access in SBFD Operation	Sony

R2-2505666	SBFD RA remaining aspects	Ericsson

R2-2505667	DC and CSI-RS measurements in SBFD	Ericsson

R2-2505751	Random Access Operation of SBFD	Nokia

R2-2505820	Views on random access for SBFD	Qualcomm Incorporated

R2-2505821	Other aspects of SBFD	Qualcomm Incorporated

R2-2505904	Remaining issues on RACH aspect in SBFD	InterDigital, Inc.

R2-2505905	Discussion on other aspect in SBFD	InterDigital, Inc.

R2-2505929	Discussion on SBFD RA open issues	Sharp

R2-2505930	Discussion on SBFD other open issue	Sharp

R2-2505952	Discussion on random access in SBFD	CMCC

R2-2505982	Remaining issues on RACH procedure for SBFD	vivo

R2-2505983	SBFD other aspects	vivo

R2-2506131	Other Aspects of SBFD	Nokia

R2-2506166	Other Aspects of SBFD	Nokia

RAN3#129

R3-255008	Reply LS on simultaneous configuration of SBFD and DC	RAN1(Xiaomi)

R3-255010	LS on TP for TS38.300 on Rel-19 SBFD	RAN1(Huawei)

R3-255089	(BL CR to 38.300) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Ericsson, Nokia

R3-255090	(BL CR to 38.401) introduction of Evolution of NR duplex operation	ZTE Corporation, Ericsson, Samsung, Qualcomm, Huawei, CATT,  Nokia, CMCC, LGE

R3-255091	(BL CR to 38.420) introduction of Evolution of NR duplex operation	CMCC, Ericsson, Huawei, Samsung, ZTE, Nokia, Jio Platforms

R3-255092	(BL CR to 38.423) introduction of Evolution of NR duplex operation	Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT

R3-255093	(BL CR to 38.470) introduction of Evolution of NR duplex operation	CATT, Ericsson, CMCC, Nokia, ZTE, Qualcomm, Samsung, Jio Platforms

R3-255094	(BL CR to 38.473) introduction of Evolution of NR duplex operation	Samsung, Huawei, Ericsson, ZTE, LG Electronics Inc., Nokia, Qualcomm, Jio Platforms

R3-255267	UE-to-UE CLI mitigation support	Charter Communications, Inc

R3-255276	[TP for BL CR TS 38.300] Introduction of SRS resource configurations for Evolution of NR duplex operation Sub-Band Full Duplex	Charter Communications, Inc

R3-255277	[TP for BL CR 38.423] Introduction of UE-to-UE CLI mitigation support signaling	Charter Communications, Inc

R3-255278	[TP for BL CR 38.473] Introduction of UE-to-UE CLI mitigation support signaling	Charter Communications, Inc

R3-255324	Signaling design to enable NR SBFD operation	Qualcomm Incorporated

R3-255438	(TP for BLCRs to TS 38.420, 38.470) Finalizing stage 2 for Sub-band Full Duplex operation	Nokia

R3-255439	(TP for BLCRs to TS 38.423, 38.473) Finalizing stage 3 for Sub-band Full Duplex operation	Nokia

R3-255480	(TP to BLCR to TS38.423 for SBFD) Discussion on SBFD WI completion	Ericsson

R3-255481	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD	Ericsson

R3-255645	Discussion on Evolution of Duplex Operation	CATT

R3-255646	(TP for 38.423 and 38.473) Support on Evolution of Duplex Operation	CATT

R3-255667	Further discussion on information exchange for CLI handling	Samsung

R3-255668	Text proposals to BLCRs of TS38.473 on CLI handling	Samsung

R3-255679	Further discussion on remaining open issues	Huawei, China Telecom, China Unicom

R3-255680	Discussion on the exchange of SRS resource configurations	Huawei, China Telecom, China Unicom

R3-255681	(TP to BL CR for 38.423) miscellaneous clean-ups for Evolution of NR duplex operation Sub-band full duplex (SBFD)	Huawei, China Telecom, China Unicom

R3-255682	(TP to BL CR for 38.423) introduction of SRS resource configurations for Evolution of NR duplex operation Sub-band full duplex (SBFD)	Huawei

R3-255683	(TP to BL CR for 38.473) introduction of  SRS resource configurations forEvolution of NR duplex operation Sub-band full duplex (SBFD)	Huawei

R3-255684	Updated workplan for Evolution of NR Duplex Operation	Huawei, Sumsang

R3-255691	(TP to BLCR/draft CR for 38.300, 38.401, 38.423 and 38.473) SBFD	ZTE Corporation

R3-255692	Discussion on information exchange for SBFD	ZTE Corporation

R3-255703	(TPs to BLCR for TS 38.423 and TS 38.473) Discussion on supporting CLI handling in SBFD	CMCC

R3-255845	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD	Ericsson

R3-255846	(TP to BL CR for 38.423) miscellaneous clean-ups for Evolution of NR duplex operation Sub-band full duplex (SBFD)	Huawei, China Telecom, China Unicom

R3-255847	(TP to BLCRs of TS38.473) on CLI handling	Samsung

R3-255848	(TP to TS 38.401 BL CR) Introduction of SBFD	ZTE Corporation

R3-255849	(TP to TS 38.420 BL CR) Introduction of SBFD	CMCC

R3-255850	(TP to TS 38.470 BL CR) Introduction of SBFD	CATT

R3-255851	SoD for CB: # 9_DuplexOperation	Huawei

R3-255854	Summary of Discussion in CB: # 10_ DuplexUEtoUECLI	Samsung

R3-255855	(TP to BLCR to TS38.423 for SBFD) exchange of SRS resource configurations	ZTE

R3-255856	(TP to BL CR for 38.473) exchange of SRS resource configuartions	CATT

R3-255920	(TP to BLCR to TS38.300 for SBFD) Introduction of SBFD	Ericsson, Huawei, ZTE, Samsung, Nokia

R3-255921	(TP to BL CR for TS 38.423) Miscellaneous clean-ups for Evolution of SBFD	Huawei, China Telecom, China Unicom, Samsung, Ericsson, LG Electronics, CMCC, CATT, Nokia, ZTE

R3-255922	(TP to BL CR for 38.473) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)	Samsung, Ericsson, LG Electronics, ZTE, Huawei, Nokia, CATT

R3-255923	(TP to TS 38.470 BL CR) Introduction of SBFD	CATT, Ericsson, ZTE, Huawei, China Telecom, Nokia

R3-255924	(TP to BL CR for 38.420) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)	CMCC, Ericsson, Samsung, ZTE, Huawei, Nokia

R3-255925	(TP to TS 38.401 BL CR) Introduction of SBFD	ZTE Corporation

R3-255926	(TP to BLCR 38.423) SBFD on UE to UE CLI	ZTE Corporation, Ericsson, China Telecom, Huawei, Charter Communications, Samsung, CATT, Nokia, Qualcomm, Verizon

R3-255927	(TP to BL CR for 38.473) Exchange of SRS resource configurations	CATT, China Telecom, Huawei, Samsung, Ericsson, Nokia, ZTE, Charter Communications, Qualcomm, Verizon

R3-255956	(TP to TS 38.401 BL CR) Introduction of SBFD	ZTE Corporation, Ericsson, China Telecom, Huawei, Nokia, Qualcomm, CMCC, Samsung, LG Electronics

R3-255986	(BL CR to 38.300) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Ericsson, Nokia

R3-255987	(BL CR to 38.401) introduction of Evolution of NR duplex operation	ZTE Corporation, Ericsson, Samsung, Qualcomm, Huawei, CATT,  Nokia, CMCC, LGE

R3-255988	(BL CR to 38.420) introduction of Evolution of NR duplex operation	CMCC, Ericsson, Huawei, Samsung, ZTE, Nokia, Jio Platforms

R3-255989	(BL CR to 38.423) introduction of Evolution of NR duplex operation	Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT

R3-255990	(BL CR to 38.470) introduction of Evolution of NR duplex operation	CATT, Ericsson, CMCC, Nokia, ZTE, Qualcomm, Samsung, Jio Platforms

R3-255991	(BL CR to 38.473) introduction of Evolution of NR duplex operation	Samsung, Huawei, Ericsson, ZTE, LG Electronics Inc., Nokia, Qualcomm, Jio Platforms

RAN4#116

R4-2509061	Topic summary for [116][218] NR_duplex_evo	Moderator (Huawei)

R4-2509120	SBFD/TDD Adjacent Channel UE-UE CLI simulation	Charter Communications, Inc

R4-2509239	Draft CR on general L1-CLI-RSSI measurements	Samsung

R4-2509305	DraftCR to update L1-CLI-RSSI measurement requirements	CATT

R4-2509306	Discussion on UE to UE CLI handling	CATT

R4-2509307	Discussion on impacts of SBFD on RRM requirements	CATT

R4-2509308	Discussion on SBFD RRM performance requirements	CATT

R4-2509322	Discussion on SBFD general issues for SBFD operation including BS conformance testing	CATT

R4-2509323	Discussion on potentially new requirements for SBFD operation including BS conformance testing	CATT

R4-2509324	Discussion on modification of existing Tx requirements for SBFD operation including BS conformance testing	CATT

R4-2509325	Discussion on modification of existing Rx requirements for SBFD operation including BS conformance testing	CATT

R4-2509326	draftCR for TS 38 104 to introduce conducted and OTA out-of-band blocking for SBFD	CATT

R4-2509385	Further discussion on updating duty cycle evaluation with the introduction of SBFD	CATT

R4-2509386	draftCR to TS 38.101-1 on updating duty cycle evaluation with the introduction of SBFD	CATT

R4-2509401	View on demodulation requirement for SBFD	Samsung

R4-2509439	On UE RRM requirements for UE-to-UE CLI handling	Apple

R4-2509616	RRM requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2509617	draftCR on core requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2509618	RRM requirements for SBFD operation	Nokia, Nokia Shanghai Bell

R4-2509619	draftCR on SCell activation for SBFD SCell	Nokia, Nokia Shanghai Bell

R4-2509620	RRM performance requirements for SBFD operation	Nokia, Nokia Shanghai Bell

R4-2509621	draftCR on L1 CLI performance requirements	Nokia, Nokia Shanghai Bell

R4-2509622	Simulation result for CSI-RS based measurement for SBFD	Nokia, Nokia Shanghai Bell

R4-2509639	Views on modification of existing Rx requirements for SBFD	Tejas Network Limited

R4-2509659	Discussion on SBFD UE demodulation and CSI requirements	MediaTek inc.

R4-2509691	DraftCR on requirements of CSI-RS based L1-RSRP Reporting with SBFD	OPPO

R4-2509692	On the accuracy requirements of CSI-RS based L1 measurement for SBFD operation	OPPO

R4-2509735	Discussion on RRM impacts of SBFD operation	LG Electronics Inc.

R4-2509736	Discussion on RRM performance requirements of Rel-19 SBFD	LG Electronics Inc.

R4-2509836	Views on L1 CLI measurement requirements	Samsung

R4-2509837	Views on RRM requirements for Rel-19 SBFD operation	Samsung

R4-2509838	Discussion on RRM performance requirements of Rel-19 SBFD	Samsung

R4-2509991	Draft CR on clarification of DL slots and UL slots in SBFD	Apple

R4-2510085	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation	China Telecom

R4-2510086	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation	China Telecom

R4-2510087	Draft CR on CSI-RS based CBD evaluation period requirements with SBFD	China Telecom

R4-2510088	Discussion on RRM performance requirements for evolution of NR duplex operation	China Telecom

R4-2510130	Discussion on BS conformance for SBFD	CMCC

R4-2510131	Discussion on existing Rx requirements for SBFD	CMCC

R4-2510132	Discussion on SBFD general part	CMCC

R4-2510133	Draft CR on TS 38.104 to add unwanted emissions for SBFD (NR_duplex_evo-Core)	CMCC

R4-2510151	Discussion on RRM impacts for SBFD operation	CMCC

R4-2510152	Discussion on RRM requirements for UE-to-UE CLI handling	CMCC

R4-2510153	Discussion on RRM performance requirements for SBFD	CMCC

R4-2510174	DraftCR on measurement period for L1-SRS-RSRP measurement	CMCC

R4-2510188	Discussion on remaining RRM requirements for UE-to-UE CLI handling	vivo

R4-2510189	Discussion on remaining RRM impacts for SBFD operation	vivo

R4-2510198	Draft CR on general for SBFD	vivo

R4-2510263	Discussion on general aspects for gNB SBFD	vivo

R4-2510417	Views on Inter-UE CLI aspects and potential solutions for SBFD deployments	Qualcomm Germany

R4-2510418	draftCR to 38.104 for introduction of receiver spurious and intermodulation requirements	Qualcomm Germany

R4-2510419	Views on UE demodulation requirements for SBFD	Qualcomm Germany

R4-2510420	views on general aspects for SBFD	Qualcomm Germany

R4-2510651	Discussion on UE CLI measurement for SBFD	Huawei, HiSilicon

R4-2510652	Discussion on RRM impacts for SBFD operation	Huawei, HiSilicon

R4-2510653	draftCR on L1-SINR measurement requirements with SBFD	Huawei, HiSilicon

R4-2510654	Discussion on performance requirements for SBFD	Huawei, HiSilicon

R4-2510655	Simulation results for L1-RSRP and L1-SINR accuracy	Huawei, HiSilicon

R4-2510684	Discussion on UE-to-UE CLI of R19 SBFD	ZTE Corporation, Sanechips

R4-2510685	Discussion on other RRM impacts of R19 SBFD	ZTE Corporation, Sanechips

R4-2510689	Discussion on Perf part of R19 SBFD	ZTE Corporation, Sanechips

R4-2510715	Discussion on SBFD demodulation requirement	Ericsson

R4-2510723	Draft CR on scheduling applicability of R19 SBFD	ZTE Corporation, Sanechips

R4-2510791	RRM requirement for UE-to-UE CLI handling	Ericsson

R4-2510792	RRM requirement for SBFD operation	Ericsson

R4-2510793	RRM performance requirement for Rel-19 SBFD	Ericsson

R4-2510794	Draft CR to 38.133 on SBFD UE-to-UE CLI	Ericsson

R4-2510795	Draft CR to 38.133 on SBFD CSI-RS based L1-RSRP	Ericsson

R4-2510796	Draft CR to 38.133 on SBFD CSI-RS based CBD	Ericsson

R4-2510797	Draft CR to 38.133 on SBFD CSI-RS based RLM	Ericsson

R4-2510798	Draft CR to 38.133 on SBFD CSI-RS based BFD	Ericsson

R4-2510799	Draft CR to 38.133 on SBFD CSI-RS based L1-SINR CSI-RS based CMR and IMR	Ericsson

R4-2510800	Draft CR to 38.133 on SBFD CSI-RS based L1-SINR CSI-RS based CMR and no dedicated IMR	Ericsson

R4-2510801	Draft CR to 38.133 on SBFD CSI-RS based L1-SINR SSB based CMR andCSI-RS based IMR	Ericsson

R4-2510868	Discussion on SBFD demodulation performance requirements	Huawei,HiSilicon

R4-2510961	SBFD general aspects	Ericsson

R4-2510962	New SBFD BS RF requirements	Ericsson

R4-2510963	Impact on SBFD BS RF TX requirements	Ericsson

R4-2510964	Impact on SBFD BS RF RX requirements	Ericsson

R4-2510965	Draft CR to 38.104 on adding receiver in-channel requirements for SBFD	Ericsson

R4-2510966	Draft CR to 38.104 on adding OTA spatial emission requirements for SBFD	Ericsson

R4-2510984	Discussion on UE and BS demodulation performance requirements for SBFD	ZTE Corporation, Sanechips

R4-2510995	On the Evolution of NR Duplex Operation - Sub-Band Full Duplex	Nokia

R4-2511105	On general aspects for SBFD	Huawei, HiSilicon

R4-2511106	On intra-operator adjacent channel inter-cell UE-UE CLI	Huawei, HiSilicon

R4-2511107	On potentially new requirements for SBFD including BS conformance	Huawei, HiSilicon

R4-2511109	Draft CR for L1-CLI-RSSI measurement period	LG Electronics Inc.

R4-2511137	Discussion on system parameters for SBFD BS	ZTE Corporation, Sanechips

R4-2511138	Discussion on UE2UE CLI analysis	ZTE Corporation, Sanechips

R4-2511139	Discussion on potentially new requirements for SBFD operation	ZTE Corporation, Sanechips

R4-2511140	Draft CR to TS 38.104 on In-channel adjacent subband leakage for SBFD	ZTE Corporation, Sanechips

R4-2511141	Discussion on conformance testing for SBFD BS	ZTE Corporation, Sanechips

R4-2511142	Draft CR to TS 38.104 on Annex EVM measurement procedure	ZTE Corporation, Sanechips

R4-2511143	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2511212	Discussion on SBFD UE-to-UE CLI	MediaTek inc.

R4-2511213	Draft CR on clarification of L3 CSI-RS measurement requirement for SBFD	MediaTek inc.

R4-2511214	Discussion on SBFD Operation	MediaTek inc.

R4-2511215	Discussion on SBFD Performance	MediaTek inc.

R4-2511250	Remaining issues for SBFD general aspects	Samsung

R4-2511251	Analysis on UE-UE CLI problem and potential solutions	Samsung

R4-2511252	Remaining issues for new requirements for SBFD operation for FR1 and FR2-1	Samsung

R4-2511253	Remaining issues on modification of existing TX requirements for SBFD-capable BS	Samsung

R4-2511254	Remaining issues on modification of existing RX requirements for SBFD-capable BS	Samsung

R4-2511255	CR to TS38.104 to introduce BS RF requirement for SBFD-capable BS	Samsung

R4-2511256	draftCR to TS38.104 on transient period requirements for SBFD-capable BS	Samsung

R4-2511257	draftCR to TS38.104 on general clauses for SBFD-capable BS	Samsung

R4-2511285	On modification of existing TX RF requirements for SBFD	Huawei, HiSilicon

R4-2511286	On modification of existing RX RF requirements for SBFD	Huawei, HiSilicon

R4-2511287	Draft CR to 38.104 on Reference sensitivity and dynamic range for SBFD	Huawei, HiSilicon

R4-2511288	Draft CR to 38.104 on in-band selectivity and blocking for SBFD	Huawei, HiSilicon

R4-2511314	Simulation and discussion on SBFD UE-UE CLI	Ericsson

R4-2511452	Topic summary for [116][306] NR_duplex_evo_General	Moderator (Samsung)

R4-2511453	Topic summary for [116][307] NR_duplex_evo_BSRF	Moderator (Huawei)

R4-2511473	Topic summary for [116][328] NR_duplex_evo_demod	Moderator (Samsung)

R4-2511494	Discussion on SBFD symbol on UL duty cycle evaluation	Ericsson

R4-2511526	On SBFD system parameters	Nokia

R4-2511527	On potentially new BS RF requirements for SBFD operation	Nokia

R4-2511528	On existing BS RF TX requirements for SBFD operation	Nokia

R4-2511529	On existing BS RF RX requirements for SBFD operation	Nokia

R4-2511530	On UE-UE CLI problem statement and potential solutions	Nokia

R4-2511531	DraftCR to TS 38.104 - In-band blocking requirements for SBFD	Nokia

R4-2511538	Discussion on SBFD–SBFD Adjacent-Channel Coexistence Scenario	ISSDU

R4-2511637	draftCR on CSI-RS based RLM evaluation period requirements with SBFD	Qualcomm Incorporated

R4-2511638	RRM requirements for UE-to-UE CLI handling	Qualcomm Incorporated

R4-2511639	RRM requirements for SBFD operation	Qualcomm Incorporated

R4-2511640	Simulation results for UE CLI measurement	Qualcomm Incorporated

R4-2511641	RRM performance requirements for SBFD	Qualcomm Incorporated

R4-2512132	Ad-hoc minutes for NR_duplex_evo	Samsung

R4-2512133	WF on RRM requirements for NR_duplex_evo	Huawei

R4-2512190	Updated summary of simulation results for L1-SRS-RSRP	Huawei, HiSilicon

R4-2512191	Draft CR for L1-CLI-RSSI measurement period	LG Electronics Inc.

R4-2512193	draftCR on core requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2512198	draftCR on L1-SINR measurement requirements with SBFD	Huawei, HiSilicon

R4-2512199	DraftCR on requirements of CSI-RS based L1-RSRP Reporting with SBFD	OPPO

R4-2512326	draftCR on core requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2512352	Big CR for RRM requirements of Sub-band full duplex	Huawei, Samsung

R4-2512528	draftCR to TS38.104 on general clauses for SBFD-capable BS	Samsung, Ericsson

R4-2512529	Way Forward for UE-UE CLI	Charter

R4-2512530	Way Forward for [116][306] NR_duplex_evo_General	Samsung

R4-2512531	Way Forward for [116][307] NR_duplex_evo_BSRF	Huawei

R4-2512608	Offline meeting minutes for [116][328] NR_duplex_evo_demod	Samsung

R4-2512615	Way Forward for [116][328] NR_duplex_evo_demod	Samsung

R4-2512665	Way Forward for [116][306] NR_duplex_evo_General	Samsung, CATT, Charter, CMCC, Ericsson, Huawei, HiSilicon, Nokia, Qualcomm, Tejas Network, ZTE

R4-2512673	Draft CR to TS 38.104 on In-channel adjacent subband leakage for SBFD	ZTE Corporation, Sanechips

R4-2512674	Draft CR on TS 38.104 to add unwanted emissions for SBFD (NR_duplex_evo-Core)	CMCC

R4-2512675	Draft CR to TS 38.104 on Annex EVM measurement procedure	ZTE Corporation, Sanechips

R4-2512676	Draft CR to 38.104 on Reference sensitivity and dynamic range for SBFD	Huawei, HiSilicon

R4-2512677	Draft CR to 38.104 on in-band selectivity and blocking for SBFD	Huawei, HiSilicon

R4-2512678	draftCR to 38.104 for introduction of receiver spurious and intermodulation requirements	Qualcomm Germany

R4-2512679	Draft CR to 38.104 on adding receiver in-channel requirements for SBFD	Ericsson

R4-2512680	Way Forward for [116][307] NR_duplex_evo_BSRF	Huawei

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template