3GPP TSG-RAN Meeting #109                                       	                                    RP-252500

# Beijing, China, September 15 – September 18, 2025

Agenda item:	9.6.4.1

Source:	Apple, Google, Huawei, HiSilicon

Title:	Discussion on Release Independence

Release:	Rel-19

Document for:	Discussion

# Introduction

During RAN4 WG meetings, more specifically, RAN4#115 and RAN4#116, the possibility of release independence for 6Rx UEs has been discussed at length. A clear consensus has not been reached. Here we provide details and context of these two previous discussions.

During RAN4#115, the following outcome was reached:

Agreement:

- Option 1:

  - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18

  - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19

- Option 2:

  - Support 6Rx from Rel-19

- Option 3:

  - Support 6Rx from Rel-18

In addition, after extensive discussion among companies, by the end of RAN4#116 the following outcome was reached, including an LS to RAN2 (R4-2511898) to provide clarification and the guidance of RAN4 Chair:

Title: LS on Release Independence of 6Rx Source: RAN4 To: RAN2 Cc: RAN1 1. Overall Description: RAN4 is discussing release independence for 6Rx UE requirements with max 6 DL MIMO layers and max 4 DL MIMO layers. Since release independence aspects are related to the max 6 DL MIMO layers feature with new UE capability signalling, RAN4 respectfully requests RAN2 to provide feedback on whether UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH as in LS R4-2505225 will be specified in Rel-19. If Yes, can this signalling be implemented early in Rel-18? 2. Actions: To RAN2 ACTION: RAN4 respectfully requests RAN2 to provide feedback on whether UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH as in LS R4-2505225 will be specified in Rel-19. If Yes, can this signalling be implemented early in Rel-18?

# On Release Independence Principles

The 3GPP specification 38.307, Requirements on User Equipments (UEs) supporting a Release-Independent Frequency Band, is the spec for the release independence issues and maintained by RAN4. By its name, the release independence should be primarily used for frequency band and band combinations.

In this contribution we want to discuss and seek for RAN plenary guidance on the principle to specify a non-spectrum feature as release independent.

Observation #1: TS 38.307 is primarily used to define release-independent band and band combinations. RAN plenary guidance on the principle to specify a non-spectrum feature is needed.

In terms of spectrum related features, there is a plethora of use cases that comprise the large majority of TS 38.307. Among others, we can find:

- NR Operating Frequency Bands, which include power classes and RF requirements.

- Supported Channel Bandwidths, which are specified per duplex mode (FDD, TDD, SDL, SUL)

- Carrier Aggregation (CA) Configurations, which cover both intra-band and inter-band CA.

- Power Classes, which details requirements related to UE power levels for different bands and modes.

The rationale here for release independence of spectrum-related features is that spectrum is a very valuable and expensive resource that needs to be managed as efficiently as possible.

Observation #2: The large majority of the release independent features are spectrum-related, on topics like NR operating frequency bands, channel bandwidths, carrier aggregation configurations, and power classes, among others.

On the other hand, there are some non-spectrum related requirements, which are generic baseband requirements for already specified features in legacy releases where, based on RAN4 consensus, are specified as release independent:

- Rel-17 UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference from Rel-15

- Rel-18 PDSCH absolute physical layer throughput requirements with link adaptation from Rel-17

- Rel-18 Demodulation requirements for 8Rx UE from Rel-17

The main principle is that the newly defined requirements for legacy features can be release independence. In other words, previous non-spectrum related release independence cases are focused on the new requirements only without additional signalling or new features included.

Here, one of the fundamental principles of release independence in 3GPP is backwards compatibility. In general, we need to avoid any protocol or signalling changes that could disrupt functionality in our earlier releases. We believe that release independence should fundamentally be designed to permit features to be introduced only when their implementation can coexist harmoniously with existing versions.

This ensures that UEs conforming to prior releases can maintain operational integrity and interoperability, preventing signalling conflicts or any sort of compatibility breakdowns. We want to maintain that backward compatibility is an essential prerequisite that should govern whether a feature can qualify as release independent by safeguarding the stability and the consistency of the evolving 3GPP ecosystem, especially now that we are starting a new generation for this standard.

Observation #3: The newly defined requirements for legacy features can be release independence without new signaling introduced. Release independent feature/requirement should be backward compatible.

Proposal#1: RAN to confirm the release independence is primarily designed for spectrum related band and band combinations

Proposal #2: RAN to confirm that the release independence of non-spectrum related feature should only apply when signalling changes on previous releases are not needed for adopting such feature.

Proposal #3: RAN to provide guidance to RAN4 on the principles behind release independence decisions.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: TS 38.307 is primarily used to define release-independent band and band combinations. RAN plenary guidance on the principle to specify a non-spectrum feature is needed.

Observation #2: The large majority of the release independent features are spectrum-related, on topics like NR operating frequency bands, channel bandwidths, carrier aggregation configurations, and power classes, among others.

Observation #3: The newly defined requirements for legacy features can be release independence without new signaling introduced. Release independent feature/requirement should be backward compatible.

Proposal#1: RAN to confirm the release independence is primarily designed for spectrum related band and band combinations

Proposal #2: RAN to confirm that the release independence of non-spectrum related feature should only apply when signalling changes on previous releases are not needed for adopting such feature.

  - Proposal #3: RAN to provide guidance to RAN4 on the principles behind release independence decisions.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

# Appendix: Ongoing RAN2 Discussion on Signalling

It is also worth mentioning that the discussion above runs in parallel to another RAN2 discussion following previous RAN4 agreements. During RAN4#113 we had the following RAN4 way forward:

        - RAN4 unable to conclude on simulation assumptions to determine feasibility for handheld UEs

- 6-layer performance requirements will be only defined for FWA.

- 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

- The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

Immediately after that meeting, the corresponding modification to the WID (RP-250780) was made during RAN#107 as per the line highlighted in what follows:

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Introduce 6 MIMO layers support as an optional feature only for FWA.

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Currently the RAN2 discussion is not capturing correctly the previous RAN4 agreement. This is, 6 MIMO layers being an optional feature only for FWA cannot be derived simply from RAN4 specifications, i.e. 38.101-4. Performance requirements in 38.101-4 can only define applicability rules for testing requirements. If RAN2 does not capture that this feature is only for FWA UEs, that effectively means that the feature would become required for handheld UEs as well. This is in direct contradiction to previous RAN4 agreements.