3GPP TSG RAN Meeting #109	RP-252728

Beijing, China, 15th - 18th September, 2025

Agenda Item:		9.5.3

Source:			Nokia

Title:			SI Summary: Study on Spatial Channel Model for demodulation performance requirements for NR

WI code(s):			FS_NR_demod_SCM

leading WG:		RAN4

Release:			Rel-19

### 1	Introduction

Limitations of the existing method for channel modelling for RAN4 performance requirements using the spatially agnostic Tapped Delay Line (TDL) or spatially deficient TDL + antenna correlation approaches have been discussed in RAN1 and in RAN4. These limitations may be attributed to the fact that TDL channels on their own are spatially agnostic, i.e., any direction to transmit signals through the channel is as good as any other, which impacts performance of different MIMO layers matched to differing large scale environments. Such shortcomings are, for example, visible in 2-codeword requirements in Rel-18, where both codewords have the same performance requirements at the same time.

Channel models with spatial property have been discussed since the beginning of NR. In Rel-14, two study items were progressed: the first one analyzing how to define a channel model representing the channel for frequencies up to 100 GHz [2], whose outcome was documented in 3GPP TR 38.900 [3], and the second one aiming to develop an NR access technology to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC [4], including the channel model definition that was finally documented in TR 38.901 [5]. RAN1 are using geometric based SCM(E) defined in 38.901 for system level simulation since this channel model is more realistic. The 38.901 CDL channel model is derived from SCM(E) with randomness reduction on some parameters, and the 38.901 TDL model is further derived from 38.901 CDL without spatial property [4]. In Rel-15, RAN4 decided to use simplified TDL model [6] for NR FR1 demodulation tests in May 2018 [7] and simplified TDL model for NR FR2 demodulation tests in Aug 2018 [8] since a majority of companies felt that TDL was enough for demodulation performance requirements, and it was hard to align simulation results using 38.901 CDL at that stage. A new SID [9] on the radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs was approved in June 2018, from which TR 38.827 [10] was approved in June 2020.

The present study item introduces Spatial Channel Models, one CDL based and one TDL based, for the purpose of demodulation requirements and CSI reporting requirements as agreed in TR 38.753, and as agreed in SID [1].

### 2	Description

This SI  has studied candidate spatial channel models for NR demodulation performance requirements in FR1, considering SU-MIMO scenarios, and evaluating CDL-based and Multi cluster TDL-based modelling approaches. The study has:

- Investigated methodology to generate repeatable spatial channel effects with manageable test complexity

- Down selected two stable SCM candidates for future requirements

- Compared performance outcomes across candidate models against agreed test cases

- Collected alignment results from multiple contributors to determine the span and average for key performance metrics.

- Found alignment for the SCM candidates in specified test cases

Regarding SCM Candidates the following observations were drawn:

Regarding rCDL-C1 the following observations can be drawn:

- The study has worked on alignment of rCDL-C1 implementations and has verified that there is a reasonable level of alignment.

- This level of alignment is expected to allow RAN4 to work on normative requirements.

- If the group considers the application of rCDL to settings that involve time-domain prediction it is necessary to further review the deterministic behaviour of rCDL and, if found necessary, identify countermeasures.

Regarding xTDL-C1 the following observations can be drawn:

- The study has worked on alignment of xTDL-C1 implementations and has verified that there is a reasonable level of alignment, among a lower number of contributors than other SCM candidates.

- This level of alignment likely allows RAN4 to work on normative requirements.

Regarding all channel models:

- Applicability of all channel models for band agnostic requirements to be evaluated for specific use case.

- All spatial channel models in this study have fixed angular properties and have non-uniform PMI statistics, hence a fixed precoder may perform better than a channel adaptive precoder found by PMI selection algorithm. Countermeasures can be identified if necessary, including but not limited to, e.g. the legacy beam steering approach defined in LTE and NR specifications [TS 38.101-4 section B.2.3.2.3].

### 3	References

[1]	RP-250577, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#107 meeting, Nokia, BT Plc

[2]	RP-160210, SID: Study on channel model for frequency spectrum above 6 GHz, RAN#71, Samsung, Nokia, March 2016

[3]	3GPP TR 38.900, Study on channel model for frequency spectrum above 6 GHz

[4]	RP-170379, SID: Study on New Radio Access Technology, NTT DOCOMO INC., RAN#75, March 2017

[5]	3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

[6]	3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

[7]	R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4#87, Intel Corporation

[8]	R4-1812001, RAN4#88 Meeting report, RAN4#88

[9]	RP-181325, New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs, RAN#80, CATR, OPPO, Samsung

[10]	3GPP TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

[11]	3GPP TS 38.104, Base Station (BS) radio transmission and reception

[12]	R4-2017678, “Way Forward on PMI Reporting Requirement for NR eMIMO”, RAN4#97-e

[13]	R4-2214397, “WF on UE demodulation and CSI requirements for feMIMO”, RAN4#104b-e

[14]	R4-2210669, “WF on CSI requirement for Rel-17 feMIMO”, RAN4#103-e

[15]	R4-2402277, Discussion on 8Rx general demodulation aspects and spatial channel models, RAN4#110, Nokia, Nokia Shanghai Bell, BT