3GPP TSG RAN Meeting #109                                                    	    	       RP-252730

Beijing, China, September 15-18, 2025

Title:	Discussion on SBFD–SBFD Coexistence Configurations and BS RF Requirements

Source:	ISSDU

Agenda item:	9.1.4.2 BS RF topics

Document for:	Discussion/Decision

Introduction

In RAN#108 meeting, a Way Forward for RAN4-led Rel-20 topics was endorsed in RP-251879. Within the BS RF centric evolution scope, one of the candidate items identified for further study in Rel-20 was [1]:

- Enhancement of co-location requirements - It is FFS on the applicable frequency ranges, e.g., >3GHz requirements, for OTA - R19 outcome should be considered - Coexistence requirement enhancement can be further discussed in RAN#109 - Down select from the following four options in RAN#109 - Additional Hybrid requirement sets for BS type 1-H/large antenna array - SBFD BS to SBFD BS adjacent channel coexistence – focusing on realistic deployment scenarios - It is limited to RAN4 only solution - Neither - Both - No RAN1/2 impact is expected - Target 0.5RF TU in Q4’25 and 1 TU allocation from Q1’26.

Building on this direction and extending the analysis provided in R4-2511538 [2], this contribution focuses on defining clear BS RF requirements for SBFD–SBFD adjacent-channel coexistence. Here, we extend the study to include other feasible configurations that use the downlink sub-band of the adjacent-band operator as a guard-band to improve adjacent-channel isolation and reduce mutual interference. The goal is to evaluate the feasibility of these configurations and support more flexible deployment options from an operator perspective.

Discussion of coexistence scenarios

For SBFD–SBFD coexistence, the DL subband frequency locations are important, since the placement of DL and UL sub-bands decides whether UL at spectrum edges is protected from adjacent DL interference. Following this principle, four feasible DL–DL boundary configurations are considered for Rel-20: UD/DU, UD/DUD, DUD/DU, and DUD/DUD.

It is observed that different configurations result in different RF requirements, especially for ACIR, ACLR, and ACS.

Scenario-specific RF requirements

Compared to DUD-based configurations, the SBFD–SBFD co-existence UD/DU deployment features an enlarged downlink sub-band at the shared boundary, which provides the adjacent operator with a wider guard-band buffer and thereby permits the use of RF filters with more relaxed attenuation and transition-band requirements.

| Scenario | UL exposure | DL sub-band characteristics | RF requirement implication | Illustration |
| --- | --- | --- | --- | --- |
| UD / DU | Both UL sub-bands at outer edges | Wide DL at shared boundary | Relaxed at DL–DL boundary, but stricter ACIR/ACLR at outer edges |  |
| UD / DUD | UL exposed on left side | Right side protected by DUD | Steeper filtering needed on UD side to avoid leakage into UL |  |
| DUD / DU | UL exposed on right side | Left side protected by DUD | Similar to UD/DUD; stricter filtering on DU side with narrow DL transition |  |
| DUD / DUD | UL protected by DL on both sides | Each DL narrower than in UD/DU | No outer-edge UL exposure, but higher selectivity needed due to narrower DLs |  |

However, this benefit is conditional. If an additional operator also places UL at the outer edges, the advantage of the enlarged DL sub-band as a guard-band is reduced. In such multi-operator scenarios, the UL becomes exposed at spectrum edges, resulting in higher interference risks from adjacent DL transmissions. This leads to the classification of UD/DU as an UL-exposed configuration, requiring stricter RF filtering despite the apparent guard-band benefit.

Observation 1: UL-exposed configurations (UD/DU, UD/DUD, DUD/DU) require stricter filtering, as outer-edge UL is more vulnerable to adjacent DL interference.

Observation 2: The protected configuration (DUD/DUD) avoids outer-edge UL exposure, but narrower DL sub-bands require higher filter selectivity.

Proposed objectives (BS RF scope)

The following objectives are proposed for Rel-20 BS RF work. First, it is necessary to limit deployment options to system-level DL–DL boundary-compliant configurations. Only UD/DU, UD/DUD, DUD/DU, and DUD/DUD should be supported, or equivalent setups that consider DL subband frequency locations to ensure adjacent-channel coexistence and protect UL reception. This work should be addressed by RAN4. Second, Rel-20 should set minimum RF requirements per configuration.

For system-level configurations UD/DU, UD/DUD, and DUD/DU, outer-edge UL is exposed, and when another operator places UL at the edges, adjacent DL interference increases, and stricter filtering is required. Clear values for ACIR, ACLR, and ACS should be defined, with stricter requirements applied to UL-exposed configurations (UD/DU, UD/DUD, DUD/DU) and more relaxed requirements for the protected case (DUD/DUD). This objective is also under the responsibility of RAN4.

Based on these objectives, it is concluded that Rel-20 should both limit deployment options to DL–DL boundary-compliant configurations and set minimum BS RF requirements defined per configuration. Accordingly, three proposals are identified:

Proposal 1: Limit SBFD deployments to configurations that consider the DL subband frequency locations, ensuring DL is aligned to protect UL reception.

Proposal 2: Restrict SBFD deployments to DL–DL boundary-compliant configurations (UD/DU, UD/DUD, DUD/DU, DUD/DUD), or equivalent arrangements that improve adjacent-channel coexistence.

Proposal 3: Define differentiated BS RF requirements per configuration, with stricter values for exposed UL cases and more relaxed requirements for protected cases.

Conclusion

This contribution addresses BS RF aspects of SBFD–SBFD adjacent-channel coexistence. The analysis shows that different configurations result in different RF requirements, especially when UL sub-bands are placed at spectrum edges. We have the following observations and proposals for RAN4 BS RF Requirements:

Observation 1: UL-exposed configurations (UD/DU, UD/DUD, DUD/DU) require stricter filtering to suppress interference from adjacent DL.

Observation 2: The protected configuration (DUD/DUD) avoids outer-edge UL exposure, but narrower DL sub-bands require higher filter selectivity.

Proposal 1: Limit SBFD deployments to configurations that consider the DL subband frequency locations, ensuring DL is aligned to protect UL reception.

Proposal 2: Restrict SBFD deployments to DL–DL boundary-compliant configurations (UD/DU, UD/DUD, DUD/DU, DUD/DUD), or equivalent arrangements that improve adjacent-channel coexistence.

Proposal 3: Define differentiated BS RF requirements per configuration, with stricter values for exposed UL cases and more relaxed requirements for protected cases.

- References

[1] RP-251879, “Way forward on RAN4 R20 package”, RAN4 Chair, RAN#108

[2] R4-2511538, “Discussion on SBFD–SBFD Adjacent-Channel Coexistence Scenario”, RAN4#115