3GPP TSG-RAN Meeting #109	RP-252751

Beijing, China, September 15-18, 2025

Agenda Item:	9.1.4.5

Source:	Intel Corporation

Title:	Views on RAN4 Rel-20 5G-A package – Demodulation topics

Document for:	Discussion and decision

# Introduction

This contribution summarizes Intel’s views on the scope of RAN4 5G-A Release 20 work scope for demodulation topics and provides further views on top of the WF endorsed in RAN#108 [1].

# Views on Demodulation topics

The following agreements are provided in RAN #108 WF [1] for demodulation centric RAN4 topics:

The high-level scope for WI on Demod enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - Normative work on SCM based requirements for SU-MIMO in FR1 - The exact scope also depends on the conclusion of R19 study - Study on SCM for MU-MIMO - Performance requirements for 6Rx UE with interference - Up to 4 MIMO layers for the target cell is considered - No RAN1/2 impact is expected - Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

## SCM based performance requirements

Views on objective scope/prioritization

- The work is expected to cover UE demodulation and CSI reporting requirements for the new channel models defined in TR 38.753 for SU-MIMO FR1 scenarios.

- It is recommended to focus on a single SCM methodology (CDL) striving to define band-agnostic requirements

- Limited set of test cases should be considered.

  - PDSCH rank 4 (4R) and rank 8 (8R)

  - eTypeII PMI reporting

- New requirements shall be defined as optional and applicable from Rel-20

Candidate objectives

- Specify FR1 SU-MIMO UE performance requirements for CDL-based models requirements defined in TR 38.753 striving to ensure band-agnostic requirements

  - PDSCH rank 4 demodulation for 4Rx UEs

  - PDSCH rank 8 demodulation for 8Rx UEs

  - eTypeII PMI reporting requirements

Proposal #1:	Demodulation objective “SCM based requirements”

- Define FR1 SU-MIMO UE performance requirements for CDL-based models requirements defined in TR 38.753 for a limited set of test cases

  - PDSCH rank 4 demodulation for 4Rx UEs

  - PDSCH rank 8 demodulation for 8Rx UEs

  - eTypeII PMI reporting requirements

## Further study on SCM

Views on objective scope/prioritization

- Recommend continuing Rel-19 studies in application to MU-MIMO scenarios. To limit the work scope, FR1 scenarios can be prioritized. Also, to avoid divergence and extra workload it is recommended to focus on a single channel modeling methodology (CDL).

- It is anticipated that the outcome of the work should be applicable to 6G as well. We also expect that certain discussions on 6G channel model framework for performance requirements would take place in the scope of 6G SI (e.g. how to accommodate updates made by RAN1 for 7-24GHz channel models in Rel-19, types of models to be used for 6G testing, etc). The new SCM channel models should be designed in a way to ensure their potential applicability to 6G testing in the future and certain alignment with parallel 6G studies should be considered.

Candidate objectives:

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements:

  - Consider CDL-based channel modelling approach. Use CDL models in TR 38.753 as the starting point and identify any necessary changes.

  - The methodology shall include both FR1 and FR2, with first priority for FR1.

  - The channel modelling methodology should be applicable for 5G-A and 6G testing

Proposal #2:	Demodulation objective “Further study on SCM”

- The study objectives shall include

  - Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements:

    - Consider CDL-based channel modelling approach. Use CDL models in TR 38.753 as the starting point and identify any necessary changes.

    - The methodology shall include both FR1 and FR2, with first priority for FR1.

    - The channel modelling methodology should be applicable for 5G-A and 6G testing

- Further discuss how to handle possible overlap with potential 6G SI RAN4 work on channel models for performance requirements.

## Performance requirements for 6Rx UE with interference

Views on objective scope/prioritization

- Per the discussion in RAN #108 the scope of the item should be limited to up to 4 MIMO layers for the target cell

- Specific scenarios to define requirements can be identified by RAN4 WG and should follow the general approach taken for 4Rx and 8Rx requirements.

Candidate objectives

- Specify PDSCH demodulation requirements for 6Rx capable UEs with up to 4 MIMO layers for the target cell

TU allocation

- The workload to introduce extra MMSE-IRC requirements for 6Rx UEs is relatively small, and the work can be planned for 6-9 months in Rel-20.

Proposal #3:	Demodulation objective “Performance requirements for 6Rx UE with interference”

- Confirm RAN#108 agreements with work planned for 6-9 months

# Conclusions

In summary, we make the following proposals on the scope of RAN4 Rel-20 5G-A demodulation topics:

Proposal #1:	Demodulation objective “SCM based requirements”

- Define FR1 SU-MIMO UE performance requirements for CDL-based models requirements defined in TR 38.753 for a limited set of test cases

  - PDSCH rank 4 demodulation for 4Rx UEs

  - PDSCH rank 8 demodulation for 8Rx UEs

  - eTypeII PMI reporting requirements

Proposal #2:	Demodulation objective “Further study on SCM”

- The study objectives shall include

  - Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements:

    - Consider CDL-based channel modelling approach. Use CDL models in TR 38.753 as the starting point and identify any necessary changes.

    - The methodology shall include both FR1 and FR2, with first priority for FR1.

    - The channel modelling methodology should be applicable for 5G-A and 6G testing

- Further discuss how to handle possible overlap with potential 6G SI RAN4 work on channel models for performance requirements.

Proposal #3:	Demodulation objective “Performance requirements for 6Rx UE with interference”

- Confirm RAN#108 agreements with work planned for 6-9 months

# References

RP-251789 “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple), RAN #108, June 2025