3GPP TSG-RAN Meeting #109	RP-251926

September 15th ‒ 18th, 2025

Beijing, CN

Agenda item:	9.1.4.6

Source: 	Moderator (RAN4 vice chair, Qualcomm Incorporated)

Title: 	Moderator's summary for RAN4 REL-20 5G-Adv: NTN topics

Document for:	Information

# Introduction

This document provides a summary of papers submitted to agenda item 9.1.4.6 related to a proposed Rel-20 RAN4 5G-Advanced topics on NTN.  The high level scope for the proposals comes from [1] and guidance for finalization of the RAN4 Rel-20 5G-Adv package in [2].

# Summary

The high level scope for NTN topics in the RAN4 5G-Adv Rel-20 package was agreed in [1] and reproduced below

  - NTN HPUE

    - Finish remaining PC1.5 (~29 dBm) work for hand-held devices for both NR NTN and IoT NTN (several studies and discussions already done in R19)

      - NR NTN with 2TX and/or FFS 1TX

  - NTN HD-FDD VSAT for Ka/Ku bands

    - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm).

    - Once the mobile VSAT with small antennas work is done,

      - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable

    - RAN1/2 impact is expected and will be further discussed in RAN#109

      - Leverage RedCap over NTN HD-FDD solution to NTN as applicable.

  - NTN DL CA

    - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC

      - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands

      - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

    - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3

      - Take one example band with minimum work with PC3

      - The rest cases can be considered as part of spectrum work once the requirements on the example band is completed

- Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26

Most companies submitted inputs into RAN #109 recognize the need to downscope the objectives from the above given the limited TU allocation available.  The company inputs categorized according to the above objectives is provided in the table below

| Tdoc | Title | Source | NTN HPUE | NTN HD-FDD VSAT for Ka/Ku | NTN DL CA | Missing channel bandwidths |
| --- | --- | --- | --- | --- | --- | --- |
| RP-252038 | Views on Rel-20 NTN topics for 5G-A | ZTE Corporation, Sanechips | 1 | 1 | 1 | 1 |
| RP-252071 | Views on 5G-A Rel-20 NTN topics | Samsung | 1.33 | 1.33 | 1.33 |  |
| RP-252161 | RAN4 led NTN topics for 5G Advanced Rel-20 | THALES |  | 1.33 | 1.33 | 1.33 |
| RP-252192 | Views on Rel-20 NTN topics in 5G-A | vivo | 2 |  |  | 2 |
| RP-252335 | NTN HD-FDD Mobile VSAT for Ka and Ku bands | SHARP Corporation |  | 4 |  |  |
| RP-252367 | RAN4 R20 NTN | OPPO | 1.33 |  | 1.33 | 1.33 |
| RP-252452 | Views on the NTN RF enhancements for Rel-20 5G-A | Apple |  |  |  |  |
| RP-252488 | Qualcomm views on RAN4 NTN topics for 5GA Rel-20 | Qualcomm Incorporated | 1.33 | 1.33 | 1.33 |  |
| RP-252515 | Rel-20 RAN4 NTN | MediaTek Inc. | 1.33 | 1.33 | 1.33 |  |
| RP-252522 | Proposal on NTN project scope | Xiaomi Technology | 1 | 1 | 1 | 1 |
| RP-252526 | Views on 5G-A Rel-20 NTN topics | LG Electronics Deutschland | 2 | 2 |  |  |
| RP-252534 | Views on UE RF topics for RAN4 Rel-20 | China Telecom | 4 |  |  |  |
| RP-252556 | Views on Rel-20 NTN topics for 5G-A | China Unicom | 2 | 2 |  |  |
| RP-252587 | Motivation for HD-FDD operation for NR NTN in bands above 10 GHz | Eutelsat Group |  | 4 |  |  |
| RP-252611 | Views on Rel-20 NTN Topics | CMCC | 4 |  |  |  |
| RP-252664 | Considerations on RAN4-led Rel-20 5G-Adv NTN topics | Nokia | 1 | 1 | 1 | 1 |
| RP-252670 | Views on NTN related RAN4 led R20 topics | CSCN | 2 | 2 |  |  |
| RP-252706 | Views on the objectives of the Rel-20 RAN4 NTN enhancements WI | CHTTL | 0 | 2 | 2 |  |
| RP-252715 | Views on NTN works for Rel-20 5G-A | CATT | 1.33 | 1.33 |  | 1.33 |
| RP-252743 | Proposals for RAN4-led Rel-20 NR NTN topics | Huawei, HiSilicon | 2 | 2 |  |  |
| Total |  |  | 27.65 | 27 | 11 | 9 |

In this table, a uniform weighting was applied to each category indicated as a preference from each of the companies inputs.  It can be seen that the categories associated with NTN HPUE and HD-FDD received the greatest interest from the companies.  However, taking into consideration the guidance provided in [2] and the near-term limitations on RAN1/2 TU capacity, only the HPUE and missing channel bandwidths should be included.  The HD-FDD and DL CA objectives can only be considered at RAN #112.

Moderator proposal:  It is proposed to focus on the NTN HPUE and Missing bandwidth objectives for inclusion into the Rel-20 package at this time.

Moderator proposal: During the discussion at RAN #109, it was clarified by companies (see table below) that the HD-FDD objective can be crafted in a way not to impact or only trivially impact RAN1/2.  Moreover, due to the industry interest in HD-FDD compared to missing channel bandwidths (see summary of contributions above), it is proposed to focus on NTN HPUE and HD-FDD for Ka/Ku objectives into the Rel-20 package at this time.

## Possible RAN1/RAN2 impact

Some companies indicate that there is no RAN1/RAN2 impact from the HD-FDD and DL CA objectives.  It may be instructive to poll whether this is the common understanding among all the companies.  Please cast your vote below and add comment if desired.

HD-FDD VSAT for Ka/Ku band

| Company | No impact whatsoever | Trivial impact (i.e., wording change, but no requirement change at all) | Small impact, mainly related to optimization | Unsure if there is impact, needs to be studied | Comment |
| --- | --- | --- | --- | --- | --- |
| SES |  |  |  |  | There is no impact on the FR1 bands as the work on the RedCap can be re-usdd for the half duplex |
| Thales | X | X |  |  | Same view as SES above |
| Hispasat |  |  |  |  | Same view as SES |
| MediaTek |  | X (if only FR1 numerology in Ku band) |  |  | Phase approached will be considered as two steps in this topic and no explicit Objectives in other WGs. RAN1 work will be triggered by RAN4 LS. |
| CSCN | X |  |  |  | No impact on RAN1/2 not only for FR1 but also for FR2. |
| CATT | X |  |  |  | Same view as CSCN |
| LGE |  | X |  |  | Expect no impact by restricting the scope as HD-FDD FR1/Ku band mobile VSAT with small antenna. Other scope can be discussed later maybe after Q2 2026. |
| CTC | X |  |  |  | Both FR1 and FR2 should be supported for HD-FDD. |
| Eutelsat | X for FR1-NTN | X for FR2-NTN |  |  | FR2-NTN needs 120 kHz added to the existing 15 & 30 kHz REDCAP SCS |
| Thales_2 | X for FR1-NTN | X for FR2-NTN |  |  | FR2-NTN needs 120 kHz added to the existing 15 & 30 kHz REDCAP SCS |
| Sharp | X |  |  |  | Same view as SES |
| Airbus |  | x |  |  | FR2-NTN needs 120 kHz added to the existing 15 & 30 kHz REDCAP SCS |
| Xiaomi |  | x |  |  | Reuse existing Redcap for NTN design for VSAT and FR2 |
| ESA |  | X |  |  |  |
| vivo |  | X |  |  | RedCap can be reused with clarification wording update in RAN1. |
| OPPO |  | X |  |  |  |
| Huawei |  | X |  |  |  |
| Spreadtrum |  | X |  |  |  |

RAN4 chair:  If there is no RAN1 impact, then what is the RAN4 impact for HD?

Eutelsat:  We need to take into account antenna directivity in RAN4

Xiaomi:  RAN1 is band and device type agnostic, but not the same for RAN4

Viasat:  We should consider the commercial interests of operators

Chair:  Can we consider HPUE and HD-FDD (remove new channel bandwidths) as the two objectives to move forward on for Rel-20 WI?

Echostar, Viasat, Alyrria, ESA, Thuraya:  Cannot agree to remove DL CA objective at this time.

ST Engineering iDirect: Agree that we should not remove DL CA

DL CA for NTN

| Company | No impact whatsoever | Trivial impact (i.e., wording change, but no requirement change at all) | Small impact, mainly related to optimization | Unsure if there is impact, needs to be studied | Comment |
| --- | --- | --- | --- | --- | --- |
| SES | X |  |  |  | For a contiguous downlink channels there is no RAN1 and on RAN2 |
| Thales | X |  |  |  | Same views as SES above |
| Viasat | X |  |  |  | Based on offline discussion with RAN2 chair, there is no specification impact; UE signalling change is BAU. If companies feel there are impacts (small or large but not trivial), we request them to specify what the specific impacts are given that we are focused on intra-SAN CA which is no different than intra-BS CA for TN Based on offline discussion with RAN1 chair, the RAN1 concern was that RAN1 will have to do something additional over and above the general CA framework if the Tx-Rx spacing was too small. However, n255/n256 does not have that issue given the large Tx-Rx spacing. |
| Thuraya | X |  |  |  | Same view as Viasat |
| Hispasat | X |  |  |  | Same view as Viasat |
| MediaTek |  | X (minor impact to apply TN DL CA into NTN) |  |  | RAN1 work will be triggered by RAN4 LS. |
| CSCN |  |  | X |  | We think it has potential impacts on both RAN1 and RAN2, such as RTT which needs further evaluation. |
| CATT |  |  | X |  | Potential impact, such as the resource allocation/ /HARQ. |
| CTC |  |  | X |  | The requirement is still not clear. |
| Eutelsat |  | X Single band above 10 GHz intra-SAN DL contiguous |  |  | RAN1/2 work would be same as any other CA combination i.e. triggered by LS for this simplest case |
| Sharp | X |  |  |  | Same views as SES |
| Airbus | X |  |  |  |  |
| ESA | X |  |  |  |  |
| ST Engineering iDirect | X |  |  |  |  |
| CSCN2 |  | X |  |  | If intra-SAN, intra-band and contiguous DL CA is considered, trivial impact on RAN1/2. At least for non-contiguous case, different Doppler shifts for CCs should be taken into account. |
| CATT |  | X |  |  | Same view as CSCN2 |
| CTC |  | X |  |  | In the NGSO scenarios, DL CA with non-contiguous carriers may result in different Doppler shifts that should be considered. |

HD-FDD:  No company identified RAN1/RAN2 impact beyond trivial word changes.

DL CA: 2 or 3 companies believe there may be possible small change required in RAN1/RAN2.  See details in the table above.  Other companies asked for more details to be able to assess the possible small change.

LGE:  HPUE and HD-FDD is the best compromise given we are limited to two objectives

Viasat, VDF:  DL CA is very important to us

ZTE:  Same as LGE.  For emergency service over NTN, why can’t this be supported with single carrier?

Viasat: We have fragmented allocations of bandwidth.  We don’t have contiguous 10 MHz.

Sharp: Support the moderator analysis of greatest support of features – HPUE and HD-FDD

CHTTL: Support HPUE and HD-FDD due to balance between L/S and Ka/Ku

Thuraya: Cannot accept HPUE and HD-FDD.  Must have DL CA

Echostar:  We suggest HD-FDD and DL CA, removing HPUE from the NTN WI.

Qualcomm:  HPUE and HD-FDD enables the most use cases so are the most important.  If there is time at the end of the release we may consider CA.

Who would object to a WID that includes only HPUE and HD-FDD?  Viasat, Alyrria, Thuraya, ST Engineering/Idirect, Echostar

Viasat:  We are making progress among NTN companies to move the HPUE objective from NTN WID into the TN WID.

Nokia: If NTN companies prefer to prioritize DL CA and HD-FDD, we are fine, but then we remove the HPUE objective.  We do not move objectives and/or TU’s from NTN to TN.

Xiaomi:  Moving objectives does not help the overall TU situation.  Guidance from RAN4 chair from last meeting, there is only 0.75 TU to TN WID.  We should present two options to the RAN chair for decision … either HPUE + HD-FDD or DL CA + HD-FDD + HPUE (3 objectives).

CMCC: Moving objectives to another WI is not appropriate.  NTN HPUE is our first priority and we want to keep it in the NTN WID.

Vivo: NTN HPUE is also our highest priority.

Samsung: TN WID is stable.  If we add work to TN, then new TU allocation needs to be added.

LGE: We don’t want to drop HPUE.  We need to consider the RAN1 impact of DL CA.

Oppo: Agree with CMCC, vivo.

Huawei: The main issue is whether DL CA has RAN1 impact.  The TN WI is already full, cannot add more objectives.

ST Eng/Idirect:  Maybe we can start the NTN HPUE work later.

Huawei:  We can have a checkpoint.  If you finish HPUE early, can consider the DL CA.  If you cannot finish HPUE early, then there is no DL CA.

ZTE: Same view as Huawei.

# WID objectives

## NTN HPUE

This objective relates to adding specifications to enable power class 1.5 for hand-held devices for both NR NTN and IoT NTN.  This power class was removed from the Rel-19 WID due to insufficient time.  Rel-19 conclusions have been captured into TR 38.863 via Big CR in R4-2510859.  The following aspects were agreeable to most companies and are expected to be stable

- Coexistence for PC1.5 was already completed in Rel-19 (PC1.5 for NR NTN and PC1/PC2 for IoT NTN)

- Applicability to hand-held devices

- Both NR NTN and NB-IoT based IoT NTN

- NR NTN with 2Tx (26 dBm + 26 dBm)

- IoT NTN with 1Tx (29 dBm)

while the following aspects may require further discussion

- Shall NR NTN with 1Tx be included?  This may require feasibility study of the PA and duplexer.  What criteria are required to confirm feasibility?

- Applicability to also include non-handheld?

- Is a duty cycle related SAR solution needed?  Discussion during Rel-19 was not productive.

- Include PC2 for Redcap NTN in FR1?  (new proposal in RP-252534 from China Telecom)

It is suggested to adopt the detailed objectives proposed in RP-252192 (vivo) as a starting point

HPUE for NTN Core Part

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Example bands include bands n256 and 256, n255 and 255

  - Support UE with power class 1.5 (PC1.5, 29 dBm) that applies to both handheld and non-handheld device types.

  - For NR-NTN, 2Tx is considered, as well as 1Tx [if the feasibility can be confirmed. A single set of requirements can apply to both 1Tx and 2Tx implementations.]

  - For NB-IoT based IoT-NTN, 1Tx is considered.

  - Specify RF requirements for PC1.5 for NR-NTN and NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

| Company | Comment |
| --- | --- |
| Viasat | It is our understanding that PC1 for IoT-NTN is already specified for handheld in Rel-19 and not additional work is required. We are ok if PC1.5 for IoT NTN is descoped to allocate TUs for other NTN topics (e.g. DL CA). For PC1.5 for 1Tx, we do not believe feasibility is required. If RAN decides to perform feasibility, the feasibility criterion should be if one company indicates feasibility. |
| Terrestar | Add n252 and 252 as part of the example bands |
| T-Mobile, Echostar | We may be able to combine the work on NTN HPUE and TN HPUE together thereby saving TU to allow other work to be included in Rel-20 package |
| Xiaomi | TN targets PC1.5 for TDD, but for NTN it is FDD. For TN, only NR, but for NTN it is both NR NTN and IoT NTN. |
| MediaTek | One alternative solution is to combine TN and NTN HP UE topic as mentioned by TM-US and Echostar. The scope can be diverse, but we need to harmonize the HP related topics between TN and NTN in future. |
| LGE | Support single set of requirement for 1Tx and 2Tx. Prefer to keep HPUE under NTN WI. |
| CMCC | NTN HPUE is our first priority. Pefer to keep NTN HPUE under NTN WI. The detailed objectives above are fine for us. |
| Sharp | The objectives are acceptable. |

## NTN HD-FDD VSAT for Ka/Ku bands

This objective relates to adding specifications to enable half duplex FDD for VSATs operating in Ka and Ku bands.  The following aspect is agreeable to most companies and is expected to be stable

- Minimize impact to RAN1 by reusing the RedCap mechanisms for UL/DL deconfliction at the UE (VSAT)

while the following aspects may require further discussion

- Antenna assumptions.

  - Some companies propose starting with “ultra small” 20x20 first before proceeding to “normal” sized (i.e., 45x45 and larger) antennas.  Other companies propose not to consider the “normal” antenna at all in the scope of this WI.

  - It is proposed in RP-252161 (Thales) to first consider non-simultaneous Tx-Rx (i.e., half duplex) first, but then to consider a dual beam Rx (still no simultaneous Tx as this is still half duplex) pointing to two different satellites in a second phase

- VSAT types.

  - Applicable to mobile VSAT only?  Or also fixed VSAT as second priority?  Only electronically steered or also mechanically steered?  NGSO and GSO?  New VSAT type needed for the “ultra small” antenna?

It is suggested to adopt the detailed objectives below as a starting point for further revision

HD-FDD VSAT for Ka/Ku bands Core Part

- Specify the support of HD-FDD mode with NR-NTN at VSAT UE level.

  - Note:  It is assumed that the existing RedCap over NTN HD-FDD solution is reused to minimize impact to RAN1/2

  - Specify RF requirements for the “ultra-small” 20x20cm mobile VSAT for NR NTN in all Ka bands (e.g., n512, n511, n510) and all Ku-bands (e.g., n5xx, n2xx) allocated to NTN satellite services.

    - Electronically steered mobile VSAT for NGSO is considered

- Specify RF and RRM requirements to support HD-FDD VSATs

- Phase 1: Starting from both Ku and Ka band with FR1/FR2 numerology

  - Introduce requirements for HD-FDD for VSAT type 6

    - VSAT type 6: Mobile VSAT communicating with LEO only with electronic steering antenna.

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm)

- Phase 2:  (after the completion of Phase 1)

  - Introduce necessary requirements for NTN HD-FDD for VSAT type 1 to 5 for both Ku and Ka band with FR1/FR2-numerology

- Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

- Specify necessary RRM requirements

| Company | Comment |
| --- | --- |
| Eutelsat | Agree with the above text with the additional step of starting with the existing Type 6 (Mobile VSAT served by LEO) to establish the baseline impact of HD-FDD (e.g., sensitivity – comparison of Tx switch with duplexer). RRM can consider requirements for HD-FDD with directional antenna (new requirements as a result of HD operation, similar to eRedCap NTN); i.e., no dependency upon antenna size or band. Two Rx beams (one Tx beam) could be considered in a second step. Then, once the HD-FDD baseline is established, proceed with specification of VSAT UE HD-FDD ‘ultra-small’ flat panel antenna core requirements. Note: SAN is FD-FDD and its specification is not impacted by HD-FDD VSAT. |
| JSAT | Electronically steered normal sized (i.e., 45x45 and larger) antennas with fixed VSAT for GSO should be considered. Users who need to visit sites for broadcasting, earthquake observation, and similar purposes, carrying antennas with them, are seeking antennas that are not only lighter but also higher in performance. In addition, in marine environments, there is a demand for antennas that make efficient use of the limited space on ships and are more durable than mechanical antennas. |
| CHTTL | In RAN4 specification, we didn’t clearly restrict the VSAT antenna size, 20x20cm is just a target. To align with the agreed WF in the previous RAN that HD-FDD support on other sizes/types of antenna is also considered, therefore we suggest the following: - Specify RF requirements for the HD-FDD VSAT for NR NTN in all Ka bands (e.g., n512, n511, n510) and all Ku-bands (e.g., n509, n508, n248, n247) allocated to NTN satellite services. - Start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm) as 1st priority, and HD-FDD for fixed VSAT and the mobile VSAT with large/normal size antenna as 2nd priority. |
| MediaTek | We also agree with Eutelsat for the starting of VSAT type 6 firstly as phase approach as following - Specify RF and RRM requirements to support HD-FDD VSATs - Phase 1: Starting from Ku band with FR1 numerology - Introduce requirements for HD-FDD for VSAT type 6 - VSAT type 6: Mobile VSAT communicating with LEO only with electronic steering antenna. - Introduce requirements for HD-FDD VSAT with small antennas (e.g., 20x20cm) - Phase 2: - Introduce NTN HD-FDD for VSAT type 1 to 5 for FR1-numolonogy Ku band - Introduce NTN HD-FDD for all defined VSAT types for FR2-numolonogy Ku and Ka-band Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable. |
| LGE | We support the objective proposed by moderator. We can also consider type 6 mobile VSAT with same priority to the “ultra-small” 20x20cm mobile VSAT. Based on agreed WF in last RAN, we are fine to consider HD-FDD for fixed VSAT and the mobile VSAT with large/normal size antenna as 2nd priority. |
| Sharp | The objectives are acceptable. No new VSAT types are needed. |
| CSCN | We support the original version of objectives proposed by moderator in principle. For the 2-phase approach based on different FRs (FR1 and FR2), it is unacceptable for us. According to the WF, we propose the revised version as followed. - Specify the support of HD-FDD mode with NR-NTN at for VSAT UE level. - Note: It is assumed that the existing RedCap over NTN HD-FDD solution is reused to minimize impact to RAN1/2 - Specify RF requirements for the “ultra-small” (e.g. 20x20cm) mobile VSAT for NR NTN in all Ka bands (e.g., n512, n511, n510) and all Ku-bands (e.g., n5xx, n2xx) allocated to NTN satellite services. - Electronically steered mobile VSAT for NGSO is considered - Specify necessary RF requirements for other VSAT types on supporting HD-FDD as 2nd priority. - Specify necessary RRM requirements Note: Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable. |
| CATT | Same view as CSCN |
| MediaTek | Based on the adhoc session discussion, updated objectives as follow - Specify RF and RRM requirements to support HD-FDD VSATs - Phase 1: Starting from both Ku and Ka band with FR1/FR2 numerology - Introduce requirements for HD-FDD for VSAT type 6 - VSAT type 6: Mobile VSAT communicating with LEO only with electronic steering antenna. - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) - Phase 2: (after the completion of Phase 1) - Introduce necessary requirements for NTN HD-FDD for VSAT type 1 to 5 for both Ku and Ka band with FR1/FR2-numerology |
| LGE | We propose to have some editorial update as highlighted yellow - Specify RF and RRM requirements to support HD-FDD VSATs - Phase 1: Starting from Ku band with FR1 numerology - Introduce requirements for HD-FDD for VSAT type 6 - VSAT type 6: Mobile VSAT communicating with LEO only with electronic steering antenna. - Introduce requirements for mobile HD-FDD mobile VSAT with small antennas (e.g., 20x20cm) - Phase 2: (after the completion of Phase 1) - Introduce necessary requirements for NTN HD-FDD for VSAT type3 to 5 for FR1-numerology Ku band - Introduce NTN HD-FDD for all defined VSAT types for FR2-numerology Ku and Ka-band |

## NTN DL CA

It is suggested to adopt the detailed objectives below as a starting point for further revision

- Specify NR NTN CA framework for UE and BS core requirements for downlink contiguous and non-contiguous intra-SAN intra-band carrier aggregation with 2 DL component carriers and 1 UL

- Specify RF and RRM core requirements for intra-SAN, intra-band contiguous and non-contiguous for the following bands:

|  | CC1 | CC2 |
| --- | --- | --- |
| n255 | 5, 10, 15, 20 | 5, 10, 15, 20 |
| n512 | 50, 100, 200, 400 | 50, 100, 200, 400 |

- Specify performance requirements for intra-SAN DL CA

| Company | Comment |
| --- | --- |

# Reference

- RP-241879, “WF for RAN4 led REL-20 topics,” RAN4 chair (Apple)

- RP-251928, “Release 20 5G-Adv RAN4 Package,” RAN/RAN1/RAN2/RAN4 Chairs

- RP-252038, “Views on Rel-20 NTN topics for 5G-A,” ZTE Corporation, Sanechips

- RP-252071, “Views on 5G-A Rel-20 NTN topics,” Samsung

- RP-252161, “RAN4 led NTN topics for 5G Advanced Rel-20,” THALES

- RP-252192, “Views on Rel-20 NTN topics in 5G-A,” vivo

- RP-252335, “NTN HD-FDD Mobile VSAT for Ka and Ku bands,” SHARP Corporation

- RP-252367, “RAN4 R20 NTN,” OPPO

- RP-252452, “Views on the NTN RF enhancements for Rel-20 5G-A,” Apple

- RP-252488, “Qualcomm views on RAN4 NTN topics for 5GA Rel-20,” Qualcomm Incorporated

- RP-252515, “Rel-20 RAN4 NTN,” MediaTek Inc.

- RP-252522, “Proposal on NTN project scope,” Xiaomi Technology

- RP-252526, “Views on 5G-A Rel-20 NTN topics,” LG Electronics Deutschland

- RP-252534, “Views on UE RF topics for RAN4 Rel-20,” China Telecom

- RP-252556, “Views on Rel-20 NTN topics for 5G-A,” China Unicom

- RP-252587, “Motivation for HD-FDD operation for NR NTN in bands above 10 GHz,” Eutelsat Group

- RP-252611, “Views on Rel-20 NTN Topics,” CMCC

- RP-252664, “Considerations on RAN4-led Rel-20 5G-Adv NTN topics,” Nokia

- RP-252670, “Views on NTN related RAN4 led R20 topics,” CSCN

- RP-252706, “Views on the objectives of the Rel-20 RAN4 NTN enhancements WI,” CHTTL

- RP-252715, “Views on NTN works for Rel-20 5G-A,” CATT

- RP-252743, “Proposals for RAN4-led Rel-20 NR NTN topics,” Huawei, HiSilicon