3GPP TSG-RAN WG1 Meeting #122bis	   R1-2506933

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	10.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on inter-vendor training collaboration for two-sided AI/ML models

Document for:	Discussion and Decision

# Introduction

In this paper, we provide our views regarding inter-vendor training collaboration for Direction A Option 4-1, considering the content/format of the dataset may be essential as SA WG study inputs. Regarding the related discussions for Direction C, they will be discussed in future meetings depending on the RAN4 feedback.

# Discussion

## Data samples attributes

Regarding Target CSI and CSI feedback, the following agreement was reached in RAN1#122 [1].

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format. - FFS: CSI feedback type and format. - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

### Target CSI

Regarding Target CSI type, we think besides precoding matrix, channel matrix should also be supported at least for TDD networks. As discussed in details in our companion paper [2], if channel matrix is used as the Target CSI at the input of the UE-side encoder, NW-side decoder can readily fuse the CSI feedback and an SRS-based channel estimate to derive a more accurate reconstructed CSI.

Regarding Target CSI quantization format, the following proposal was discussed in FL summary [3].

### Proposal 1-3: For Option 4-1 under Direction A in AI/ML based CSI compression, for target CSI format, further study the following two approaches: - Option 1: Target CSI format reuses the same format as NW-side data collection for training. - Option 2: Target CSI format for inter-vendor training collaboration is designed separately.

Assuming dataset delivery is performed via a non-OTA solution, the overhead may not be an essential issue and higher resolution quantization methods should be supported to facilitate more accurate UE-side training. This proposal is tangled with the ongoing discussion about the Target CSI quantization for NW side data collection in Agenda 10.1.1.2. If RAN1 achieves convergence on a high-resolution quantization method/format, e.g., high-resolution eType II-like codebook or scalar quantization in Agenda 10.1.1.2, the same format can be applied to the Target CSI of the exchanged dataset. Otherwise, if the group cannot achieve consensus on a high-resolution quantization in Agenda 10.1.1.2, then scalar quantization FP32, Float16, or 8 bits quantization should be considered for the Target CSI of the exchanged dataset.

Proposal 1: For Target CSI in Option 4-1 under Direction A:

- For Target CSI type, support both precoding and channel matrix.

- For Target CSI quantization format, it depends on the discussion of Agenda 10.1.1.2

  - If RAN1 achieves consensus on supporting the high-resolution quantization method/format for Target CSI in NW side data collection, the same format can be applied for dataset exchange.

  - Otherwise, support SQ with FP32, Float16, or 8 bits quantization.

### CSI feedback

The following agreement was achieved in RAN1#122 on CSI feedback type and format.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent message before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

The issue for down selection is whether the exchanged CSI feedback is the latent message before quantization or the binary sequence after quantization (Option 1 vs. Option 2). Based on Rel-18 studies, both options are functional while Option 1 shows slightly better SGCS performance. We prefer Option 1 both based on its SGCS performance and the fact that Option 1 provides more information to the UE side. Considering that the quantization information is anyway conveyed to/known by the UE side, UE side can also obtain quantized bit sequence if latent message before quantization is provided as in Option 1. Thus, Option 1 enables UE side more flexibility to train the UE part model based on either latent message or quantized bit sequence.

On the other hand, regarding the performance target calculation, it should be calculated based on the quantized bit sequence.

Proposal 2: For CSI feedback in Option 4-1 under Direction A, support Option 1: the exchanged CSI feedback is the latent message before quantization.

- Regarding the performance target calculation, it should be calculated based on the quantized bit sequence.

### Association between Target CSI and CSI feedback

Regarding how to represent the association between Target CSI and CSI feedback, the content of Target CSI and the content of its corresponding CSI feedback can be mapped by order into specific fields. In particular, for different combinations of {Tx port pattern, subband pattern}, the contents of Target CSI can be mapped to separate fields. Similarly, for each Target CSI, the contents of its corresponding CSI feedbacks for different values of {CSI payload size} are also mapped to separate fields. The association between the Target CSI and its corresponding CSI feedbacks can be determined by the specific locations of their corresponding fields in the dataset. An example of the mapping order is provided in Section 2.5.

Proposal 3: For Option 4-1 under Direction A, the association between Target CSI and CSI feedback can be represented by mapping the content of Target CSI and the content of its corresponding CSI feedback by order into specific fields.

## Assisted information for dataset

Performance target

Regarding performance target, the following agreement was reached in RAN1#122:

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following methods for performance target in the exchanged dataset, - SGCS - FFS: Average SGCS - FFS: SGCS values at X-percentiles - NMSE - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization. - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook. - FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration.

Regarding performance metric, we think SGCS is more appropriate if UE first trains the nominal decoder before training its encoder while NMSE is more appropriate if the UE directly trains its encoder. Since how UE trains its model(s) may not be specified and left to the UE implementation, we propose to include both SGCS-based and NMSE-based performance targets in the exchanged dataset.

When the performance target is SGCS-based, we think both Average SGCS and X-percentile values for SGCS can be useful performance targets. For X-percentile, it reflects the distribution/variation of the testing performance, and can be visible/controllable during the training procedure, e.g., by setting the batch size, epoch number, etc. Meanwhile, the exact training method and the composition of the dataset samples are left to implementation, we suggest to include both Average SGCS and X-percentile values for SGCS as performance targets in the exchanged dataset.

Regarding the two FFSs under NMSE performance target, irrespective to whether the format of the exchanged CSI feedback is FP values (i.e., latent message) or quantized bit sequence, the NMSE metric should be calculated based on the latent message dequantized from the bit sequence, since this reflects the anticipated performance under inference phase where the CSI feedback is also in forms of bit sequence. Similarly, for SGCS, the metric is also calculated based on the latent message dequantized from the bit sequence, i.e., the recovery CSI should be the output of the Decoder with quantized CSI feedback as the Decoder input.

Proposal 4: For Option 4-1 under Direction A, for performance target in the exchanged dataset:

- Support both SGCS and NMSE as performance targets.

- Support both Average value and X-percentile values.

- For the calculation of both SGCS and NMSE, the metric is calculated based on the latent message dequantized from the bit sequence irrespective to whether the format of the exchanged CSI feedback is floating vector (i.e., latent message) or quantized bit sequence.

Regarding the last FFS, we think it is reasonable to have multiple performance targets for different layers (when the target CSI type is precoding matrix), since, based on SI evaluations, SGCS performance of higher layer indices (e.g., layers 3 and 4) is considerably lower than the SGCS performance of the lower layer indices (e.g., layers 1 and 2) and using the same SGCS target for all layers requires to lower the SGCS target for layer 1 and layer 2 to a value that is also achievable for layer 3 and layer 4. However, such an approach can substantially reduce the reconstructed target CSI accuracy for layer 1 and layer 2.

In addition, for a specific combination of {Tx port number, subband pattern, CSI payload size}, separate performance target values are provided, as different combinations may correspond to different CSI compression ratios: It is reasonable that a larger CSI compression ratio corresponds to a smaller performance target value and vice versa. Note that each CSI payload size is represented by a specific combination of values of d (length of the latent message), Q (number of bits per scalar/segment), L (length of the segment), as well as partial UCI omission pattern (considering partial UCI omission of bit sequence within a layer may occur during inference as analyzed in [2]).

Proposal 5: For Option 4-1 under Direction A, support multiple performance targets

- for different layers when the target CSI type is precoding matrix.

- for different combinations of {Tx port pattern, subband pattern, CSI payload size}, where each CSI payload size is represented with a specific combination of values of d (length of the latent message), Q (number of bits per scalar/segment), L (length of the segment), as well as partial UCI omission pattern.

Considering the exchanged dataset corresponds to NW side nominal Encoder (E_NW), the UE side trained actual Encoder (E_UE) should mimic E_NW regardless of the training method. Thus, the metrics should at least be applied for Encoder (if UE side directly trains Encoder, using Target CSI as input and CSI feedback as label), and Decoder (if UE side first trains Decoder, using CSI feedback as input and Target CSI as label). If we only consider End-to-End metric, UE side may train the E_UE paired well with its own Decoder (D_UE) but does not mimic E_NW well, which will cause failed pairing later in the inference phase.

Proposal 6: For the performance target in Option 4-1 under Direction A, at least consider to provide the performance metric (in forms of NMSE) for the Encoder (between the Encoder output and the testing CSI feedback) as well as the performance metric (in forms of SGCS) for the Decoder (between the Decoder output and the testing Target CSI).

- End-to-End performance metric (in forms of SGCS) between the Decoder output and the testing Target CSI can be additionally provided.

Model structure related information

The following FL proposal on model structure related information has been discussed during RAN1#122.

### Proposal 2-2 (v1): For Option 4-1 under Direction A in AI/ML based CSI compression, further study the following two approaches to align model design aspects: - Option 1: Additional assisted information is transmitted to align the model design aspects: - Model backbone type for reference encoder, as well as hyper parameters if needed - Tokenization dimension and feature dimension mapping in the reference encoder - Scalability options used in reference encoder. - Note: NW-side proprietary information should not be disclosed. - Option 2: UE assumes the dataset is generated with the same tokenization and scalability options of the RAN4 defined / specified.

In our view, the benefit of Option 4-1 compared with Option 3a-1/Option 5 is that it can avoid exposing the proprietary information of the model/model structure. During SI, at least our evaluation shows that the model backbone mismatch between NW part model and UE part model will not impact the pairing performance.

Regarding the necessity of sharing the Tokenization/feature dimension information as well as the scalability option, the Tokenization and feature dimension is only valid for the transformer model backbone, and they are neural-network-layer specific. Usually in the encoder design, whether for Transformer or CNN, there is not only one NN layer, and the input dimension of each NN layer are different due to vendor implementations. Scalability options is also related deeply with the exact model structure. Therefore, it also needs a clear justification why they should be exposed to the other side.

Proposal 7: For the performance target in Option 4-1 under Direction A, do not support sharing the model structure related information.

## Pairing ID

In RAN1#122, RAN1 reached the following agreement related to Pairing ID.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

A major debate during RAN1#122 was whether it is within RAN1 purview to discuss Pairing ID design and its possible uniqueness or this discussion should be left to RAN2. We think Pairing ID is an integral part of specification of dataset exchange which is led by RAN1 and, therefore, RAN1 can at least agree on some characteristics of Pairing ID before sending a LS to RAN2 for more detailed design. Note also that how to assign the Pairing IDs to different datasets has RAN1-related implications. For instance, is it allowed that two different datasets have the same Pairing ID and, if it is allowed, what is the UE side training behavior? For instance, should the UE side only use the more recent dataset or merge the two datasets for training? Despite the fact that the training method is UE implementation, what Pairing ID represents should be clarified in RAN1 to facilitate a reasonable training implementation at the UE side.

Considering the UE side needs to receive datasets from different NW vendors, to avoid any ambiguity during pairing procedure, it is reasonable to identify each dataset by a globally unique Pairing ID. This can be achieved by specifying that the Pairing ID is unique per PLMN so that it can be applied together with PLMN ID to achieve the global uniqueness.

Proposal 8: Consider the Pairing ID as PLMN unique.

To address the FFS in the above agreement, note that the global uniqueness of Pairing ID does not imply that the same Pairing ID cannot be used when multiple datasets are in fact different batches of the same larger dataset. Two main examples of sending one dataset through more than one batches are:

- Case 1: When a new dataset batch is used to update an exchanged dataset batch with new samples.

- Case 2: When one dataset is split into multiple dataset batches due to, for instance, overhead restrictions.

In either of the above cases, the same Pairing ID should be used for all batches to indicate to the UE side that the batches actually belong to the same larger dataset and, therefore, may be augmented for the training purpose. However, additionally, it is useful to assign a unique subset ID to each dataset batch to facilitate identifying each batch at both NW side and UE side.

Proposal 9: Subject to one Pairing ID, subset ID can be introduced to indicate a subset of the full dataset represented by the Pairing ID, considering the following two cases:

- Case 1: When a new dataset batch is used to update an exchanged dataset batch with new samples.

- Case 2: When one dataset is split into multiple dataset batches before dataset exchange.

## Quantization codebook

In RAN1#122, RAN1 reached the following agreement related to quantization codebook.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, if the quantization codebook for CSI feedback is not fixed by the specification, support exchange of quantization codebook along with dataset exchange, - FFS: Quantization codebook exchange method (e.g, look up table, , or how to exchange quantization codebook related information, or other methods) - FFS: Common or different quantization codebook for different CSI payload size - Note: leveraging the discussion in agenda item 10.1.1.1 when applied

To address FFSs in above agreement, first note that even if the latent message before quantization is used to train the UE side encoder (and nominal decoder) during inter-vendor collaboration, the trained model should be tested under a condition that mimics the actual inference phase where the latent message goes through quantization to generate CSI feedback before OTA transmission and the CSI feedback is de-quantized before being fed as the NW side decoder input. This implies that the same CSI feedback quantization methods that are supported in inference phase should also be supported during inter-vendor collaboration. This also reduces the specification effort for CSI feedback quantization for different operational phases.

Regarding the quantization codebook description, we think a look up table can be adopted. For SQ, each scalar element is quantized to Q bits with uniform quantization between 0 and 1. For VQ, the codebook is composed of 2^Q codewords each with an index represented by Q bits, and each codeword of length L is represented with L FP32 elements.

Regarding the issue of common or different quantization codebook for different CSI payload sizes, we think a unified codebook but with flexible configurations can be considered to adapt to different layers and configurations of {Tx port, subband number, CSI payload size}. In particular, for SQ, different quantization granularities (2 bits or 4 bits) can be configured. For VQ, as mentioned in our companion contribution [2], a common master codebook is adopted with a fixed L for different layers or other configurations (e.g., Tx port, subband number, length of d which could be a multiple of L, etc.). And a subset of the codebook is selected which results in flexible Q values and, therefore CSI payload size under different configurations/layers.

Proposal 10: For Option 4-1 under Direction A inter-vendor training collaboration, consider a look up table for SQ or VQ quantization methods for CSI feedback:

- SQ: each scalar element is quantized to Q bits (configurable Q=2, 4) with uniform quantization between 0 and 1.

- VQ: the codebook is composed of 2^Q codewords each with an index represented by Q bits, and each codeword of length L is represented with L FP32 elements.

  - Common master codebook with fixed segment size L across all layers (for precoding matrix feedback) and configurations of {Tx port, subband number, CSI payload size (including d/Q/L)} is supported. Different Q values can be obtained by selecting subset(s) from the master codebook to achieve different CSI payload sizes for different configurations/layers.

## Dataset structure

The model should be trained for multiple supported numbers of Tx ports, sub-bands, layers/Rx antennas (for precoding/channel matrix), and CSI payload sizes. Therefore, for each layer/Rx antenna, dataset should include Target CSI samples for different numbers of Tx ports and sub-bands. Further, for each Target CSI sample, multiple CSI feedbacks with different payload sizes can be included. As for the supported values, we think 16, 32, 64, and 128 ports for Tx port number and 1,2,3,4 for layer/Rx-port number should be supported. We think the value range of subband number can follow legacy MIMO supported values where the subband configuration can be indicated as in csi-ReportingBand.

In addition, as discussed in previous sections, Pairing ID, subset ID, quantization information for Target CSI and CSI feedback as well as performance target information should be included in the dataset. Finally, testing data samples with similar construction format as training data samples should be included in the dataset.

An example of dataset structure for Option 4-1 Direction A is provided below.

Table 1 Example of the dataset structure for Option 4-1

| Pairing ID |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subset ID (Optional) |  |  |  |  |
| Target CSI Config#1 (Tx port, subband info, layer/Rx index) | Target CSI1#1 | Target CSI1#2 | … | Target CSI1#N1 |
| CSI feedback Config#1 (d, Q, L, UCI omission info) | CSI feedback1#1 | CSI feedback1#2 | … | CSI feedback1#N1 |
| CSI feedback Config#2 (d, Q, L, UCI omission info) | CSI feedback2#1 | CSI feedback2#2 | … | CSI feedback2#N1 |
| … |  |  |  |  |
| CSI feedback Config#L (d, Q, L, UCI omission info) | CSI feedbackL#1 | CSI feedbackL#2 | … | CSI feedbackL#N1 |
| Testing data samples (with similar construction format as training data samples) |  |  |  |  |
| Performance target information for {Target CSI Config#1, CSI feedback Config#1},…,{ Target CSI Config#1, CSI feedback Config#L} |  |  |  |  |
| Target CSI Config#2 (Tx port, subband num info, layer/Rx index) | Target CSI2#1 | Target CSI2#2 | … | Target CSI2#N2 |
| CSI feedback Config#1,…, CSI feedback Config#L | … | … | … | … |
| Testing data samples (with similar construction format as training data samples) |  |  |  |  |
| Performance target information for {Target CSI Config#2, CSI feedback Config#1},…,{ Target CSI Config#2, CSI feedback Config#L} |  |  |  |  |
| … |  |  |  |  |
| Target CSI Config#M (Tx port, subband info, layer/Rx index) | Target CSIM#1 | Target CSIM#2 | … | Target CSIM#NM |
| CSI feedback Config#1,…, CSI feedback Config#L | … | … | … | … |
| Testing data samples (with similar construction format as training data samples) |  |  |  |  |
| Performance target information for {Target CSI Config#M, CSI feedback Config#1},…,{ Target CSI Config#2, CSI feedback Config#L} |  |  |  |  |
| Quantization information for Target CSI |  |  |  |  |
| Quantization information for CSI feedback |  |  |  |  |

# Conclusions

Proposal 1: For Target CSI in Option 4-1 under Direction A:

- For Target CSI type, support both precoding and channel matrix.

- For Target CSI quantization format, it depends on the discussion of Agenda 10.1.1.2

  - If RAN1 achieves consensus on supporting the high-resolution quantization method/format for Target CSI in NW side data collection, the same format can be applied for dataset exchange.

  - Otherwise, support SQ with FP32, Float16, or 8 bits quantization.

Proposal 2: For CSI feedback in Option 4-1 under Direction A, support Option 1: the exchanged CSI feedback is the latent message before quantization.

- Regarding the performance target calculation, it should be calculated based on the quantized bit sequence.

Proposal 3: For Option 4-1 under Direction A, the association between Target CSI and CSI feedback can be represented by mapping the content of Target CSI and the content of its corresponding CSI feedback by order into specific fields.

Proposal 4: For Option 4-1 under Direction A, for performance target in the exchanged dataset:

- Support both SGCS and NMSE as performance targets.

- Support both Average value and X-percentile values.

- For the calculation of both SGCS and NMSE, the metric is calculated based on the latent message dequantized from the bit sequence irrespective to whether the format of the exchanged CSI feedback is floating vector (i.e., latent message) or quantized bit sequence.

Proposal 5: For Option 4-1 under Direction A, support multiple performance targets

- for different layers when the target CSI type is precoding matrix.

- for different combinations of {Tx port pattern, subband pattern, CSI payload size}, where each CSI payload size is represented with a specific combination of values of d (length of the latent message), Q (number of bits per scalar/segment), L (length of the segment), as well as partial UCI omission pattern.

Proposal 6: For the performance target in Option 4-1 under Direction A, at least consider to provide the performance metric (in forms of NMSE) for the Encoder (between the Encoder output and the testing CSI feedback) as well as the performance metric (in forms of SGCS) for the Decoder (between the Decoder output and the testing Target CSI).

- End-to-End performance metric (in forms of SGCS) between the Decoder output and the testing Target CSI can be additionally provided.

Proposal 7: For the performance target in Option 4-1 under Direction A, do not support sharing the model structure related information.

Proposal 8: Consider the Pairing ID as PLMN unique.

Proposal 9: Subject to one Pairing ID, subset ID can be introduced to indicate a subset of the full dataset represented by the Pairing ID, considering the following two cases:

- Case 1: When a new dataset batch is used to update an exchanged dataset batch with new samples.

- Case 2: When one dataset is split into multiple dataset batches before dataset exchange.

Proposal 10: For Option 4-1 under Direction A inter-vendor training collaboration, consider a look up table for SQ or VQ quantization methods for CSI feedback:

- SQ: each scalar element is quantized to Q bits (configurable Q=2, 4) with uniform quantization between 0 and 1.

- VQ: the codebook is composed of 2^Q codewords each with an index represented by Q bits, and each codeword of length L is represented with L FP32 elements.

  - Common master codebook with fixed segment size L across all layers (for precoding matrix feedback) and configurations of {Tx port, subband number, CSI payload size (including d/Q/L)} is supported. Different Q values can be obtained by selecting subset(s) from the master codebook to achieve different CSI payload sizes for different configurations/layers.

# References

Report of RAN1#122, Chairman, August 25th – 29th, 2025.

R1-2506931, “Discussion on inference related aspects for CSI compression,” Huawei, HiSilicon, RAN1#122bis.

R1-2506470, FL summary #2 for inter-vendor training collaboration, Moderator (Apple), RAN1#122.