3GPP TSG RAN WG1 #122bis                                 																											R1-2507005

Prague, Czech, Oct 13th – 17th, 2025

Source: 	CMCC

Title:	Discussion on UE features for NTN for NR Phase 3

Agenda item:	9.8

Document for:	Discussion & Decision

- Introduction

In this contribution, we will discuss and share our views on UE features for NTN for NR Phase 3, including aspects of support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands and NR-NTN uplink capacity/throughput enhancement.

- Discussion

  - 2.1 UE feature for RedCap and eRedCap UEs with NR NTN

During the RAN1#121 meeting, an agreement was achieved to introduce a FG to support handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE [1].

Agreement: Update FG 65-2-1 as follows: - Confirm Component 1 and 2 as they are - Remove “FFS: whether to divide this FG into two separate FGs between case 3 and case 4” - Put whether handling of collision case 4 as FFS - Remove “FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state” - Put N/A for Need of FR1/FR2 differentiation - Clarify definitions of collision case 3 and 4 such that “Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states, and collision Case 4 is for RRC CONNECTED state only” 65-2-1; Handling collision Case 3 [and collision Case 4] for half-duplex FDD operation for (e)RedCap UE; [1. Support handling collision Case 3] [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state; FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2; YES; N/A; Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported; Per band; N/A; FFS N/A; N/A; Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission [Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission] Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states[, and collision Case 4 is for RRC CONNECTED state only]; Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG.

Feature group column

The brackets around “[2. Support handling collision Case 4]” can be removed.

Regarding “FFS: whether to divide this FG into two separate FGs between case 3 and case 4”, prefer to maintain a unified FG given FG28-3 (NR_RedCap-> Half-duplex FDD operation type A for RedCap UE) already encompasses Cases 1/2/5/6.

Prerequisite feature groups column

Regarding “FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2”, the following dependency hierarchy should be established:

- 26-1 (NR_NTN_solutions->Uplink Time and Frequency pre-compensation and timing relationship enhancements)

- One of {28-1 (NR_RedCap-> RedCap UE), 48-1 (NR_redcap_enh-> RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1), 48-2 (NR_redcap_enh-> RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1)}

- 28-3 (NR_RedCap-> Half-duplex FDD operation type A for RedCap UE)

While FG26-4 (NR_NTN_solutions-> UE reporting of information related to TA pre-compensation) is deemed non-mandatory for this feature implementation.

Need of FR1/FR2 differentiation column

According to the WID, only FR1-NTN is considered for the collision issue. Then only FR1 should be considered for the UE capability.

Mandatory/Optional column

Regarding “FFS: A UE that supports FG 28-3 in NTN band must support this FG”, we don’t think HD-FDD UE must support handling collision Case 3 and collision Case 4, thus we prefer to remove the FFS.

Based on the above discussion, the following FG for the support of handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE can be considered.

Proposal 1: Introduce the following FG for the support of handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE:

| 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | [ 1. Support handling collision Case 3 ] [ 2. Support handling collision Case 4 ] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state | FFS: 26-1, 26-4, 28-1, 28-3, one of { 28-1, 48-1, 48-2 } | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | FFS N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 2.3 UE feature for NR NTN uplink capacity/throughput enhancement

As discussed in AI 9.11.3, in the RAN1#119 meeting, RAN1 agreed on the uplink capacity enhancement technology based on inter-slot OCC. And the following agreement was made.

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

According to Note 1, there will be separate UE capabilities for OCC length 2 and OCC length 4, where the UE capability for OCC length 2 is a prerequisite for the UE capability for OCC length 4, so the following two FGs should be Adopted.

According to reply LS from RAN4[4], the phase continuity requirements for the duration of one OCC group with PUSCH is same as DMRS bundling. Specifically, ≤25° between adjacent slots. The side conditions will also be aligned with DMRS-bundling requirement. Therefore, Similar description keeping phase continuity and power consistency for DMRS bundling can be reused for OCC.

Another important information from RAN4 is opinion that UE should be able to indicate support for OCC independently for GSO and NGSO deployments. For DMRS bundling, two UE FGs are defined, i.e. FG30-4 and FG44-2. The FG30-4 only applies to TN and GSO scenario while FG44-2 applies to NGSO scenario. The main difference for NGSO and GSO scenario is whether UE need to perform pre-compensation to keep phase rotation due to timing drift. Consider the similar situation for OCC and DMRS bundling, separate FGs should be adopted.

Proposal 2: Adopt the following FGs for NR NTN uplink capacity/throughput enhancement.

| 65-3-1 | PUSCH with inter-slot OCC in NGSO scenarios | 1. Support inter-slot OCC for DFT-s-OFDM PUSCH transmissions with repetition Type A 2. Support RV cycling across OCC groups 3. Support collision handling between PUCCH and PUSCH with OCC 4. Support of keeping phase continuity and power consistency across one OCC group 5.Supported OCC length 6.Support of pre-compensation to keep phase rotation due to timing drift across one OCC group | PUSCH repetition Type A | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Component 5 candidate values: {(2), (2 and 4)} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-3-2 | PUSCH with inter-slot OCC in GSO scenarios | 1. Support inter-slot OCC for DFT-s-OFDM PUSCH transmissions with repetition Type A 2. Support RV cycling across OCC groups 3. Support collision handling between PUCCH and PUSCH with OCC 4. Support of keeping phase continuity and power consistency across one OCC group 5.Supported OCC length | PUSCH repetition Type A | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Component 5 candidate values: {(2), (2 and 4)} | Optional with capability signalling |

- Conclusions

In this contribution, we mainly share our views on the feature groups introduced for Rel-19 NR NTN enhancement. The proposals are summarized as follows:

Proposal 1: Introduce the following FG for the support of handling of collision Case 3 and collision Case 4 for half-duplex FDD operation for Rel-19 NR-NTN (e)RedCap UE:

| 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | [ 1. Support handling collision Case 3 ] [ 2. Support handling collision Case 4 ] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state | FFS: 26-1, 26-4, 28-1, 28-3, one of { 28-1, 48-1, 48-2 } | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | FFS N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FR1 FDD only Note: collision Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Adopt the following FGs for NR NTN uplink capacity/throughput enhancement.

| 65-3-1 | PUSCH with inter-slot OCC in NGSO scenarios | 1. Support inter-slot OCC for DFT-s-OFDM PUSCH transmissions with repetition Type A 2. Support RV cycling across OCC groups 3. Support collision handling between PUCCH and PUSCH with OCC 4. Support of keeping phase continuity and power consistency across one OCC group 5.Supported OCC length 6.Support of pre-compensation to keep phase rotation due to timing drift across one OCC group | PUSCH repetition Type A | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Component 5 candidate values: {(2), (2 and 4)} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65-3-2 | PUSCH with inter-slot OCC in GSO scenarios | 1. Support inter-slot OCC for DFT-s-OFDM PUSCH transmissions with repetition Type A 2. Support RV cycling across OCC groups 3. Support collision handling between PUCCH and PUSCH with OCC 4. Support of keeping phase continuity and power consistency across one OCC group 5.Supported OCC length | PUSCH repetition Type A | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Component 5 candidate values: {(2), (2 and 4)} | Optional with capability signalling |

- References

- Chairman's Notes RAN1#121, St Julian’s, Malta, May 19th – 23rd, 2025

- Chairman's Notes RAN1#122, Bengaluru, India, Aug 25th – 29th, 2025

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, December 9-12, 2024.

- R1-2506415	Session Notes of AI 9.8 	Ad-Hoc Chair (NTT DOCOMO, INC.)

- R1-2506732(R4-2512641), Reply LS on requirements for phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, Prague, CZ, 13th – 17th October, 2025