3GPP TSG-RAN WG1 Meeting # 122bis	R1-2507786

# Prague, Czech, Oct 13th – 17th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on UL and DL reciprocity modelling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reciprocity requires the same channel coefficients if transmitter and receiver switch their roles. Reciprocity between UL and DL channels is supported in 38.901 by swapping arrival and departure parameters. Needless to say, the same random parameters generated for DL channel should be reused to generate the reciprocal UL channel. However, using the same the 2x2 polarisation matrix for both UL and DL would result in different UL and DL channel coefficients. Channel coefficients in equation (7.5-22) can be simplified for a certain link in the “forward” direction (e.g., a TRP to UE link): Hforward=Frx,θθZOA,ϕAOAFrx,ϕθZOA,ϕAOATCPMforwardFtx,θθZOD,ϕAODFtx,ϕθZOD,ϕAOD If we introduce the following backward-direction variables, where arrival angles and departure angles are swapped, i.e., Ftx,θ=Frx,θ, Frx,θ=Ftx,θ, Ftx,ϕ=Frx,ϕ, Frx,ϕ=Ftx,ϕ, θZOD=θZOA, θZOA=θZOD, ϕAOD=ϕAOA and ϕAOA=ϕAOD, we get: Hbackward= HbackwardT=Frx,θθZOA, ϕAOAFrx,ϕθZOA, ϕAOATCPMbackwardFtx,θθZOD, ϕAODFtx,ϕθZOD, ϕAODT=Frx,θθZOA,ϕAOAFrx,ϕθZOA,ϕAOATCPMbackwardTFtx,θθZOD,ϕAODFtx,ϕθZOD,ϕAOD Hence, for channel reciprocity (Hforward=Hbackward) we require CPMforward=CPMbackwardT, i.e., CPMbackward=CPMforwardT, meaning the transpose of the 2x2 polarisation matrix in forward link should be used to generate the backward link. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the requirement that the 2x2 polarisation matrix of a DL path should be transposed to generate the corresponding reciprocal UL path. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the same the 2x2 polarisation matrix should be used for both UL and DL channels, and it would result in different UL and DL channel coefficients, i.e., reciprocity is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis: This CR has isolated impact as it corrects an erroneous reference. The CR has no direct impact on UE implementation or network implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.5	Fast fading model

The radio channel realizations are created using the parameters listed in Table 7.5-1. The channel realizations are obtained by a step-wise procedure illustrated in Figure 7.5-1 and described below. It has to be noted that the geometric description covers arrival angles from the last bounce scatterers and respectively departure angles to the first scatterers interacted from the transmitting side. The propagation between the first and the last interaction is not defined. Thus, this approach can model also multiple interactions with the scattering media. This indicates also that e.g., the delay of a multipath component cannot be determined by the geometry. In the following steps, downlink is assumed. For uplink, arrival and departure parameters have to be swapped, and the 2x2 polarisation matrix of a DL path has to be transposed to generate the corresponding reciprocal UL path.

< Unchanged parts are omitted >