3GPP TSG-RAN WG4 Meeting #116bis	R4-2513048

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	8.8

Source:	Samsung

Title:		Discussion on demodulation for 6GR

Document for:	Approval

# Introduction

In the RAN plenary RAN#108 meeting the SID [1] for Study on 6G Radio has been approved and further revised in RAN#109 meeting [2]. It’s well known 6G study represents an opportunity to revisit the foundation of radio system design. The outcome of the study will serve as the foundation of wireless communication for the next decade and beyond. In which, the details about Demodulation objective descriptions as below.

- Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

This is our first meeting on 6G Demodulation in RAN4. In this contribution, we provide our analysis and views on the potential issues for 6G Demodulation.

# Discussion

In 3GPP standards, demodulation performance testing defined by RAN4 is critical for ensuring reliable and efficient operation of a cellular system. Rather than specifying receiver algorithms, 3GPP defines minimum performance requirements to ensure the receiver performance and interoperability between different vendors’ equipment. Analyzing these metrics involves understanding the test assumptions defined in the standards and the practical challenges of interference/channel model/receiver design.

Given that this is a study item with ample time for in-depth analysis and discussion. We’d like to approach the topic systematically from the following aspects.

- Time planning for demodulation performance

- Framework and key assumptions influencing demodulation performance

- Evaluation methods for demodulation performance

- Demodulation specification

For the framework and key assumptions of demodulation and performance requirements, the influencing factors can be categorized into four main groups: physical layer assumptions, channel modeling, interference modeling and receiver algorithms. For the demodulation specification structures and drafting principles, further analysis and discussions are also needed.

## Time Planning for 6G Demodulation

During 5G NR stage, one study item lead by RAN4 on the test methods study for New Radio (which including 5G demodulation) held from RAN#75 on March 2017 to RAN#80 on June 2018, last for 15 months [3]. Later on, one work item lead by RAN1 for New Radio (NR) Access Technology started from RAN#75 on March 2017 to RAN#82 on December 2018, in which 5G demodulation last total 5 RAN4 meeting time [4]. After the first 5G NR demodulation performance related WI NR_newRAT-Perf, normally total 5 RAN4 meeting time was allocated for demodulation performance part for work items which have impacts on RAN4 demodulation performance part lead by other groups. However, based on the lessons learned from 5G, we realized that five RAN4 meeting time is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality. We recommend establishing a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Observation 1: Based on the lessons learned from 5G, we realized that five RAN4 meeting time which usually scheduled for RAN4 demodulation part is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality.

Proposal 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

## Framework and Key Assumptions for 6G Demodulation

2.2.1 Physical Layer Assumptions

The physical layer serves as the foundation for data transmission in wireless systems, and its design directly impact demodulation performance and requirements definition. According 6G SID [2] descriptions as below, the 6G physical layer structure about waveforms, modulation, frame structure, channel coding, channel bandwidth, data scheduling, HARQ operation, MIMO operation, duplexing, physical signals, physical channels should be studied by RAN1, and the study is ongoing now.

- Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

Accordingly, the basic physical layer related configurations for 6G demodulation requirements may need update. Recently, some initial agreements achieved for 6G waveform, 6G frame structure and 6G modulation schemes in RAN1#122 meeting.

For 6G waveform, CP-OFDM and DFT-s-OFDM waveforms are selected as the basis for 6G uplink, and CP-OFDM waveform is supported as the basis for 6G downlink. The agreements are listed as below. RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Proposal 2: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

For 6G frame structure, initial agreements about the subcarrier spacing (SCS) and cyclic prefix (CP) reached as below. Per our understanding, it’s not necessary to cover all the SCS options for demodulation performance requirements. It’s a reasonable and meaningful solution to define demodulation performance requirements with typical SCS configurations, e.g., 15kHz SCS for FDD, 30KHz SCS for TDD for sub 6GHz, 30kHz and/or 60kHz SCS for around 7GHz, 60kHz SCS for around 15GHz, and 120kHz SCS for 24.25GHz-52.6GHz, etc.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Agreement - 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP

Proposal 3: RAN4 6G Demodulation study could start with following SCS options

      - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

      - For around 7GHz, 30kHz, 60kHz

      - For around 15GHz, 60kHz

      - For between 24.25GHz - 52.6GHz, 120kHz

For 6G modulation schemes, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported for 6G DL, and QPSK, 16QAM, 64QAM, 256QAM are supported for 6G UL with both CP-OFDM and DFT-s-OFDM. Furthermore, pi/2 BPSK are supported for 6G UL with DFT-s-OFDM. Hence, it’s straightforward to study the demodulation performance with these modulation schemes.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Proposal 4: RAN4 6G Demodulation study should cover following modulation schemes at least

      - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

      - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

      - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

For other parameters related to 6G physical layer structure, which are still on discussion in RAN1. The best approach is to closely follow RAN1's discussion progress, and once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study.

Proposal 5: RAN4 closely follow RAN1's discussion progress on other parameters related to 6G physical layer structure. Once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study.

2.2.2 Channel Modelling

Channel model is one of the most fundamental aspects for demodulation performance requirements, and test cases of 3GPP demodulation performance are built around specific channel models.

Generally, the channel model with spatial property has been discussed at the beginning of 5G NR, and RAN1 are using geometric based SCM(E) defined in TR38.901 for system level simulation since this channel model is more realistic. TR38.901 CDL channel model are derived from SCM(E) with some parameters simplified/discarded, and TR38.901 TDL are further derived from TR38.901 CDL without spatial property. RAN4 decided to use simplified TDL model for 5G NR FR1 demodulation test in May 2018 [5] and simplified TDL model for 5G NR FR2 demodulation test in Aug 2018 [6] since majority companies think TDL is enough for 5G NR demodulation performance requirements and it is hard to align simulation results using TR38.901 CDL at that stage.

A New SID [7] on spatial channel model for demodulation performance requirements was approved in RAN#104 meeting, which concluded a reasonable level of alignment on rCDL-C1 (a channel model with CDL-C cluster truncation and randomness reduction based on TR38.901) in RAN4#116 meeting [8]. Accordingly, a new 5G WID targeting on normative work with SCM for SU-MIMO in FR1 approved in RAN#109 meeting [9]. At the same time, a new 5G SID targeting on study SCM for MU-MIMO approved in RAN#109 meeting [10].

Figure 2-1. Channel model evolutions for 5G RAN4

Consequently, a clarification on the channel model for 6G demodulation is needed. One potential solution may be only SCM channel model will be selected for 6G demodulation requirements since it has spatial characterics. Another soultion could be generally RAN4 TDL channel model will be used for majority features as no spatial characterics needed, and SCM channel model will be selected for some special features which need spatial characterics. Furthermore, for one feature, only one channel model (either TDL or CDL) should be selected.

Observation 2: RAN4 Demodulation requirements will be defined with both TDL and SCM (rCDL-C1) channel models for 5G from Rel-20.

Proposal 6: RAN4 clarify the channel model used for 6G demodulation requirements. Only SCM channel model, or both TDL and SCM channel model.

Proposal 7: RAN4 clarify the channel model used for one specific feature in 6G, prefer only one channel model selected (either TDL or SCM).

According the frequency range descriptions in 6G Study SID [1] and carrier frequency descriptions in TR38.914 [11] as following, it’s clearly that the frequency range for 6G is wide. Related channel model should be further studied to align with the wide range since current rCDL-C1 is for 3.5GHz Uma [8].

Frequency range descriptions in [1]

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Carrier frequency description in [11]

Table 4.1: Attributes for indoor hotspot Attributes; Values or assumptions. Carrier Frequency; Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz. Table 4.2: Attributes for dense urban Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700MHz Around 2GHz Around 4 GHz Around 7 GHz Around 15GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz. Table 4.3: Attributes for rural scenario Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1). Table 4.4: Attributes for urban macro Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz. Table 4.5: Attributes for Sub-Urban macro scenario Attributes; Values or assumptions. Carrier Frequency; Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz +Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz+Around 700 MHz.

Observation 3: The outcome rCDL-C1 of SCM channel model in RAN4 SCM study item is for 3.5GHz Uma only.

Observation 4: The frequency range for 6G is wide and multiple typical values for the carrier frequency proposed for different 6G scenarios.

Proposal 8: Either an unified channel model agnostic to different carrier frequencies or seperate channel models for different carrier frequencies should be defined for RAN4 6G Demodulation.

2.2.3 Interference Modelling

Modern dense urban cellular networks are fundamentally interference-limited. The primary performance bottleneck is no longer background noise but co-channel interference from other cells and users sharing the same time-frequency resources. This interference includes:

Inter-Cell Interference: Signals from neighbouring based stations.

Intra-Cell Interference: In multi-user MIMO (MU-MIMO) schemes, signals intended for other users within the same cell.

However, interference modelling is a major challenge in link level evaluation platform, since the interference from inter-cell or intra-cell inter-user may not easy to model like system level evaluation platform. 5G RAN4 demodulation defined requirements with neighbour-cell interference, with 1-2 interfering cells transmitting full-buffer data to mimic congested networks by different INRs; and 5G RAN4 demodulation defined requirements with intra-cell inter-user interference, with random QPSK or 16QAM co-scheduled UEs. With the high possibility of introducing SCM channel model in 6G, for 6G interference modelling, further discussion and analysis on the modelling of directions, INRs, modulation orders of interference(s) may be needed.

Proposal 9: For 6G Demodulation with interference modelling, further discussion and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

2.2.4 Receiver Algorithms

The performance of a communication system is fundamentally determined by the receiver’s ability to accurately recover the transmitted information from a corrupted and noisy signal. The receiver algorithm is the core intelligence that performs this task, and its design is absolutely critical to the system’s demodulation performance. A well performed receiver algorithm should be robustness to impairments including inter-symbol interference, frequency offsets, phase noise, etc., and could balance the computational complexity and performance

From 5G NR Rel-15, MMSE-IRC and R-ML receivers have been selected as optional receivers for 5G UE, the definition could be reached in TS38.306 as below. From 5G NR Rel-17, MMSE-IRC receiver has been considered as the mandatory receiver for 5G UE and default receiver for BS, which could provide superior performance in interference-limited conditions and optimal balance for practical implementation.

TS38.306 Chapter 5.2

## 5.2 UE receiver features Definitions for feature. SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].. MMSE-IRC (Minimum Mean Square Error - Interference Rejection Combining) receiver It is optional for UE to support MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference..

For 5G demodulation performance requirements, baseline receiver algorithm MMSE-IRC and advanced receiver R-ML are defined from UE side, and MMSE-IRC is defined from BS side.

For 6G demodulation performance, we suppose MMSE-IRC should be the baseline receiver algorithm.

Proposal 10: For 6G Demodulation, use MMSE-IRC as the baseline receiver algorithm for both UE and BS side.

## Evaluation Methods for 6G Demodulation

It is essential to define clear and reproducible metrics and methodologies for assessing 6G demodualtion performance. This may involve BLER vs SNR, SNR vs Throughput analysis, etc.

For 5G NR, the methodology for evaluating 5G demodulation performance is fundamentally sound and rational when the objective is to isolate and assess the pure physical layer capability of a receiver, independent of the higher-layer network algorithms. RAN4 defines the SNR at fractional of maximum throughput as the test metric for PDSCH demodulation performance requirements, the SNR could reach specified probability of missed downlink scheduing grant (Om-dsg) as the test metric for PUCCH demodulation performance requirements, the MCS values as the test metric for sustained downlink data rate requirements, the gama vlaue as the test metric for CSI reporting performance requirements. All these test are based on specified MCS value, fixed rank value, fixed channel bandwith and fixed subframe configuration, which without the impact from OLLA algorithms in network side.

By excluding network-side algorithms like OLLA, the evalution focuses squarely on the efficiency and robustness of fundamental receiver functions. OLLA is a network algorithm that dynamically adjusts the MCS values based on BLER feedback from UE to maintain a target reliability. Introducing OLLA would:

      - Introduce variability: The evaluation result would depend on the convergence speed and specific implementation of the OLLA algorithm, which is vendor-specific and not standardized.

      - Mask true PHY performance: A receiver with poor raw demodulation capability might appear better if the OLLA algorithm is overly conservative, while an excellent receiver might be penalized by an aggressive OLLA algorithm.

      - Complicate root-cause analysis: If performance is poor, it would be difficult to determine if the issue lies in the demodulator itself or in the network’s link adaptation strategy.

Therefore, for demodulation performance test, current method of using specified fading channels and FRCs(Fixed Reference Channels) stype, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration is the correct and reasonable approach. It provides an ”apples-to-apples” comparison of the demodulation signal processing chain.

Observation 5: For demodulation performance test, current method of using FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration is the correct and reasonable approach.

Proposal 11: For 6G Demodulation study, use 5G NR FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration as the starting point.

## 6G Demodulation Specification

2.4.1 Specification Structures

The implementation of an optimal specification structure is fundamentally necessary for effective communication and comprehension. It allows the readers to focus on the content’s substance rather than wasting cognitive energy deciphering a disorganized flow. A well-defined structure is not a constraint but a powerful tool that ensures precision, reduces ambiguity and maximizes the impact of the specification.

For 5G, the demodulation requirements specified for UE are described in TS38.101-4, the demodulation requirements specified for network are described in TS38.104; while the NTN related demodulation requirements from UE side are captured in TS38.101-5, and the demodulation requirements specified from network side are described in TS38.108.

For the demodulation specification structures, generally speaking, current specification structure of TS38.101-4 is clear, robust, and easy to expand. Hence it’s a good starting point for 6G demodulation specification structures.

Proposal 12: For 6G Demodulation specification structures, take TS38.101-4 as a starting point.

2.4.2 Specification Drafting Principles

About the specification drafting principles, based on lessons learned from 5G demodulation specification maintenance, below recommendations are proposed.

Firstly, for the test parameters, the descriptions should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Proposal 13: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Secondly, for the FRCs definition, current 5G specification define FRCs for demodulation performance testing using table-based approach. It’s no doubt that FRCs are explicitness and simplicity for implementation, which means the parameters for each FRC are directly readable from a table without any interpretation, and test equipment vendors can directly hard-code the values from the specification. However, this approach addresses high maintenance overhead, propagation of errors, lack of scalability and opacity of underlying principles, i.e., the tables are massive and monolithic. A single error in a parameter requires a formal CR, review, and update the tables, which is an inefficient process; Errors can be copy-pasted across similar FRCs, making them difficult to root out; As 6G introduces more complex scenarios, the number of required FRCs will explode; The tables often obscure the logical rules governing the relationship between different parameters. This makes it harder for engineers to understand the intent behind a specific FRC. Therefore, for 6G demodulation, we would like to propose a formula-based or pseudo-code-based definition instead of listing every parameter combination. For this new approach, demodulation specification only needs to describe the calculation formula clearly and list key inputs for separate test cases, such as bandwidth, SCS, code rate, modulation order/MCS value, MIMO rank, etc.

Proposal 14: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

# Conclusion

In this contribution, we provide analysis and views on potential issues for 6G Demodulation.

The proposals and observations could be summarized as:

Observation 1: Based on the lessons learned from 5G, we realized that five RAN4 meeting time which usually scheduled for RAN4 demodulation part is not sufficient for thorough technical discussions. This often led to rushed conclusions and compromised quality.

Proposal 1: For RAN4 6G Demodulation, RAN4 establish a more realistic and structured timeline to ensure adequate depth in technical deliberation and a well-paced progression toward completion.

Proposal 2: RAN4 6G Demodulation could start with CP-OFDM and DFT-s-OFDM waveforms for 6G uplink demodulation study, and CP-OFDM waveform for 6G downlink demodulation study.

Proposal 3: RAN4 6G Demodulation study could start with following SCS options

      - For sub 6GHz, 15kHz SCS for FDD, 30KHz SCS for TDD

      - For around 7GHz, 30kHz, 60kHz

      - For around 15GHz, 60kHz

      - For between 24.25GHz - 52.6GHz, 120kHz

Proposal 4: RAN4 6G Demodulation study should cover following modulation schemes at least

      - For downlink, QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

      - For uplink with CP-OFDM waveform, QPSK, 16QAM, 64QAM, 256QAM

      - For uplink with DFT-s-OFDM waveform, pi/2 BPSK, QPSK, 16QAM, 64QAM, 256QAM

Proposal 5: RAN4 closely follow RAN1's discussion progress on other parameters related to 6G physical layer structure. Once relevant conclusions are available, RAN4 could consider adopting them for RAN4 6G Demodulation study.

Observation 2: RAN4 Demodulation requirements will be defined with both TDL and SCM (rCDL-C1) channel models for 5G from Rel-20.

Proposal 6: RAN4 clarify the channel model used for 6G demodulation requirements. Only SCM channel model, or both TDL and SCM channel model.

Proposal 7: RAN4 clarify the channel model used for one specific feature in 6G, prefer only one channel model selected (either TDL or SCM).

Observation 3: The outcome rCDL-C1 of SCM channel model in RAN4 SCM study item is for 3.5GHz Uma only.

Observation 4: The frequency range for 6G is wide and multiple typical values for the carrier frequency proposed for different 6G scenarios.

Proposal 8: Either an unified channel model agnostic to different carrier frequencies or seperate channel models for different carrier frequencies should be defined for RAN4 6G Demodulation.

Proposal 9: For 6G Demodulation with interference modelling, further discussion and analysis on the modelling of directions, INRs, modulation orders of interference(s), number of layers from interference(s) are needed.

Proposal 10: For 6G Demodulation, use MMSE-IRC as the baseline receiver algorithm for both UE and BS side.

Observation 5: For demodulation performance test, current method of using FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration is the correct and reasonable approach.

Proposal 11: For 6G Demodulation study, use 5G NR FRC style, i.e., specified MCS value, fixed rank value, fixed channel bandwidth and fixed subframe configuration as the starting point.

Proposal 12: For 6G Demodulation specification structures, take TS38.101-4 as a starting point.

Proposal 13: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

Proposal 14: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

# Reference

[1] RP-251881, New SID: Study on 6G Radio, RAN#108, NTT DOCOMO

[2] RP-252912, Revised SID: Study on 6G Radio, RAN#109, NTT DOCOMO, CMCC, AT&T, Vodafone

[3] RP-170824, New SID on Study on test methods for New Radio, RAN#75, Intel Corporation, CATR, Qualcomm Incorporated, MediaTek

[4] RP-170855, New WID on New Radio Access Technology, RAN#75, NTT DOCOMO, INC.

[5] R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4 #87, Intel Corporation

[6] R4-1812001, RAN4#88 Meeting report, RAN4#88

[7] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[8] R4-2509114, 3GPP TR38.753 V1.1.0, RAN4#116

[9] RP-252957, WID: NR demodulation performance: Phase 6, RAN#109, Apple

[10] RP-252956, New SID: Study on spatial channel model for demodulation performance requirements for NR: Phase 2, RAN#109, Apple

[11] RP-252882, 3GPP TR38.914 V0.2.0, RAN#109