3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513049

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on AI framework and use cases for 6GR

Agenda item:			8.9

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [1]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we would like to provide our viewpoints on RAN4-related AI/ML issues, including RAN4 driven use cases (only except the testability and OTA related AI) and RAN4-relevant issues of AI/ML framework.

# 2. Scope and timeline for RAN4 6G AI/ML study

## 2.1 RAN4 6G AI/ML study scope

By reviewing the existing 3GPP works on RAN1-led SI/WI (AI/ML for air interface) and RAN2-led SI/WI (AI/ML for mobility), as illustrated in the above figure, the new use cases and AI/ML framework (including LCM procedure and data collection and data management) are identified as key objectives for AI/ML in 6G RAN4 study. Particularly, the RAN4 working group is tasked to (1) identify RAN4-relevant AI/ML use cases and (2) study AI/ML framework in which LCM procedures and corresponding requirements are relevant to RAN4.

- AI/ML Use Cases: AI/ML can address both existing and new use cases, offering improvements from different perspectives. For example, Rel-18/19 RAN1-led study identified the use case of enhancing MIMO precoding by compressing channel information, thereby improving spectral efficiency, while Rel-19 RAN2-led study identified the AI-enabled mobility management, in which AI/ML can optimize handover procedures, reducing latency and ensuring seamless connectivity. From RAN4 perspective, the use cases can be considered including AI/ML-enabled advanced receiver, AI/ML-based compensation for practical TX/RX RF chain non-linearity, AI/ML-enabled SRS, AI/ML-enabled PRACH receiver, AI/ML-enabled RRM, etc. The criteria for RAN4-driven use case should be clarified. Furthermore, as provided in WID, it is expected that the coordinated discussions with related design areas are essential to ensure that AI/ML solutions are aligned with broader network objectives.

- AI/ML Framework (LCM procedure relevant): The AI/ML framework must be extendable and tailored to the identified use cases in 6G. Key aspects of advanced AI/ML framework should at least include model transfer and other inter-vendor interoperability, which has been studied in previous releases for one/two-sided models. Furthermore, LCM procedures for AI/ML management should also be considered, which shall include configuration, (de-)activation, fallback mode, training, data collection, verification, testing, and validation to ensure reliable and scalable AI/ML operations. For instance, AI/ML models can be dynamically activated or deactivated based on network conditions, ensuring optimal performance without unnecessary resource consumption. From RAN4 perspective, the relevant LCM procedure and corresponding requirements shall be studied.

- AI/ML Framework (RAN4 specific issues): While other groups harvest use-cases and the high-level AI/ML framework, RAN4 must ground them in implementable, testable specifications. After Rel-18/19 discussion of NR AI/ML for air interface, a gap still unaddressed is the feasibility and necessity of post-deployment verification: a standardised mechanism to allow AI/ML model update and re-tuning with reasonable and limited effort to guarantee performance conformance. Limited study in Rel-18/19 and the constraints of legacy 5G NR have left this open. Furthermore, based on previous releases’ experience, AI/ML implementation related parameters and issues, including AI/ML model complexity and power consumption, UE and network AI/ML capability, are also interesting topics. For example, Rel-19 AI/ML study involves identifying “low-complexity test decoder” compared to CNN-based baseline, while the numerical metrics for AI/ML complexity and the corresponding low/high complexity compared to UE/NW capability needs to be further studied.

Based on the above analysis, we would like to provide the following proposal for detailed RAN4 scope planned for AI/ML in 6G study:

- In 6G AI/ML study, the detailed RAN4 scope shall include:

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

  - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility.

  - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures: RAN4 shall study relevant RAN4 requirements for LCM procedures.

  - RAN4 specific issues to guarantee AI/ML functionalities is implementable and testable, including post-deployment, AI/ML model complexity and power consumption, UE and network AI/ML capability, etc.

## 2.2 RAN4 6G AI/ML study timeline

Based on 6G SID [1], the period of 21 months is planned for RAN4 6G study, in which RAN4 chair has suggested the scope of Q4’25 and Q1’26 should be RAN4 driven use case [2, R4-2511652].

| 6G Topics | Scope | Scope in Q4'25 and Q1'26 |
| --- | --- | --- |
| AI | RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI | RAN4 driven use cases |

By noting that the assigned TU number to 6G study is increased from 7 to 20 after Q2’26, to complete the expected study on time, we suggest that two stages planning shall be considered.

For Stage 1, RAN4 is going to take the first 3 meetings (i.e., in Q4’25 and Q1’26) to (1) clarify the study scope for RAN4 AI/ML framework and (2) select the top-prioritized AI/ML use cases with the detailed plan for further Stage-2 evaluation. Due to limited time in Stage 1, there seems no sufficient time for companies to carry out comprehensive performance evaluations with the aligned parameters and restriction. Hence, RAN4 shall consider to select the use cases based on their popularity and potentiality from RAN4 perspective. The coordinated discussions with other working groups are essential to avoid overlapping scope and wasted efforts.

For Stage 2, RAN4 shall focus on AI/ML framework including RAN4 aspects for LCM procedures and other RAN4 specific issues. For the use cases, RAN4 shall throughout study those use cases selected in Stage 1 with the detailed evaluation planning and possibly carry out down selection on candidate solutions at the end of Rel-20. Furthermore, RAN4 should also plan the evaluate efforts for other WG-driven use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

Hence, we would like to suggest the following timeline.

- Suggested RAN4 timeline for Rel-20 6G AI/ML study

Stage 1: Q4'25 and Q1'26

- To clarify study scope for RAN4 AI/ML framework

- To select the top-prioritized AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

Stage 2: Q2’26 to Q2’27

- To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

- To study comprehensively the use case(s) selected in Stage 1

- To study other WG-driven use cases with RAN4 analysis and evaluation

# 3. RAN4 AI/ML framework

## 3.1 AI/ML Framework (LCM procedure relevant)

Based on Rel-18/19 relevant study and work items, the NR AI/ML framework for the air-interface is a compromise between practicality and forward compatibility. The main aspects of the NR AI/ML framework can be summarized with the following points in Table 1:

Table 1 AI/ML framework review of 5G-Adv AI/ML for air interface

|  | Rel-18/19 Discussion Points | Experience learnt for 6G |
| --- | --- | --- |
| AI/ML Framework and Life Cycle Management (LCM) | Data Collection (UE-sided): - Enabled through UE measurement - NW provide configuration and reference signal transmissions - Data collection framework was not complete in Rel-18/19. Data Collection (NW-sided): - Extension of the MDT framework. | Data collection work is mainly RAN2/SA related. |
|  | Applicability Report (UE-sided model) - UE reports its ML capabilities via capability reporting. - NW configurations, based on the reported capabilities, are used to enable inference operations. - UE may additionally report applicability/inapplicability of AI/ML functionalities in response to the NW’s indication. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model management and model-ID-LCM (UE-sided model) - Model management is up to UE implementation. - NW controls the ML usage at the UE via functionality-based LCM. - Functionality-based LCM indirectly supports model selection, activation, deactivation, switching, and fallback Model management and model-ID-LCM (NW-sided model) is up to the NW implementation. | The necessity of model management depends on RAN4 discussion. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Protection for NW-side implementation (UE-sided model) - NR framework leverages the concept of an Associated ID - Associated ID is to implicitly indicate NW-side additional conditions that may impact performance of a UE-side model. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Performance monitoring (UE-side model) - The monitoring outcome may assist NW decisions on configurations/functionalities switching including fallback to non-AI/ML approaches. - Two performance monitoring approaches: UE-assisted performance monitoring where performance monitoring metric is calculated at the UE and NW-side metric calculation considering ground-truth measurements report from UE as input. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model training: - In the NR offline model training is assumed. Thus, there was no explicit specification support for on-device (online) model training and fine-tuning. | Need to reconsider by taking the solutions for RAN4 use case into account Model retuning and/or online training should also be considered. |
|  | Inter-vendor training collaboration (two-sided model): - Rel-19 identified three inter-vendor training collaboration options, namely: i) fully specified reference model ii) reference encoder parameters sharing-based approach iii.) encoder-related training dataset sharing-based approach. | No need to consider this if only one-sided model is considered for RAN4 use case. If two-sided model is used, Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Model Transfer: - In the NR model transfer across vendors, e.g., from NW to the UE, has not been supported. - It is assumed that the UE-part of the models are trained offline by UE-side vendors and delivered to the UEs in a transparent manner. | Work is mainly RAN2/SA related. Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Processing unit and timeline: - The 5G NR acknowledged AI/ML processing may be carried out by a separate hardware from non-AI/ML processes. - AI/ML processing unit (APU) is introduced to quantify the number of concurrently processed AI/ML-based CSI reports. - A separate CSI processing timeline is considered for AI/ML-based CSI reports. | Relevant experience can be considered and leveraged in RAN4 6G AI/ML study |
|  | Testing and Performance Requirements - Test cases, core and performance requirements, and generalization requirements are use-case specific - Requirements for LCM - Post-deployment testing (LCM based is prioritized). - Model level testing is not supported. | RAN4 centric work in 5G-A discussion. |

Based on Rel-18/19 experience, the LCM-relevant discussion went slowly, and it would be complicated and inefficiency to design LCM procedures for each studied use case. Therefore, to align with existing RAN1-led use cases (AI-BM, AI-CSI prediction and AI-positioning) and the new 6G use cases driven by different working groups, the unified LCM framework shall still be desired and targeted for.

- The same and unified LCM framework shall be targeted for RAN4-driven use cases and use cases driven by other working groups.

By examining the Rel-18/19 outcome for LCM framework, RAN4 shall base its LCM study for AI/ML on the NR framework in TR 38.843, covering its terminology, UE/network data collection, applicability reporting, implicit-condition IDs, performance monitoring, APU timelines, and corresponding test/performance requirements.

- For RAN4 study of LCM procedure for AI/ML framework, take 5G NR AI/ML framework as a starting point for RAN4-driven use cases, by at least considering

-	Terminologies in TR 38.843

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

From RAN2 perspective, the following LCM signaling procedure is introduced for RAN1-led work item. In Rel-19 study item for AI-mobility, RAN2 agreed that for AI mobility, the general LCM framework for beam management can be the baseline (where applicable) for AI mobility. It should be noted that fallback to non-AI is not yet introduced.

Figure 1 LCM signalling procedure considered in 5G NR AI/ML work items

However, AI/ML model-based prediction/inference is power thirsty, it is reasonable for a UE to fall back to legacy non-AI model when all inference configuration become inapplicable or it enters low-power status. Meanwhile, in 6G, new AI/ML capabilities/applicable scenarios/use cases will be studied, how/when to trigger interference is also needed to be studied use case by use case.

## 3.2 AI/ML Framework (other RAN4 specific issues)

For model complexity, at least the following concepts are involved and used for Rel-18/19 studies:

-	Computational complexity of model inference: TOPs, FLOPs, MACs

-	there may be a disconnect between the actual complexity and the complexity evaluated as captured in clause 6 using these KPIs due to the platform-dependency and implementation (hardware and software) optimization solutions

-	Computational complexity for pre- and post-processing

-	Model complexity: e.g., the number of parameters and/or size (e.g., Mbyte)

-	Complexity shall be reported in terms of "number of real-value model parameters" and "number of real-value operations" regardless of underlying model arithmetic

AI/ML processing capability is considered as one key issue during Rel-18/19 discussion. For instance, in Rel-19 CSI compression study, the AI/ML model with reduced complexity are evaluated by considering different UE implementation. In TR38.843, it is concluded that

The practical processing capability and implementation complexity for device under test should be assumed when specifying RAN4 requirements. - The UE capability may be needed to handle different complexity for one side and two-side models. - The complexity of UE should also be studied when making assumption on gNB side model, and vice versa.

However, the concept of AI/ML processing capability has not been throughout studied, especially lacking of the study by considering practical UE/NW implementation. Furthermore, different AI-based receivers are expected to be studied for RAN-driven use cases, in which the neural networks could be integrated into existing receiver signal processing chain in different levels, e.g., by replacing one conventional method or jointly optimize multiple blocks. This will also highlight the importance to analyze and quantify the level of AI/ML processing capability.

Another aspect is energy efficiency metrics for AI operation, which could potentially include AI/ML model inference power consumption and other relevant AI/ML procedure related power consumption. Among them, AI/ML model inference related execution power scales directly with inference complexity and how often it is invoked; by counting the total MACs inside any observation window we can turn operation count into milli-watts. Study on power consumption issues are also important aspects to push 3GPP defined AI/ML framework into practical implementation.

- For RAN4 specific issues in AI/ML framework, the following issue can be further studied:

- The AI/ML model complexity

- UE and network AI/ML capability and capability-level categorization

- Power consumption model for AI/ML inference and other AI/ML relevant procedures.

# 4. RAN4 driven AI/ML use cases

## 4.1 Principle to select RAN4-driven AI/ML use cases

As elaborated in the above section, the period of Stage 1 (Q4’25 – Q1’26, three meetings) will be used to clarify the RAN4 AI/ML study boundary and to select the use-cases that promise the benefits from RF/base-band angle; with no time for full-scale simulations, selection will be driven by evident popularity and RAN4 relevance, while tight liaison with other WGs prevents double work.

The criteria for RAN4-driven use case should be clarified. Different from other working groups, RAN4 shall consider to select the use cases from RAN4 perspective. Those AI/ML use cases that fall within RAN4 core responsibility – namely, RF and baseband performance, by either solving the key RAN4 pain points or related to AI-receiver with limited RAN1/2 impacts. Obviously, the coordinated discussions with other working groups are required to clarify the boundary and to avoid overlapping scope between different groups.

- A use case can be selected as RAN4-driven AI/ML, if at least one of the following conditions are met:

(1) The use case is to solve key pain point which is better understood in RAN4.

(2) The use case can be classified as AI-receiver with no or limited RAN1/2 impacts, and major specification impacts are related to RAN4.

Note: Collaboration/interaction between UE and NW shall be minimized in RAN4-driven use case.

- Coordinated discussions with other working groups are required to avoid overlapping scope between different working groups.

## 4.2 General consideration for RAN4-driven AI/ML use cases

The counterpart of RAN1 works on 6G AI/ML has already begun in the August meeting, and the following agreement is reached for use case identification/categorization, which will also serve for further use case selection:

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

RAN1 studies should focus on identifying promising new AI/ML-enabled use cases related to the physical layer for both UEs and the network, and some of relevant aspects to be studied should also be considered in RAN4 study.

- At least the following RAN1 agreed aspects to be studied and reported for each proposed (sub-)use case should be considered for RAN4 study for AI/ML use cases identification/categorization:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

  - no collaboration/interaction

  - UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

In addition to the above aspects in Proposal 8 which is same as RAN1, the particular RAN4 specific issues like implementation feasibility, including computational complexity, power consumption etc. shall be considered.

- Additionally, RAN4 study for AI/ML use case identification/categorization shall consider implementation feasibility, including computational complexity, power consumption etc.

## 4.3 Use Case-1: AI/ML-enabled non-linearity compensation

4.3.1 Motivation and use case description

In general, the ever-growing data rate demands in mobile communications motivate the exploitation of higher order modulations (HOM) such as 256QAM and 1024QAM. At a given transmitter-receiver distance, the HOM requires larger transmission power than QPSK, 16QAM, and 64QAM due to its reduced distance between constellation points, which results in smaller coverage for HOM. A natural way to improve the HOM coverage would be to increase the transmission power level. However, the possible increase of transmission power level is limited by the maximum power that can be provided by the power amplifier (PA). To maximize the HOM coverage, it is expected that the PA output power should be as close to its saturation level as possible. This would, however, result in severe performance loss due to the nonlinear distortion caused by typical characteristics of radio frequency (RF) components such as PA. It is clear that such non-linear distortion needs to be compensated at either transmit side (TX) with pre-distortion mechanism or receive side (RX) with equalization to ensure block error rate (BLER) or coverage requirements for HOM.

To leverage HOM, it is essential to develop a more effective scheme to compensate the nonlinear distortion. One alternative is to embrace artificial intelligence (AI), which has been recognized as a promising tool to deal with complicated issues in many fields.

AI-enabled non-linearity compensation (AI-NC) at RX side is proposed, which can support fast online training without additional overhead.  The receiver block diagram of AI-NC is shown in Figure 2 Rx block diagram of Figure 2.

Figure 2 Rx block diagram of AI/ML-enabled non-linearity compensation

Figure 2 is a schematic of AI-NC receiver structure with NRx-antenna. The received signal of each RX antenna is transformed into frequency domain by using FFT. Then conventional CE is performed based on local frequency domain demodulation reference symbol (DMRS) to estimate channel coefficients per RX antenna. By using frequency domain RX data and estimated channel coefficients of each RX antenna, conventional MIMO equalization is performed to distinguish TX data streams as well as remove the impact caused by multipath channel.

After conventional frequency domain MIMO equalization, neural network is applied to compensate the non-linear distortion. As non-linear distortion of PA is generated and introduced in time domain, using time-domain neural NW is a better way compared with frequency domain processing. Meanwhile, our investigation and analysis show that if frequency domain processing is applied, training overhead and computational complexity will be much larger compared with a time-domain neural NW with similar performance. As a result, after conventional frequency domain CE and MIMO equalization, IFFT is applied per data stream for time domain processing of neural NW. In addition, one high order term construction is applied to derive the 3rd term (e.g., the so-called high order term) of the time domain signal samples. Per our analysis, such term can further improve the performance of using ESN.

Then neural network is utilized to do the online training based on time domain DMRS samples and inference based on RX data samples after training to compensate non-linear distortion. After that, the resulted signal is transformed into frequency domain again and subsequent procedures, including de-modulation and decoding, are performed to retrieve the transmitted bits as convention. To summarize the model structure and training related information in Table 2.

Table 2 Model structure and training related information for AI/ML-enabled non-linearity compensation

| Model location | Network side |
| --- | --- |
| Model input | Training: estimated RS signal time domain sample with 1st, 3rd terms Inference: estimated data signal time domain sample with 1st, 3rd terms |
| Model output | Training Label: original/known RS signal Inference Output: data signal with non-linear distortion compensated |
| Training types | Online training |
| Collaboration between UE and NW | no collaboration/interaction |

4.3.2 Preliminary evaluation results

Link level evaluations have been conducted to verify the ability of proposed AI-NC scheme. The performance is evaluated by BLER with different number of layers/users, different numbers of RX antennas, and multiple EVM values. For current results, the benchmark is the normal reception schemes without distortion compensation.

The evaluation assumptions are listed in Table 3. Note that each TX layer corresponds to 1 TX antenna. Both one TX layer (1T) and two TX layers (2T) are evaluated. For 2T case, frequency domain multiplexing (FDM) is considered in the evaluation, which means that 2 layers occupy two sets of adjacent frequency resources and out-of-band non-linear interference is concerned and handled by AI-NC. It should be noted that one layer can be regarded as one user for evaluating multi-user case equivalently. NR frame structure with 10 data symbols and 2 DMRS symbols per slot is applied in LLS evaluation. For conventional scheme, 2 DMRS symbols are used for linear CE and equalization. For proposed AI-NC scheme, after linear CE and EQ, the first DMRS symbol is reused for neuron network training.

Table 3 Configuration of link level simulation

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| # of TX layers/users | 1/2 |
| # of RX antennas | 4/8/64 |
| Waveform | NR based |
| # of PRB | 50 |
| # of symbol per slot | 12 |
| # of DMRS per slot | 2 |
| # of DMRS re-used for training | 1 |
| DMRS subcarrier position | Odd index (0 for even index) |
| FFT size | 2048 |
| CP length | 144 |
| Modulation | 256 / 1024/ 4096 QAM |
| Coding rate | LDPC 0.9395 |
| Transport block size | 45120 for 1Tx/256 QAM 56392 for 1Tx/1024 QAM 67680 for Tx/4096 QAM 90200 for 2Tx/256 QAM |
| Channel model | TDL-A 30ns |
| Subcarrier spacing | 15 kHz |
| Evaluation metric | BLER |
| Benchmark | Conventional receiver (without non-linearity compensation) |

The evaluation assumptions are listed in Table 3. Note that each TX layer corresponds to 1 TX antenna. Both one TX layer (1T) and two TX layers (2T) are evaluated. For 2T case, frequency domain multiplexing (FDM) is considered in the evaluation, which means that 2 layers occupy two sets of adjacent frequency resources and out-of-band non-linear interference is concerned and handled by AI-NC. It should be noted that one layer can be regarded as one user for evaluating multi-user case equivalently. NR frame structure with 10 data symbols and 2 DMRS symbols per slot is applied in LLS evaluation. For conventional scheme, 2 DMRS symbols are used for linear CE and equalization. For proposed AI-NC scheme, after linear CE and EQ, the first DMRS symbol is reused for neuron network training.

Figure 3(a) shows the results of 256QAM with 4 and 64 RX antennas. Several EVM values are considered to observe the performance of AI-NC compared with conventional linear scheme. It should be noted that 3.5% is the minimum EVM requirement for 256QAM and 2.5% is for 1024QAM specified by RAN4 in 5G NR. We relax those two values to verify the capability of AI-NC to compensate non-linearity with larger EVM values. For 1T case, the performance is good enough even without high-order term construction. As a result, high-order term construction is turned off. From those results, it can be seen that with 4 RX antennas, the performance gain of proposed AI-NC over conventional scheme gets larger with the increase of EVM, from less than 1dB with 2% EVM to around 2dB with 3.5% EVM at BLER of 10%. When EVM increases to be 6% and 8%, proposed scheme can still achieve BLER performance of 10% at SNR of 24.4 dB and 27.2 dB, respectively. However, conventional scheme fails to deal with such serious non-linear distortion with EVM of 6% and 8%, so the corresponding curves are not shown in the figure. If the number of RX antennas increases to 64, more obvious gain can be observed. Even with small EVM value of 2%, the performance gap between the proposed and conventional scheme at BLER of 10% can reach 3.6 dB.

Figure 3(b) shows evaluation results of 1024QAM with 4 and 64 RX antennas. For 2.5% EVM with 4 RX antennas, conventional scheme fails to decode within the evaluated SNR range, while the performance of proposed scheme can meet the typical 10% BLER requirement within a reasonable SNR range. Even with 1.5% EVM with 4 RX antennas, more than 1dB gain can be observed compared with convention scheme. If the number of RX antennas increases to 64, conventional scheme fails to decode 1024QAM within the evaluated low SNR range but the proposed scheme can support 1024QAM reaching 10% BLER performance.

Figure 3(c) shows evaluation results of 4096QAM with 4 RX antennas. Note that current wireless communication systems do not support such high modulation order. The results show that with proposed AI-NC, the performance gain over conventional scheme is around 1.8 dB with 1% EVM. Meanwhile, if EVM increases to 1.5%, conventional scheme failed to decode within evaluated SNR range while AI-NC still shows acceptable performance.

- AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values.

Figure 3 Evaluation results for (a)256QAM, (b)1204 QAM, and (c)4096QAM

4.3.3 Study focus in RAN4

The following Note, including a check point, was captured in Rel-20 WID discussion (New WID: UE RF enhancements for NR FR1, Phase 5) [2] during RANP#109 as the guidance for RAN4 work. Motivated by the potential commonality of non-linearity modeling of transmission signals and feasibility study steps, which are likely to be agnostic of 5G-A and 6G solutions, the group decided to combine the 5G and 6G discussion into one track to avoid the duplicated efforts and inconsistence. Specifically, it was emphasized that the 5G-A solution should be non-AI based and should not be any impact to RAN1. In other words, for 6G solution, new waveforms and/or AI-based solution can be considered.

In our view, the spirit of this study is to increase the possibility of low-end PA transmitting higher power for higher modulation schemes, with advanced compensation solutions implemented at BS side. Therefore, we believe the non-linearity model(s) selected to evaluate BS compensation capabilities and UE MPR reduction limit should also account other RFIC impairments including at least I/Q imbalance, other factors such as fractional timing error are not precluded. In addition, to simplify the discussion considering the time constrains, it is advisable to prioritize FR1 single CC operation, for PC3 with 1Tx and PC2 with both 1Tx and 2Tx. Further, selecting CP-OFDM as the waveform is recommended which is suitable for both Layer-1 and Layer-2.

- The goal of the work in 6G is to enable low-end PA and extreme high modulation to transmit higher power, leveraging the capabilities of advanced BS receiver.

4.3.3.1 General assumption for both AI and Non-AI solutions

In accordance with the chair's guidance, discussions under the UE RF agenda should focus solely on non-AI-related UE RF topics, while AI-related matters are to be addressed in the AI agenda. However, it is our understanding that certain aspects, such as the non-linearity model(s) selected for evaluation purpose and steps to confirm feasibility, can be determined in UE RF session. These are agnostic to AI/Non-AI solutions and can help best organize the study. Additionally, by assuming the same waveform and non-linearity model(s), results for both AI and non-AI solutions can be effectively compared. This approach ensures a more organized and efficient study process.

At the initial stage of 6G discussion, while the discussion on the new waveform and new PA modeling for 6G are going, it is advisable to prioritize the study of AI and non-AI solutions based on 5G waveforms and 5G PA models. Once 6G waveforms and 6G PA models are agreed, they can be incorporated into the study.

- Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions.

To evaluate the effectiveness of proposed techniques related to nonlinearity, PA models in TR 38.803 can be considered for evaluation purpose, e.g., Rapp model and generalized memory-based polynomial (GMP) model, in conjunction with modelling of other non-ideal factors including I/Q imbalance, fractional timing error and so on. Our detailed proposals are provided in our accompanying Tdoc [3].

4.3.3.2 Working steps for feasibility study for both AI and Non-AI solutions

To efficient the complex discussion, it is essential to establish clear criteria for confirming feasibility. We propose to agree on the following steps as below. As detailed below, the feasibility study involves extensive demodulation work.

- Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, and demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity TX model: SNR-a

    - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b

    - Case 3: Conventional receiver without any non-linearity TX model: SNR-c

    - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-d and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment

    - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-d derived based on Case 4 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation:

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting (i.e., MPR reduction) for different power class

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

## 4.4 Use Case-2: AI/ML-enabled channel estimation

4.4.1 Motivation and use case description

In 5G NR system, the structure of DMRS was designed flexibly pending on different use cases in practical scenario. In frequency domain and Time domain, two different DMRS RE mapping types based on Comb and orthogonal complementary code (OCC) structure are designed to support multi-layer transmission and multi-user multiplexing, i.e., type 1 and type 2, up to 12 ports can be multiplexed in existing NR Rel-15 design. And up to 24 ports can be multiplexed in NR Rel-18 design. In time domain, flexible number of DM-RS symbol can be configured pending on the scheduling data symbol length, up to 4 DMRS symbols can be supports. Therefore, up to total 28% DMRS overhead is expected in existing NR Rel-15 design as illustrated in Figure 4.

Figure 4 DMRS patten and overhead for uplink transmission

- Up to 28% DMRS overhead is expected in existing NR Rel-15 design.

For 6G, it is expected to support higher number of layers via enhanced RS (e.g., 64 DMRS ports) for both uplink and downlink for multiple layers transmission or multi-user multiplexing. It will result in the large overhead of RS if the DM-RS pattern design is based on existing DM-RS design structure.  To address the issue of high overhead of DM-RS, it is straightforward to consider the lower overhead DM-RS design in 6GR. To maintain the channel estimation accuracy similar as high density RS patten, the operation with advanced channel estimation or receiver implementation is required for 6G, especially with AI/ML technologies.

In general, assisting by AI/ML technologies, we expect that the following benefits can be achieved

- Throughput performance improvement directly by improving channel estimation accuracy performance compared with legacy receiver.

- Throughput performance improvement by reducing the DMRS overhead with maintaining acceptable channel estimation accuracy

There are two major directions proposed by companies for 6G SI AI use cases. One is the orthogonal manner as legacy with sparse orthogonal RS design, another is superimposed pilot, called SIP, which allows the DMRS and data/control channel are superimpose at the same REs.  Regarding the DMRS pattern design, from RAN1 perspective, they will study AI-based DMRS enhancement for 6GR, including the potential design of orthogonal DMRS ports with the extension of FD-OCC length, CDM group number, TD-OCC length with double symbol structure, as well as the flexible DMRS symbol configuration.

From RAN4 perspective, the new demodulation requirements and testing should be considered based on DMRS configuration based on UE and network AI/ML capability.

In current RAN4 demodulation requirement, pending on different features or targeting scenarios, the DMRS configuration can be configured flexible to meet the test purpose.  The following typical DMRS configurations were considered when defining PDSCH and PUSCH requirements

- Rel-15 NR type 1 with 2 symbols structure up to 4 ports configured for normal PDSCH and PUSCH requirement

- Rel-18 NR enhanced type 1 with 2 symbols structure up to 8 ports configured for normal PDSCH and PUSCH requirement

- Rel-15 NR type 1 with 3 symbols structure configuration for PDSCH and PUSCH requirement in high-speed scenario

- Re1-15 NR type 1 with 2 symbols structure configuration for PDSCH and PUSCH requirement with repetition transmission and joint channel estimation (JCE)

Table 4 MRS patten and overhead for uplink transmission

| FR | Velocity | Doppler | DMRS config |
| --- | --- | --- | --- |
| FR1 ( 3.5GHz) | 500km/h | 2334Hz | 1+1+1 |
| FR2 (28GHz) | 350km/h | 19444Hz | 1+1+1 |

Figure 5 DMRS patten and overhead for uplink transmission with repetition

For normal PDSCH and PUSCH demodulation, under the existing NR type 1 with 2 symbols structure, the performance can be improved with AI/ML-enabled advanced receiver.

For the high-speed train scenario, we think the throughput performance improvement by reducing the DMRS overhead with maintaining acceptable channel estimation accuracy with AI/ML-enabled advanced receiver compared with legacy receiver with 3 DMRS symbol pattern.

For the repetition transmission, especially for joint channel estimation, to enable the joint channel estimation, DMRS bundling across PUSCH transmission was considered, this objective has impact on both UE and Network side. UE behaviour needs to be defined if the phase coherency of PUSCH repetition, and new channel estimator needs to be implemented at receiver to process DMRS across multiple repetitions. RAN4 have discussed and specified the UE RF core requirement to keep power consistency and phase continuity. With large number of slots, power consistency and phase continuity should be maintained for UE side, which is determined by UE. If the power consistency and phase continuity is violated, the gain coming from JCE is limited. Assisting by AI/ML-enable advanced receiver, we expect the channel estimation performance can be improved even the power consistency and phase continuity can not be guaranteed by UE side, which means the UE core requirement on power consistency and phase continuity can be relaxed to enable large number of repetitions for JCE processing assisting by AI/ML-enabled advanced receiver at Network side based on UE reporting maximum number of DMRS bundling size.

Base above analysis, RAN4 could study the AI/ML-enable channel estimation with above potential scenarios. Since the DMRS pattern for PDSCH and PUSCH transmission in 6G is not decided yet according to RAN1 design, from RAN4 perspective, we think the study on feasibility of AI/ML-enable channel estimation in 6GR could be started based on existing DMRS patterns as starting point.

- RAN4 shall study AI/ML-enable channel estimation for 6GR, including the potential scenarios based on existing DMRS patterns as starting point:

- Rel-15 NR type 1 with 2 symbols structure up to 4 ports configured for normal PDSCH and PUSCH requirement

- Rel-18 NR enhanced type 1 with 2 symbols structure up to 8 ports configured for normal PDSCH and PUSCH requirement

- Rel-15 NR type 1 with 2 symbols structure configuration for PDSCH and PUSCH requirement in high-speed scenario with AI/ML- enabled channel estimation

- Re1-15 NR type 1 with 1 symbol structure configuration for PDSCH and PUSCH requirement with repetition transmission and joint channel estimation (JCE) with AI/ML- enabled channel estimation

4.4.2 Preliminary evaluation results

As for evaluation, the following types of AI based advanced receivers are proposed for comprehensive evaluation on channel estimation for 6GR:

- Baseline: Legacy receiver (non-AI based)

- Alt. 1 receiver: AI-based denoise only

- Alt. 2 receiver: AI-based channel estimation directly

- Alt. 3 receiver: AI-based {channel estimation +equalization}

Specially, the legacy receiver (non-AI based) can be considered as the baseline for performance comparison. The Alt.1 receiver comprises AI-based denoise processing, and legacy channel estimation for interpolation, channel equalization and data demodulation modules. The Alt.2 receiver comprises AI-based channel estimation, legacy channel equalization and legacy data demodulation modules. And the Alt.3 receiver comprises AI-based channel estimation and equalization, and legacy data demodulation modules. Meanwhile, we also provide the ideal channel estimation for comparison for Alt.1 and Alt.2 to check the benefit of AI based channel estimation.

Depending on the different AI based receiver assumption, the input, output and labeled dataset for each receiver are illustrated in Table 5.

Table 5 Illustration of the input, output, label data generation and model backbone

| Receiver assumption | Input | Output | Label dataset | Model backbone |
| --- | --- | --- | --- | --- |
| Alt.1 receiver: | DMRS signal after decorrelation and de-occ operation | The denoised DMRS signal | Ideal channel estimation, it can be obtained by RS measure in high SNR or label free | MLP-mixer |
| Alt.2 receiver | DMRS signal after de-correlation and de-occ operation | The estimated channel in full frequency and time domain REs | Ideal channel estimation, it can be obtained by known data or sequence and measured in the high SNR condition |  |
| Alt. 3 receiver | -DMRS signal after de-correlation and de-occ operation -The transmitted DMRS signal - The received data signal - The estimated noise variance on DMRS signal | The estimated data symbols in full frequency and time domain | Known transmission data signal |  |

For AI based receiver, it is purely one-sided model without the collaboration between UE and NW side. The model can be deployed either UE or NW side for data processing and the training is done in an offline manner.

The following KPIs to comprehensively the performance can be considered for the evaluation:

- NMSE (Normalized mean square error):

- BLER/Tput: to quantitatively assess the impact of block error rate performance.

- MFLOPs: to evaluate the computational complexity

From Rel-18 study, NMSE is also one of the intermediate KPI for the evaluation. In this use case, in a similar way, the NMSE can be used. Specifically, the NMSE between the ground-truth and the estimated channel is defined as following

NMSE=10log10j=0Nsymbslot-1i=0NRB·NscRB-1Hideali,j-Hout(i,j)2j=0Nsymbslot-1i=0NRB·NscRB-1Hideal(i,j)2

where Hout(i,j) is estimated channel based on AI based and Hideali,j is the ideal channel coefficients, and Nsymbslot, NscRB and NRB denote as the number of symbols per slot, number of subcarriers per RB, and number of allocated RB, respectively.

In current evaluation, the intermediate KPI NMSE is only applied for Alt.1 receiver (AI based denoise) and Alt.2 receiver (AI based channel estimation). As an eventual KPI, the BLER or throughout can be considered.

To study the feasibility of AI/ML-enabled channel estimation, the following DMRS patterns can be considered for evaluation with different AI based channel estimation or receivers’ assumption in both frequency domain and time domain. As benchmark, the existing NR Rel-15 type 1 DMRS structure can be referred as in Figure 6, where FD OCC as 2, FD density =1/2, and number of DMRS symbols is configured. It was selected as the typical assumption to specify the PUSCH demodulation performance with fading channel condition. Meanwhile, in the time domain, we also use the existing type 1 DMRS structure with 3 DMRS symbols to check the feasibility of AI/ML channel estimation as pattern 4 in Error! Reference source not found.Figure 7, which it is a typical configuration under high doppler scenario, such as high-speed train.

Figure 6 DMRS pattern 1, FD OCC=2, FD density =1/2, Rel-15 NR type 1 with 2 symbols

Figure 7 DMRS pattern 4, FD OCC=2, FD density =1/2, Rel-15 NR type 1 with 3 symbols

Comprehensive simulations demonstrate the effectiveness of AI/ML across with the typical configuration applied in 3GPP RAN4 for PUSCH requirement evaluation in TS 38.104. The evaluation assumptions are listed in Table 6.

Table 6 Evaluation assumptions for AI/ML-enabled channel estimation

| Parameter |  | Value |
| --- | --- | --- |
| Carrier Frequency |  | 4GHz |
| Channel |  | CDLC300 |
| Antenna configuration |  | 2 Tx ports, [M, N, P, Mg, Ng] = [1, 1, 2, 1, 1] 2 Rx ports, [M, N, P, Mg, Ng] = [1, 1, 2, 1, 1] |
| UE speed |  | 3km/h, 30km/h, 60km/h |
| MCS |  | MCS 16 in table 1 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 15 kHz SCS: FDD |
| HARQ | Maximum number of HARQ transmissions | Disabled |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | Patten 1: Rel-15 DMRS type 1, FD density =1/2, 2 DMRS symbols Pattern 4: FD density =1/2, 3 DMRS symbols |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 (11) for 2 DM-RS symbols {pos1, pos2} = {7,11} for 3 DM-RS symbols |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | Pattern 1: -3dB Pattern 4: -3dB |
|  | DM-RS port | {0, 1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 8RB and 2 RB for legacy receiver 8 RB for Alt.1 receiver and Alt.2 receiver: 2 RB for Alt.3 receiver: |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer |  | 0 |
| Time/Frequency offset |  | 0 |
| Interference |  | INR=5.35dB Interference channel: CDLC300-100 Interference signal: 16QAM |
| Channel estimation |  | LS+ Linear interpolation |
| Channel equalization |  | MMSE-IRC |

Firstly, to check the efficiency of AI/ML for channel estimation, the NMSE results for Alt.2 Receiver (AI based channel estimation) compared with the legacy channel estimation (LS+ linear interpolation) were provided in Figure 8. The results show that the proposed AI/ML receiver (alt. 2) can achieve good NMSE performance with DMRS pattern 1 under different doppler scenarios

(a) CDLC300-11                          (b) CDLC300-42                                  (c) CDLC300-667

Figure 8 NMSE performance comparison with legacy receiver and AI/ML based receiver (Alt 2), 8RB

Figure 9 shows the BLER results with DMRS pattern 1 under different AI based receiver assumption in different channel conditions. With the support of AI based channel estimation, up to 1.5 dB gain can be achieved. The evaluation results demonstrate that AI based receiver is capable of operating with orthogonal DMRS configuration with FD domain to improve the performance.

Figure 9 BLER performance comparison with legacy receiver and AI/ML based receiver (Alt 2), 8 RB, without TO and without interference, CDLC300-667

Figure 10 shows the BLER performance of AI/ML receiver (Alt. 1) compared with legacy receiver for DMRS pattern 1 under CDLC300-11 channel condition.

Figure 10 BLER performance comparison with legacy receiver and AI/ML based receiver (Alt 1), CDLC300-11, 8RB

Figure 11 shows the BLER performance of AI/ML receiver (alt. 3) compared with legacy receiver for different DMRS pattern under CDLC300-667 channel condition. The results show that the proposed AI/ML receiver (alt. 3) can achieve around 0.5 dB gain at 30% BLER.

Figure 11 BLER performance comparison with legacy receiver and AI/ML based receiver (Alt 3), CDLC300-667, 2 RB

- AI/ML-enabled receiver is capable of operating with orthogonal DMRS configuration with FD domain to improve the performance.

Furthermore, Figure 12 provides the BLER performances of legacy receiver with DMRS pattern 1 and DMRS pattern 4,

under high doppler scenario (CDLC300-667). The results show that with AI/ML based channel estimation under DMRS pattern 1 can still achieve better performance with DMRS pattern 4. That means it is feasible that AI/ML can help to improve the throughput with reducing the DMRS overhead in the time domain while maintaining the comparable performance with legacy receiver under high doppler scenario.

Figure 12 BLER performance comparison with legacy receiver with high density DMRS pattern 4 in time domain and AI/ML based receiver (Alt 2) with DMRS pattern 1, CDLC300-667, 8RB

- AI/ML-enabled receiver can help to improve the throughput with reducing the DMRS overhead in the time domain while maintaining the comparable performance with legacy receiver under high doppler scenario.

In Table 7, the model complexity and model parameters for different type of AI receiver is summarized

Table 7 Model complexity comparison

| Receiver assumption | Flops | Parameters | Note |
| --- | --- | --- | --- |
| Alt. 1 receiver: AI-based denoise | 4.35M | 267K | 8RB as input |
| Alt. 2 receiver: AI-based channel estimation directly | 5.9M | 340K | 8RB as input |
| Alt. 3 receiver: AI-based {channel estimation +equalization} | 83.61M | 11.43M | 2 RB as input |

In our current study, we only focus with typical DMRS configuration with FD-OCC=2 for evaluation. Besides the FD-OCC, the TD-OCC, double symbol structure and flexible DMRS symbol configuration are also supported in current 5G NR design to support multiple layers transmission and more UE multiplexing.  Therefore, in terms of supporting up to 48 orthogonal DMRS ports in 6G Day-1, pending on RAN1 design on DMRS pattern, RAN4 could investigate and evaluate the PDSCH and PUSCH performance with AI based channel estimation solution or receiver based on DMRS pattern specified in RAN1 for 6GR, and define the corresponding PDSCH and PUSCH requirement with AI/ML-enabled advanced receiver based on UE and network AI/ML capability.

- RAN4 could investigate and evaluate the PDSCH and PUSCH performance with AI based channel estimation solution or receiver based on DMRS pattern specified in RAN1 for 6GR, and define the corresponding PDSCH and PUSCH requirement with AI/ML-enabled advanced receiver based on UE and network AI/ML capability.

Generalization performance plays a paramount role in the cost-benefit analysis of AI/ML use cases as well as determination of aspects that should be considered/configured in the training, dataset collection and inference. Besides the channel fading, we think the time offset/frequency offset, and interference will also have the impact on the channel estimation performance and the final throughput. The poor time offset/ frequency offset estimation and interference condition will result in the inaccuracy estimation performance. Figure 13 shows the BLER performance with different DMRS patterns under interference condition, where one interference with INR as 5.35dB is applied based on 3GPP RAN4 assumption.

Figure 13 BLER performance with different DMRS patterns under interference condition, INR=5.35dB, 8RB, AI/ML based receiver (Alt 2), CDLC300-667

- Taking the interference into account, the performance degradation was suffered. These impacts should be considered when performing the AI/ML model generalization verification.

4.4.3 Study focus in RAN4

With supporting of AI/ML-enabled advanced receiver for channel estimation, from RAN4 perspective, the PDSCH and PUSCH performance should be investigated pending on UE and Network AI/ML capability.  For AI/ML based receiver, although the details of AI/ML receiver are up to implementation, the input and output of AI/ML model will be different pending on different receiver assumption. Given that, it may have the impact the LCM procedure, such as data collection, performance monitoring as model inference. Therefore, it should be better to clarify the evaluation assumption of AI receiver, related with the model input and output, and the corresponding data collection method as well as the label data generation. Meanwhile, for practical scenario, such as JCE, the UE may relax the requirement on power consistency and phase continuity, and report the corresponding capability to support the maximum number of DMRS bundling size.

- In 6GR, consider to support of AI/ML-enabled advanced receiver for channel estimation, the following aspects shall be studied:

- UE and network AI/ML capability

- RAN4 demodulation requirement with AI/ML-enable advanced receiver

- LCM procedure: including data collection, performance monitoring, and model interference pending the assumption of AI/ML receiver

- UE capability on related the supported maximum number of DMRS bunding size for JCE.

## 4.5 Use Case-3: AI/ML-SRS-assisted channel reconstruction

4.5.1 Motivation and use case description

When the channel dimensions are large, e.g., large number of ports for X-MIMO, mTRP, large CSI bandwidth, prediction/measurement instances, DL CSI acquisition at the network faces several challenges.

Challenges of DL CSI acquisition based on CSI-RS measurement: One of the issues of DL CSI acquisition for a large number of antenna ports is overhead for measurement (CSI-RS) and feedback. There are various UE-side channel prediction methods to reduce the CSI-RS measurement overhead. However, these approaches remain constrained by feedback overhead, or by quantization loss when lower feedback overhead is assumed. This is a consequence of the large channel dimensions that UE has to report CSI for after prediction or reconstruction. The large dimension implies large number of ports, large number of CSI reporting frequency units (subbands) and time-domain instances.  As a toy example, UE-side spatial-domain prediction form 64 measured ports to 256 CSI feedback ports, reduces the measurement overhead by 1/4. However, this approach may still be constrained by high feedback overhead as the feedback corresponds to larger number CSI ports (channel dimension).

Challenges of DL CSI acquisition based on SRS measurements: The overhead of SRS based DL channel measurement is agnostic to the number of NW’s antenna ports. This might make SRS an attractive alternative for DL CSI acquisition for a large number antennas in X-MIMO. However, SRS-based DL CSI acquisition is constrained by various factors, including limited coverage (especially when bandwidth increases), power imbalance and SRS overhead when the number of served UEs increases. One approach to address SRS coverage could be frequency-domain partial sounding, i.e., to boost SRS power. However, this incurs large latency to cover large bandwidth and results in channel ageing. Figure 14 shows the SGCS degradation (as compared to the reference, i.e., unit SGCS) as a result of channel ageing and power imbalance.

Figure 14 Impact of channel ageing and SRS power imbalance in the DL channel acquisition

- SRS based DL CSI acquisition for large channel dimensions is challenged by factors including SRS coverage or channel ageing and SRS power imbalance.

To address the aforementioned issues, we discuss approaches that combine the benefits of CSI-RS based CSI feedback and SRS-based measurement. As an example, the UE may measure spatially sub-sampled CSI-RS and feedback explicit CSI (channel matrix) for sub-sampled channel without prediction to larger dimension (up sampling). The sub-sampling could also be in the frequency and temporal domains. The network may combine SRS measurements with the CSI feedback on the explicit (channel matrix) as shown in the Figure 15. The CSI feedback corresponding to spatially sub-sampled measurements, e.g., 64 ports out of 256 ports, carries the channel information for the entire CSI BW. On the other hand, one-shot SRS transmission covers only the fraction of CSI BW, e.g., 3 RBs, to achieve comparable coverage with CSI-RS transmission. In contrast, the SRS measurement provides full spatial domain information, e.g., 256 ports, as it is assumed to be received by all the receive ports. Here, the SRS may also be transmitted from the subset of the received ports as opposed to the entirety of them. In particular, it may be sufficient if the UE transmits SRS on the antenna ports with higher power. Additional advantage of such NW-side up sampling or reconstruction is that the reconstruction models could easily be site-specific, trained for the specificity of deployment scenario and network-side additional conditions, e.g., antenna layout, subsampling patter, TxRU mapping, etc.

As illustrated in Figure 15, for the above purpose, two cases can be considered.

- Case1: NW-side model-based approaches. Here, the network-side model may combine CSI feedback and SRS measurement. The CSI feedback may not necessarily rely on AI-based feedback as long as it is for explicit channel matrices, e.g., we may consider CB-based feedback or linear projection-based approach, as outlined in the previous section.

- Case 2: Two-sided model-based approach. Here, the CSI encoding and the corresponding reconstruction must be trained to be compatible, e.g., based on the training collaboration approaches outlined in the NR.

Figure 15 Two-sided model for SRS-assisted CSI reconstruction from spatially sub-sampled CSI-RS measurement

4.5.2 Preliminary evaluation results

To evaluate performance of SRS-assisted explicit channel reconstruction, the parameters in Table 8 are considered. The two-sided model-based approach as illustrated in the Figure 16 is considered. It is shown that the two-sided model-based approach achieves a significant gain, e.g., close to 100% SGCS gain for layer 1, as compared to the impaired SRS measurement. The SRS measurement is impaired by channel aging as a result of 30ms interval to cover the CSI bandwidth as well as power imbalance. It is to be noted these results are for non-fixed power imbalance ratios. The SRS power imbalance coefficients are varied across their mean value with uniform distribution in the range [-1, 1]dB. Therefore, the results can be considered reasonably generalizable across diverse UE-side implementations and variations.

Table 8 Simulation parameters for results for joint SRS and CSI feedback

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| M (measured CSI-RS ports) | 64 | Channel | UMa Dense-urban |
| N (NW antenna ports) | 256 | SRS measurement interval | 30ms |
| Nr (number of UE receive ports) | 4 | Mean power imbalance (4 antennas) | [0 -3 -6 -9]dB |
| Carrier frequency/BW | 7 GHz/ 20MHz 13 SBs | Power imbalance distribution | Uniform [-1,1] dB centered on the mean. |

Figure 16 SRS-assisted explicit CSI reconstruction: SGCS gain as compared to impaired SRS-based DL CSI acquisition

- Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions.

4.5.3 Study focus in RAN4

The SRS-assisted explicit channel reconstruction can work together with two-sided model, and also NW-sided model, as long as the explicit channel can be obtained for data collection for training.

- Study SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement for both NW-sided model and two-sided model.

## 4.6 Use Case-4: AI/ML-enabled PRACH receiver

4.6.1 Motivation and use case description

In mobile communication system, the random access (RA) procedure serves as a critical mechanism for establishing uplink synchronization by resolving two key parameters: the preamble index (RAPID) and timing advance (TA) between UE and BS. For contention-based random access (CBRA), UEs autonomously select preambles from a limited pool of preamble sequence, creating inherent collision risks when multiple UEs select the identical preamble. Such collisions force contending UEs to initiate power-ramped preamble retransmissions, resulting in higher power consumption and latency on the UE side, especially for power-constrained or latency-sensitive devices.

The NR PRACH resource partitioning exacerbate this collision challenge during each Random Occasion (RO). While Rel-15 5G NR systems allocated 64 dedicated preambles per RO, emerging services in 5G evolution accelerates preamble resource fragmentation, thus reducing the available contention-based preamble subset for random access and thereby amplifying the challenge of multi-user collision.

Conventional RAPID detection relies on threshold-based peak identification utilizing the power delay profile (PDP), derived by correlating the PRACH preamble with a locally generated ZC sequence. A preamble is declared detection if the maximum peak of PDP in the search window (SW) exceeds the predetermined threshold, simultaneously resolving the RAPID and TA. This approach exhibits three fundamental limitations: (a) Static-setting threshold through empirical methods: Predefined threshold could fail to adapt to dynamic channel condition, particularly in low SNR scenarios. (b) Near-far power disparity: In multi-user environments, strong interferers mask weaker signal, thus complicate the threshold setting for multi-user detection. The analysis underscores the need to develop an effective preamble detection algorithm designed to operate efficiently in scenarios involving multiple users.

Machine learning (ML)-based PRACH detection has merged as a transformative approach and drawn attention in both academic and industrial societies. With AI-based PRACH receiver, early contention resolution can be feasible in which the BS employs the AI-based receiver to detect a preamble and estimate the number of UEs that selected and transmitted the same preamble. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants, where the number of grants corresponds to the BS’s estimation of how many UEs transmitted the same preamble in the same PRACH resource. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants. The contented UE can choose the corresponding TA and UL grand for msg3 transmission to solve the preamble collision in early stage. By enabling this kind of contention resolution at the Msg1/Msg2 stage for a substantial portion of collisions, it reduces access delay, improves first-attempt success probability, and enhances uplink resource efficiency, making it particularly beneficial for 6G networks with massive and bursty access demands.

To enable the early contention resolution with AI/ML-enabled PRACH receiver, from RAN1 and RAN2 prospective, random-access procedure to solve multi-user collision is needed, especially for msg2 to support grants for multi-user corresponding to one preamble, and msg3 transmission in UE side.

In general, assisting by AI/ML technologies, we expect that the following benefits can be achieved

- Detection performance improvement including both missing detection and false alarm detection compared with legacy receiver in both single-user and multi-user collision scenario

- Access delay and power consumption reduction for UE random access

From RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified. For demodulation requirement, only a single preamble detection performance without considering the multi-user collision was specified in current NR system, it may be difficult to reflect the multi-user collision performance considering 6G requirement on massive connectivity put challenges on random access. Meanwhile, current RAN4 performance requirement for PRACH detection is 99% detection probability and 0.1% false alarm probability, it is also difficult to meet the requirement and real field in cases the multi-user in collision scenario.

For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access. For example, the correct behaviour for UE when receiving random access response is that UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random-Access Response contains a Random-Access Preamble identifier corresponding to the transmitted Random-Access Preamble. In case RAR including multiple grands and TA corresponding to one preamble ID, the UE correct behaviour may need to be studied.

- For RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified.

- For PRACH requirement, only single preamble detection performance without considering the multi-user collision was specified.

- For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access.

4.6.2 Preliminary evaluation results

The following shows a preliminary result of detection performance at BS AI/ML-based PRACH receiver for both single-user and multi-user collision scenario. For simulation, we utilize the typical assumption to assess the performance of the proposed method in relation to both detection probability and false detection probability, referred to as Pd and Pf, respectively. According to the targeting requirements outlined in 3GPP, Pd must be equal to or greater than 99%, while Pf should be less than 0.1%. The conditions for the simulation are delineated in Table 9.

Table 9 Simulation conditions for AI/ML-enabled PRACH receiver

| Parameter | Values |
| --- | --- |
| Channel type | AWGN/TDLC300-100 |
| PRACH format | C0 |
| Ncs | 0/23 |
| Logical index | 0/1 |
| Preamble ID | 0/5 |
| Number of Preamble | 64 |
| Number of UE | 1/2 |
| Doppler Shift | 2334Hz for restrict set B |
| Number of Tx/Rx | 1/2 |
| Timing offset between 2UE | 20samples |

Figure 17 Multi-user collision detection performance in both AWGN and fading scenario

The Figure 17 illustrates the detection performance Pd of two UEs in AWGN and TDLC300-100, respectively. -8dB and -2dB correspond to the minimum SNR required by 3GPP specifications to meet the performance for a single user under AWGN and TDLC300-100, respectively. When the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%.

Meanwhile, for single user scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches, as illustrated in Figure 18. Also, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region as illustrated in Figure 19.

Figure 18 Single-user detection performance in AWGN scenario, (Ncs, RAPID, logical root sequence index) as (0,0,0)

Figure 19 Single-user detection performance in HST scenario, (Ncs, RAPID, logical root sequence index) as (23,5,1) for HST restriction setB

4.6.3 Study focus in RAN4

With supporting of AI/ML-enabled advanced receiver for PRACH detection, from RAN4 perspective, the PRACH performance should be investigated pending on Network AI/ML capability. For AI/ML based receiver, although the details of AI/ML receiver are up to implementation, the input and output of AI/ML model will be different pending on different receiver assumption. Given that, it may have the impact the LCM procedure, such as data collection, performance monitoring as model inference. Therefore, it should be better to clarify the evaluation assumption of AI receiver, related with the model input and output, and the corresponding data collection method as well as the label data generation.

Meanwhile, pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, the RRM requirement for random access also need to be specified.

- RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver.

- Pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, RRM requirement for random access also need to be studied, including the UE correct behaviour when receiving random access response.

## 4.7 Use Case-5: AI/ML-enabled RRM

4.7.1 Background: Rel-19 AI/ML-enabled mobility

Figure 20 Sub-use cases/Scenarios/AI model functions/Goals studied in Rel-19 5G AI/ML-enabled mobility study

As illustrated in Figure 20, measurement prediction and measurement event prediction are discussed in Rel-19 study, in which temporal case A, temporal case B and inter-frequency case are included.

4.7.2 Potential RAN4 AI-RRM use cases

Based on the experience from Rel-19 for AI/ML-enabled time/frequency/spatial prediction from beam-level and cell-level, the following use cases for AI-RRM use cases can be considered in RAN4 6G AI-RRM: AI/ML-enabled RRM: MG and MO reduction.

4.7.2.1	AI/ML-enabled RRM measurement enhancement: MG and MO reduction

RRM measurements on serving and neighbour cells are performed by the UE both in idle/inactive and connected modes in order to ensure that the UE is at all times served by a cell, and measurements are almost involved in all RRM procedures. Based on the current measurement mechanism, when UE performs measurement, measurement gap (MG) and measurement object (MO)s are key factors.

Regarding MG, two main UE capabilities are considered: per-UE gap and per-FR gap, for the gap types, we have: Type 1 gap (Gap(s) configured via GapConfig without suffix), Type 2 gap (Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17), NCSG, MUSIM gap, Positioning gap, Rel-18 concurrent gap, PreConfigured gap, and in 6G, maybe more gap types will be introduced/defined. And based on the gap pattern configuration defined in TS 38.133 9.1.2, we have #0~#25 gap patterns. Take FR1 as example, the MGs in FR1 are used for inter-frequency measurements, inter-RAT measurements with NR target RAT, intra-frequency measurements on PCC and SCCs when the SSB is not within the active DL BWP

Problem/Motivation

- Such cumbersome gap design make UE implementation becomes complicated.

- Throughput degradation due to measurement gap. When UE use MG to perform measurement, UE cannot receive/or transmit on downlink/uplink shared channels of the serving cell during such MG.

- UE power consumption is relatively high. When the UE to perform the measurement of a frequency, two steps from UE side should be considered：

Step 1:  	The UE receives the RSs of the frequency via RF receiver, RF retuning is required if necessary. The proportion of power consumption is this step is about 80%.

Step 2:  UE processes the RS to obtain the measurement result (e.g., RSRP) of all the cells in the frequency via BB.

Figure 21 Example of MG reduction with MO skipping

- Trade-off between measurement overhead and mobility performance. Based on the current requirement definition, the measurement period for inter-frequency measurements with gaps is related to MGRP configuration, when MGRP is large, the measurement overhead is lower, while the measurement period becomes long, the subsequent mobility action will be delayed, but short MGRP will lead to significant measurement overhead.

Considering the above motivation for MG reduction and/or MO(s) reduction, this use case can be further studied in 6G AI-RRM.

- In 6G, RAN4 can study and evaluate potential gains of AI/ML-enabled mobility for MG reduction and/or MO(s) reduction, considering the enabling AI/ML techniques including

- Cell-level measurement prediction for multiple MOs scenario (s) (e.g., inter-frequency scenario)

- Both co-located and non-collocated cell prediction including time-domain and frequency-domain, co-located cell prediction can be as starting point

4.7.2.2 New scenarios for AI-RRM

Most of companies have negative attitude on non-collocated scenario prediction in Rel-19 SI due to the low spatial correlation between the two frequencies, so inter-frequency prediction starts evaluation from co-located scenario. From our understanding, the application scope of inter-frequency prediction can be extended to non-collocated scenario as long as the frequencies of two CCs is not very far away. And based on the observation in 5G SI that, the closer the input and output frequencies, the higher the correlation between input and output, we think from the strong learning capability of AI/ML model, the key features between two non-collocated frequencies in the same band can be learnt especially based on frequency-cluster based consideration. However, the impact on timing issue and how to define generalization requirement and the corresponding testing need to be studied.

Figure 22  N-carrier single cell scenario

In 5G NR AI-mobility SI, we studied intra-cell intra-frequency time domain prediction, inter-cell inter-frequency frequency domain prediction. Considering N-carrier single cell can be an important way for frequency aggregation; therefore, we suggest time and/or carrier domain prediction can also be considered in this new scenario.

- In 6G AI/ML-enabled RRM mobility, the application scope of AI-prediction can be extended the following scenarios:

- among CA carriers in FR1 non-collocated NR-CA

- among N carriers in N-carrier single cell scenario.

# 5. Conclusion

In this contribution, we have provided our viewpoints on RAN4-related AI/ML issues, including RAN4 driven use cases (only except the testability and OTA related AI) and RAN4-relevant issues of AI/ML framework.:

Scope and timeline for RAN4 6G AI/ML study

- In 6G AI/ML study, the detailed RAN4 scope shall include:

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

  - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility.

  - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures: RAN4 shall study relevant RAN4 requirements for LCM procedures.

  - RAN4 specific issues to guarantee AI/ML functionalities is implementable and testable, including post-deployment, AI/ML model complexity and power consumption, UE and network AI/ML capability, etc.

- Suggested RAN4 timeline for Rel-20 6G AI/ML study

Stage 1: Q4'25 and Q1'26

- To clarify study scope for RAN4 AI/ML framework

- To select the top-prioritized AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

Stage 2: Q2’26 to Q2’27

- To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

- To study comprehensively the use case(s) selected in Stage 1

- To study other WG-driven use cases with RAN4 analysis and evaluation

RAN4 AI/ML framework

- The same and unified LCM framework shall be targeted for RAN4-driven use cases and use cases driven by other working groups.

- For RAN4 study of LCM procedure for AI/ML framework, take 5G NR AI/ML framework as a starting point for RAN4-driven use cases, by at least considering

-	Terminologies in TR 38.843

-	UE-side and NW-side data collection if needed

-	Applicability report

-	Associated ID to indicate additional conditions that may not be explicitly configured

-	Performance monitoring

-	Dedicated AI/ML processing unit (APU) and timeline

-	Testing and Performance Requirements

- For RAN4 specific issues in AI/ML framework, the following issue can be further studied:

- The AI/ML model complexity

- UE and network AI/ML capability and capability-level categorization

- Power consumption model for AI/ML inference and other AI/ML relevant procedures.

RAN4 driven AI/ML use cases - General

- A use case can be selected as RAN4-driven AI/ML, if at least one of the following conditions are met:

(1) The use case is to solve key pain point which is better understood in RAN4.

(2) The use case can be classified as AI-receiver with no or limited RAN1/2 impacts, and major specification impacts are related to RAN4.

Note: Collaboration/interaction between UE and NW shall be minimized in RAN4-driven use case.

- Coordinated discussions with other working groups are required to avoid overlapping scope between different working groups.

- At least the following RAN1 agreed aspects to be studied and reported for each proposed (sub-)use case should be considered for RAN4 study for AI/ML use cases identification/categorization:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

  - no collaboration/interaction

  - UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

- Additionally, RAN4 study for AI/ML use case identification/categorization shall consider implementation feasibility, including computational complexity, power consumption etc.

Use Case-1: AI/ML-enabled non-linearity compensation

- AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values.

- The goal of the work in 6G is to enable low-end PA and extreme high modulation to transmit higher power, leveraging the capabilities of advanced BS receiver.

- Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions.

- Adopt the following working steps for feasibility study for both AI and Non-AI solutions.

  - Study item phase (demod centric)

    - Step-1: Selection of Non-linearity model(s) of transmission signals

- UE RF session firstly to determine the non-linearity model(s)

    - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

      - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, and demodulation relevant evaluation is performed in AI session for AI based solution

- Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM

- For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

  - Performance evaluation

    - Case 1: Conventional receiver including non-linearity TX model: SNR-a

    - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b

    - Case 3: Conventional receiver without any non-linearity TX model: SNR-c

    - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d

    - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

  - Performance comparison

    - Compare SNR-d and SNR-c, the results should be comparable

      - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment

    - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver

- Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-d derived based on Case 4 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3.

  - Perform different PA models, if needed

  - WI phase (UE RF centric)

    - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation:

  - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

  - For instance for 256QAM Tx EVM, 3.5% -> 6%,

    - For selected PA model(s), evaluate the allowed TX power boosting (i.e., MPR reduction) for different power class

    - FFS the necessity for new EVM model for RAN4 demodulation test cases.

Note: Above approaches are applicable for both 5G waveforms and 6G waveforms

Use Case-2: AI/ML-enabled channel estimation

- Up to 28% DMRS overhead is expected in existing NR Rel-15 design.

- RAN4 shall study AI/ML-enable channel estimation for 6GR, including the potential scenarios based on existing DMRS patterns as starting point:

- Rel-15 NR type 1 with 2 symbols structure up to 4 ports configured for normal PDSCH and PUSCH requirement

- Rel-18 NR enhanced type 1 with 2 symbols structure up to 8 ports configured for normal PDSCH and PUSCH requirement

- Rel-15 NR type 1 with 2 symbols structure configuration for PDSCH and PUSCH requirement in high-speed scenario with AI/ML- enabled channel estimation

- Re1-15 NR type 1 with 1 symbol structure configuration for PDSCH and PUSCH requirement with repetition transmission and joint channel estimation (JCE) with AI/ML- enabled channel estimation

- AI/ML-enabled receiver is capable of operating with orthogonal DMRS configuration with FD domain to improve the performance.

- AI/ML-enabled receiver can help to improve the throughput with reducing the DMRS overhead in the time domain while maintaining the comparable performance with legacy receiver under high doppler scenario.

- RAN4 could investigate and evaluate the PDSCH and PUSCH performance with AI based channel estimation solution or receiver based on DMRS pattern specified in RAN1 for 6GR, and define the corresponding PDSCH and PUSCH requirement with AI/ML-enabled advanced receiver based on UE and network AI/ML capability.

- Taking the interference into account, the performance degradation was suffered. These impacts should be considered when performing the AI/ML model generalization verification.

- In 6GR, consider to support of AI/ML-enabled advanced receiver for channel estimation, the following aspects shall be studied:

- UE and network AI/ML capability

- RAN4 demodulation requirement with AI/ML-enable advanced receiver

- LCM procedure: including data collection, performance monitoring, and model interference pending the assumption of AI/ML receiver

- UE capability on related the supported maximum number of DMRS bunding size for JCE.

Use Case-3: AI/ML-SRS-assisted channel reconstruction

- SRS based DL CSI acquisition for large channel dimensions is challenged by factors including SRS coverage or channel ageing and SRS power imbalance.

- Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions.

- Study SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement for both NW-sided model and two-sided model.

Use Case-4: AI/ML-enabled PRACH receiver

- For RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified.

- For PRACH requirement, only single preamble detection performance without considering the multi-user collision was specified.

- For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access.

- RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver.

- Pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, RRM requirement for random access also need to be studied, including the UE correct behaviour when receiving random access response.

Use Case-5: AI/ML-enabled RRM

- In 6G, RAN4 can study and evaluate potential gains of AI/ML-enabled mobility for MG reduction and/or MO(s) reduction, considering the enabling AI/ML techniques including

- Cell-level measurement prediction for multiple MOs scenario (s) (e.g., inter-frequency scenario)

- Both co-located and non-collocated cell prediction including time-domain and frequency-domain, co-located cell prediction can be as starting point.

- In 6G AI/ML-enabled RRM mobility, the application scope of AI-prediction can be extended the following scenarios:

- among CA carriers in FR1 non-collocated NR-CA

- among N carriers in N-carrier single cell scenario.

# 6. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2511652, “RAN4 6G Plan and Meeting Arrangement”, RAN4 chair, RAN4#116.

[3] R4-2511652, “Discussion on general RF and UE RF for 6GR”, Samsung, RAN4#116bis.