3GPP TSG-RAN WG4 Meeting # 116bis                                      R4-2513088

Prague, CZ, 13th – 17th Oct., 2025,

Agenda Item: 7.10.3

Source: MediaTek Inc.

Title: Discussion on RRM impact for NR MIMO Phase 6

Document for: Discussion

# Introduction

RAN#109 agreed on the revised WID “NR MIMO Phase 6” [1]. According to WID, RAN4 is supposed to define requirements for the following enhancements:

- On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions andrequirements from the legacy RAN4 spec for DMRS bundling should be retained. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

As this is the first meeting, we would like to discuss the potential RRM core impact.

# Discussion

## 2.1 Objective#1: Enhancing UL capacity and coverage

For Objective#1, there are two enhancements. Both of the enhancements are on SRS transmission. For SRS transmission, RAN4 RRM session defined the interruption requirements due to SRS carrier switching and antenna switching. One enhancement in R20 is on the start RB index. The legacy requirements are defined transparent to the start RB index. So we can conclude that the enhancement “Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding” has no impact on RRM core requirements.

Proposal 1: The enhancement “Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding” has no impact on RRM core requirements.

Another enhancement is to support cross-slot SRS. The legacy interruption requirements due to SRS carrier switching is defined based on slot level. The interruption requirement for SRS antenna switching are defined based on either symbol-level or slot-level, depending on the number of SRS resources. Even cross-slot SRS is supported, the legacy requirements are also applicable. We also observed that there is proposal in RAN1 to introduce SRS resources with more than 14 symbols. If it is agreed, RAN4 may need to further check whether the legacy requirements are still applicable.

Proposal 2: For the enhancement “Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”, wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable.

## 2.2 Objective#2: Enhancing DL CSI acquisition

According to the WID, there are three scenarios to enhance. In the next, we would like to discuss the RRM impact one by one.

UE transitioning from IDLE/INACTIVE to CONNECTED mode

One scenario is for UE transitioning from IDLE/INACTIVE to CONNECTED mode. As RAN4 never defines any delay requirements for transition from IDLE/INACTIVE to CONNECTED mode, RAN4 does not need to define any RRM core requirements for DL CSI acquisition for transitioning from IDLE/INACTIVE to CONNECTED mode.

Observation 1: RAN4 never defines any requirements for transition from IDLE/INACTIVE to CONNECTED mode.

Proposal 3: Not to define any RRM core requirements for DL CSI acquisition for transitioning from IDLE/INACTIVE to CONNECTED mode.

SCell activation

Another scenario is for SCell activation. There are two aspects. One aspect is early CSI/CSI-RS triggering. Checking the legacy SCell activation delay requirements, the scenario that CSI-RS triggered at the same time as SCell activation command had already been considered. The ending point of legacy SCell activation requirements is that UE reports the first valid CSI report for the SCell. So RAN4 does not need to define any new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

Proposal 4: No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

Another aspect is early SRS triggering. Legacy SCell activation requirements include three parts: THARQ, Tactivation_time and TCSI_reporting. Even early SRS is triggered for SCell activation, the first two parts, i.e., THARQ, Tactivation_time, should be the same. The only issue is whether to update TCSI_reporting to the time to transmit SRS or something others. From UE’s perspective, after Tactivation_time, UE is ready to receive or transmit on the SCell. The ending point is just the point to notify NW that SCell has been activated. We are open to use either the first valid CSI report or the first SRS transmission as the ending point.

Proposal 5: When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

SCell dormancy

The third scenario is switching out of SCell dormancy. RAN4 defined the switching delay and interruption requirements for SCell dormancy. After the switching delay, UE will be ready to transmit SRS or receive CSI-RS for CSI acquisition. Switching delay and interruption on other carriers are caused by UE BB/RF processing. Early SRS/CSI/CSI-RS triggering has no impact on UE BB/RF processing for BWP switch, so there is also no impact on the switching delay and interruption requirements of SCell dormancy.

Proposal 6: For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

# Summary

In this paper, we provide some views on the potential RRM core impact of NR MIMO Phase 6. We have the following proposals and observation:

Proposal 1: The enhancement “Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding” has no impact on RRM core requirements.

Proposal 2: For the enhancement “Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”, wait for RAN1 conclusions and check whether the legacy interruption requirements due to SRS carrier switching and antenna switching are still applicable.

Observation 1: RAN4 never defines any requirements for transition from IDLE/INACTIVE to CONNECTED mode.

Proposal 3: Not to define any RRM core requirements for DL CSI acquisition for transitioning from IDLE/INACTIVE to CONNECTED mode.

Proposal 4: No need to define new RRM core requirements for early CSI/CSI-RS triggering for SCell activation.

Proposal 5: When early SRS is triggered for SCell activation, the two parts THARQ, Tactivation_time, in legacy SCell activation delay requirements should remain the same. FFS whether to update the last component “TCSI_reporting”.

Proposal 6: For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable.

# 4 Reference

RP-252510, Revised Work Item: NR MIMO Phase 6.