3GPP TSG-RAN WG4 Meeting #116bis		                         R4-2513091

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR

Source: 	OPPO

Agenda item:	6.10.3

Document for:	Approval

Introduction

In last meeting, the performance requirements for fast SCell activation for UE supporting Rel-18 EMR were discussed and the agreements were captured in the WF [1]. In this paper, we will provide our views on the remaining open issues captured in [2].

Discussion

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

In last meeting, the test case list was agreed above and the test case for direct SCell activation at SCell addition for FR2 is FFS. Considering the requirements for direct SCell activation at SCell addition with EMR report are defined, the corresponding test case should also be defined. And based on the agreed bigCR in [3], the following conditions are needed to apply the fast SCell activation:

Requirements in this clause apply if the SCell is unknown, provided: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode,

If the SCell belongs to FR1, Tactivation_time is TFirstSSB_MAX + Trs + 5ms provided that the side condition Ês/Iot ≥ -2dB is fulfilled, if the following conditions are met: - During the period equal to 5s for FR1 before the reception of the SCell activation command: - the UE has sent a measurement report with SSB index according to the reporting requirements in 4.7.3 or 5.8.3. - the SSB measured is detectable from the earliest measurement to the end of the defined SCell activation latency (i.e., a valid CQI reporting) - If measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 is configured, the earliest measurement refers to the time instance measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 before Msg1 transmission. - If only measIdleDuration-r16 is configured, the earliest measurement refers to the time instance when the UE obtained the configuration parameter. - the SSB measured remains detectable according to the IDLE/INACTIVE mode measurement conditions specified in 4.2 or the CA/DC measurement conditions specified in 4.4 when UE is in IDLE/INACTIVE state and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3 when UE is in CONNECTED mode.

There are three types of scenarios to apply the requirements. To verify the fast SCell activation delay requirements, the configurations in the test cases need to meet the above conditions based on UE capability. It was agreed to set configurations of Rel-18 capabilities in last meeting, but considering UE may only have the Rel-16 capability, the third configuration should also be included. And the three configurations can be the sub-tests of each scenario. For simplifying the test, if UE support Multiple configurations, UE is allowed to pass only one of the sub-tests.

Proposal 1: Define test case for direct SCell activation at SCell addition for FR2.

Proposal 2: For each test case, configuration 3 should also be included.

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode.

For normal SCell activation and PUCCH SCell activation, there are different test cases for different test configurations, but we do not need to define all the similar test cases for fast SCell activation and choose only one set of configurations (e.g., non-DRX, 640ms SCell measurement cycle) is enough.

Proposal 3: For normal SCell activation and PUCCH SCell activation, choose only one set of legacy configurations (e.g., non-DRX, 640ms SCell measurement cycle) to define the test case for fast SCell activation.

Based on the discussion on test case list, there are many similar test cases are defined which results to repeated testing. To reduce the testing efforts, some applicability rules can be defined, e.g., for UE supporting direct SCell activation or PUCCH SCell activation, the testing for normal SCell activation can be skipped.

Proposal 4: RAN4 to define some applicability rules to reduce the testing efforts.

Regarding the test procedure, we understand the test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively. And in the test, there are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test.

During T1, UE is in RRC_IDLE mode and performs cell reselection measurement or CA/DC measurement on the two cells based on the configurations. At the beginning of T2, UE is connected to Cell 1, but is not aware of Cell 2. The test configurations of T1 and T2 can use A.6.1.1/A.7.1.1 or A.6.6.9/A.7.6.23 as baseline. At the beginning of T3, UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2, and the UE now starts monitoring the SCC. At the beginning of T4 (denoted as slot #n), UE receives the MAC message at the UE antenna connector for activation of SCell. The UE should meet the fast SCell activation delay requirements during T4. For direction SCell activation, T3 and T4 can be merged into one. The test configurations of T3 and T4 can use A.6.5.3 as baseline.

Proposal 5: The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively.

- T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation.

- For direct SCell activation, T3 and T4 can be merged into one.

Proposal 6: The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline.

Proposal 7: The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline.

Conclusions

In this paper, we provided our views on the performance requirements for fast SCell activation for UE supporting Rel-18 EMR. And the following proposals are provided.

Proposal 1: Define test case for direct SCell activation at SCell addition for FR2.

Proposal 2: For each test case, configuration 3 should also be included.

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode.

Proposal 3: For normal SCell activation and PUCCH SCell activation, choose only one set of legacy configurations (e.g., non-DRX, 640ms SCell measurement cycle) to define the test case for fast SCell activation.

Proposal 4: RAN4 to define some applicability rules to reduce the testing efforts.

Proposal 5: The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively.

- T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation.

- For direct SCell activation, T3 and T4 can be merged into one.

Proposal 6: The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline.

Proposal 7: The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline.

Reference

- R4-2512125 WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2509055 Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT

- R4-2512348 Big CR for NR Radio Resource Management (RRM) Phase 5