3GPP TSG RAN WG4 Meeting #116bis		R4-2513169

Prague, Czech,  13 – 17 October 2025

Title: 	TP to TR 38.793 to introduce UL CA_n5-n8

Agenda Item:	5.9.3

Source: 	CATT

Document for:	Approval

1 Introduction

This contribution provides text proposal on the NR band combination UL CA_n5-n8.

2 Text Proposal

--- < Start of change> ---

5.x	CA_n5-n8

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A15 | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |

NOTE 15: For CA_n5-n8, two uplinks are supported for UEs that support IE [supportOfnon-ConcurrentInterbandULDL-r19] capability with non-concurrent n5 downlink and n8 uplink, and UE is not expected to transmit in the UL band n8 when any reception is needed in the DL band n5. Otherwise uplink is only in n5.

5.x.1.3	UE co-existence studies for 1 band UL

The UL/DL harmonics/harmonic mixing analysis and cross band isolation can be found in the clause 5.1.2 of TR 38.872.

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n8, the TIB,c and RIB,c values are given in the tables below, which have been specified in the spec.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n5-n8 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n5-n8 | 0.4 | 0.4 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

The MSD due to cross band isolation for DL band n8 from aggressor UL n5 has been specified in the spec.

Table 5.x.1.5-1: MSD due to cross band isolation for CA_n5-n8

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | 2.8 | >ACLR2 |

As there is no concurrent between UL n8 and n5 DL, there is no need to specify the MSD due to cross band isolation for DL band n5 from aggressor UL n8.

5.x.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements for CA_n5-n8.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n8A | 23 | +2/-3 |

5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n5 + Band n8 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n5 and Band n8 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 91 | 69 | 1728 | 1750 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 973 | 1006 | 733 | 766 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2632 | 2665 | 2552 | 2585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1877 | 1921 | 1557 | 1601 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 138 | 182 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3536 | 3580 | 3376 | 3420 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3456 | 3500 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2436 | 2381 | 2836 | 2781 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 697 | 642 | 1097 | 1042 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4200 | 4255 | 4440 | 4495 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4280 | 4335 | 4360 | 4415 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, there is no IMD issue for CA_n5-n8 with the following frequency range restriction.

n5	UL: 824 MHz – 835 MHz	DL: 869 MHz – 880 MHz;

n8	UL: 904 MHz – 915 MHz	DL: 949 MHz – 960 MHz.

However, IMD3 may fall into the DL band n8 if both band n5 and n8 transmit the signal without frequency restriction.

For CA_n5-n8, although PHS frequency range 1884.5-1915.7MHz is as protected band of band n5, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n5-n8. Thus no protected bands are needed.

5.x.2.3	REFSENS requirements

Referring to the IMD3 MSD value from CA_n8-n20, 25dB MSD can be specified for UL CA_n5-n8 without frequency range restriction by implementing non-concurrent n5 DL and n8 UL solution.

Table 5.x.2.3-1: MSD test parameters due to the two UL IMD3

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n81 | n5 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |
|  | n8 | 882.5 | 5 | 25 | 927.5 | 25 | FDD | IMD3 |
| NOTE 1: This test point is applicable to UEs that support IE [supportOfnon-ConcurrentInterbandULDL-r19] capability with non-concurrent n5 downlink and n8 uplink.. |  |  |  |  |  |  |  |  |

5.x.3	The solution with non-concurrent n5 DL and n8 UL in Rel-19

5.x.3.1	RF implementation assumption:

Generally, the RF implementation could be achieved for different RRC configuration as below.

|  | RRC configuration | RFFE configuration |
| --- | --- | --- |
| Case 1 | 2UL/2DL | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx - #B: n5-UL:Tx, n8-UL:Tx, n8-DL:Rx |
| Case 2 | 1UL/2DL (like SDL) | - #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx |

Based on companies’ contributions, there are two assumed RF implementations for 2UL/2DL RRC configuration at least.

RF Implementation 1: the RFFE configuration can be switched between RFFE#A and RFFE#B under RRC configuration 2UL/2DL.

RF Implementation 2: the RFFE configuration can be configured as RFFE#B under RRC configuration 2UL/2DL.

5.x.3.2	The investigation about the impacts from other working groups

Investigation on RAN2’s spec:

According to the RAN2’s LS R2-2501572, CA_n5-n8 with non-concurrent n5 downlink and n8 uplink (option 2 in TR 38.872) is feasible without RAN2’s spec impacts beyond the introduction of UE capability.

Investigation on RAN1’s spec:

According to the paper R4-2511054, assuming n5 and n8 are collocated, power control does not seem to be an issue as n5 UL can use the n8 DL for pathloss reference in power control through the configuration of pathlossReferenceLinking.

It’s observed that there is no RAN1/RAN2’s spec impacts beyond the introduction of UE capability for the proposed solution with non-concurrent n5 DL and n8 UL for both RF implementation 1 and RF implementation 2.

Investigation on RRM’s spec:

An example is illustrated without RRM impacts as follows to enable UL CA_n5-n8 and balance the time-frequency domain resources between UL and DL:

| RRC configuration case 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | Pcell |  | Scell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A (k+1 Slots) | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+k-1 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+k | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| RF Switching | Slot N+k+1 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| RFFE#B (17-k Slots) | Slot N+k+2 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | …… | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
| RF Switching | Slot N+19 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |

Step 1: Access n5 as PCell

Step 2: RRC reconfiguration to Case 2: Add n8 with only n8-DL as SCell  UL Tx in SCell via n5-UL

  - RFFE configuration is #A: n5-UL: Tx, n5-DL:Rx, n8-DL:Rx

Step 3: RRC reconfiguration to Case 1: PCell remains at n5, reconfigure SCell n8 by adding n8-UL

  - Freedom of RFFE configuration

    - Alt.#1: Not changed, remaining as #A: (n5-UL: Tx, n5-DL:Rx, n8-DL:Rx)

    - Alt#2: Or reconfigured to #B: (n5-UL:Tx, n8-UL:Tx, n8-DL:Rx)

Step 4: When n8-UL needs to be scheduled from Slot #N+k+2 to Slot #N+m (where m=18 for the illustration),

  - NW to choose a slot #N+k or before slot #N+k, where DCI issued to the UE that n8-UL transmission happens at slot #N+k+2

    - k is chosen

      - one slot switching period is large enough so the UE can make successful RFFE reconfiguration to Alt. #2

      - There will be no SSB/PDCCH/CSI-RS on n5-DL to be received by the UE between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration)

    - The DCI also schedules n5-UL between Slot #N+k+1 and #N+m+1 (where m=18 for the illustration) via 1 TB mapping to multiple slots via features:

      - Repetitive (Rel-15)

      - Non-repetitive (Rel-17)

Hence n5-UL transmission during this period is not impacted.

  - UE to complete reconfiguration of RFFE between Slot #N-k and #N (k is chosen to guarantee that no n5-DL reception is required)

  - UE to make UL transmission at n8-UL at Slot #N until Slot #N+m

  - Once UE complete n8-UL transmission, reconfigure RFFE from #B to #A immediately with one slot switching period.

  - There is no uplink transmission on n8-UL for the UE between Slot #N and Slot #N+k

Step 5: Repeat Step 4 if new n8-UL transmission is required.

NOTES:

- PCell is always n5, and n8 always serves as SCell.

- SCell is active

  - No SCell activation issue

- PCell and SCell is collocated, and have the same frame timing

  - No UL timing issue

- Only RFFE Alt.#A works (i.e., when RRC reconfigured from Case 2 to Case 1, RFFE configuration do not change), otherwise, NW do not configure the UE with concurrent n5-DL and n8-UL (scheduling restriction)

- UE capability required

  - NW needs to know the UE capability : (1) No RFFE reconfiguration when adding n8-UL to SCell; (2) RFFE reconfigured back to 1UL/2DL when finishing UL transmission on n8-UL.

Besides the UE capability, UE just follows NW instructions, no need for change of behaviours, signalling, etc., and no additional requirements.

5.x.3.3	The Gain investigation for this solution

The Time-Frequency Scheduling Occasion are summarized below for RRC configuration case 1 and case 2.

| RRC configuration case 1 (Where k=9) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | Pcell |  | Scell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A (10 Slots) | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+5 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+6 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+7 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+8 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| RF Switching | Slot N+9 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| RFFE#B (8 Slots) | Slot N+10 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+11 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+12 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+13 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+14 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | NA | PUCCH/PUSCH | PDCCH+PDSCH |
| RF Switching | Slot N+19 | PUCCH/PUSCH | NA | NA | PDCCH+PDSCH |
| Time Domain Scheduling Occasion | Unit / (ms) | 20 | 8 | 8 | 18 |
| Time-Frequency Scheduling Occasion | Unit / (MHz*ms) | 200 | 80 | 120 | 270 |
| Total TFSO (UL+DL) | Unit / (MHz*ms) | 670 |  |  |  |
| Total TFSO (UL) | Unit / (MHz*ms) | 320 |  |  |  |
| Total TFSO (DL) | Unit / (MHz*ms) | 350 |  |  |  |

| RRC configuration case 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RF Conf. | CBW (MHz) | 10 |  | 15 |  |
|  | Slot Index | Pcell |  | Scell |  |
|  |  | UL band n5 | DL band n5 | UL band n8 | DL band n8 |
| RFFE#A | Slot N | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+1 | PUCCH/PUSCH | SSB+CSI-RS+PDCCH | NA | SSB+CSI-RS+PDCCH |
|  | Slot N+2 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+3 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+4 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+5 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+6 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+7 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+8 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+9 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+10 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+11 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+12 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+13 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+14 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+15 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+16 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+17 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+18 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
|  | Slot N+19 | PUCCH/PUSCH | PDCCH+PDSCH | NA | PDCCH+PDSCH |
| Time Domain Scheduling Occasion | Unit / (ms) | 20 | 18 |  | 18 |
| Time-Frequency Scheduling Occasion | Unit / (MHz*ms) | 200 | 180 |  | 270 |
| Total TFSO (UL+DL) | Unit / (MHz*ms) | 650 |  |  |  |
| Total TFSO (UL) | Unit / (MHz*ms) | 200 |  |  |  |
| Total TFSO (DL) | Unit / (MHz*ms) | 450 |  |  |  |

For the exemplary two cases, it’s observed that the total Time-Frequency Scheduling Occasion for the RRC configuration case 2 (1UL/2DL: 650 MHz*ms) is less than he RRC configuration case 1(2UL/2DL: 670 MHz*ms). The system gain is observed for the proposed solution. In addition, the proposed solution can help enable UL CA_n5-n8 and balance the UL and DL time-frequency for one UE which can support UL CA_n5-n8 with non-concurrent n5 downlink and n8 uplink.

--- < End of change> ---

Reference

[1] RP‑251762, “TR 38.768 v1.0.0 on Low NR band carrier aggregation via switching”, Apple

[2] RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS

[3] R1- 2505061, “Introduction of low NR band carrier aggregation via switching for NR”, Samsung

[4] R4-2508118, “WF on UE RF requirements for LB CA via switching”, Apple