3GPP TSG RAN WG4 Meeting #116bis		R4-2513178

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on Sensing/ISAC for 6G study

Agenda Item:	8.11

Source: 	CATT

Document for:	Approval

# 1 Introduction

In the RAN#109 meeting, a 6G RAN WG SI [1] was updated with the following objectives related to sensing.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

Based on the RAN4 Chair’s guideline [2] on the RAN4 6G discussion and meeting arrangement in Rel-20 and the draft of RAN4#116bis meeting agenda, the general scope for UE RF requirements include the following parts.

- Sensing/ISAC [FS_6G_Radio] # NOTE: The scope includes RAN4 related PHY functions and procedure, RF, coexistence and testability

Based on the draft of RAN4#116bis meeting agenda, there are five aspects to be discussed under the sensing/ISAC agenda. Meanwhile, we’d like to look back the studies’ outcomes for Sensing/ISAC in the R19 and R20 for NR. After that, we’d like to provide our initial views about 6G Sensing/ISAC as one important function of multi-functional RAN at the 6G stage.

# 2 Current status for the studies of NR sensing/ISAC

1) Referring to SA1’s inputs (TS 22.137 [3] / TR 22.837 [4]), the service requirement, performance requirements and a lot of use cases were studied and specified by SA1.

Three implementation ways were considered, i.e. Monostatic sensing, Bistatic sensing and Multistatic sensing.

Clause 4.2 sensing operation of TS 22.137 Sensing operation can be implemented in a couple of different ways, from radar like sensing where the sensing transmitter and sensing receiver are co-located in the same entity (figure 4.2-1), called Monostatic sensing, to have the sensing receiver and sensing transmitter in different entities (figure 4.2-2), also called Bistatic sensing. A more advanced scenario with multiple sensing transmitters and receivers is also possible, called Multistatic sensing. Figure 4.2-1: Example of sensing with co-located sensing receiver and sensing transmitter Figure 4.2-2: Example of sensing with separated sensing receiver and sensing transmitter

5G wireless sensing performance requirements included Accuracy of positioning estimate, Accuracy of velocity estimate, Confidence level, Sensing Resolution (e.g., range, velocity), Missed detection probability, False alarm probability, Max sensing service latency and Refreshing rate.

The clause 6.1 5G wireless sensing performance requirements of TS 22.137 The following set of key performance requirements is used and shown for each service scenario in Table 6.2-1. - Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude. - Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity. - Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy. - Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude. - Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results. NOTE 1: An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive. - False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results. NOTE 2: An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative. - Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. - Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. 5G wireless sensing performance requirements are applied to 3GPP sensing data and sensing results.

For different scenarios (e.g. Object detection and tracking, Environment monitoring and Motion monitoring), sensing results with the performance requirements in Table 6.2-1 of TS 22.137 [3] should be provided by 5G system.

2) During the release 19 of RAN1 study, the channel modelling for ISAC was studied with the outcome in the clause 7.9 of TR 38.901 [5]. The following objects are studied based on R19 SID [6] on the channel modelling for ISAC in order to support object detection and/or tracking (as per the SA1 meaning in TS 22.137 [3]).

- UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency

All six sensing modes were considered during the R19 channel modelling study for ISAC (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.)

3) During the release 20, one SID [7] on NR ISAC was established and led by RAN1 with the following objectives and scope.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

As RAN1 only discussed this topic in one meeting, RAN1#122 in August, 2025, the relevant outcomes were captured in the latest status report [8].

Agreement Horizontal/vertical positioning accuracy are agreed as performance metrics for NR ISAC. - It is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target. - Note: in RAN1 evaluations, there should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Scenario; UMa-AV, Optional RMa-AV. Carrier frequency; Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz]. System bandwidth; 100 MHz. Numerology; SCS = 30kHz. BS Layout; Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees. Inter-BS (2D) distance; Uma-AV: 500m, optional 1000m, RMa-AV: 1732m. BS antenna height; 25m for UMa-AV, 35m for RMa-AV. Wrap-round; No wrap-round. Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Target type; UAV with small size (0.3m x 0.4m x 0.2m). Target distribution when target(s) are dropped; N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m. Mobility; horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h. Note: N=0 will be discussed in a later proposal in relation to false alarm. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Minimum BS-target 3D distance; 10 m. Minimum target-target (3D) distance; 10 m. Target outdoor/indoor proportion; 100% outdoor. LOS/NLOS; LOS and NLOS. Orientation; Random in horizontal domain. RCS model; RCS model 1 for UAV with small size. Agreement For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions: - One true target is associated with at most one detected object - One detected object is associated with at most one true target - The same association applies to miss detection, false alarm probability Type 2 (if defined) and positioning/velocity accuracy Companies to report the method used for association of the detected object(s) and the true target(s) Agreement - Velocity accuracy is agreed as performance metric for NR ISAC. - Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target. - For single TRP monostatic sensing, - The radial velocity accuracy can be estimated - The true radial velocity is the projection of true velocity on the direction from TRP to target for TRP monostatic. - The true velocity accuracy can be estimated.

The whole timeline and scope on ISAC for NR are summarized below.

# 3 Discussion for 6G Sensing/ISAC

## 3.1 Scenario, Use case and KPI for 6G Sensing/ISAC

Referring to the latest TR 38.914 [9], the following scenarios, use cases and Key Performance Indicator (KPI) were summarized below.

# 4 Deployment scenarios Editor note: Sensing related aspects will be included in section 4 4.0 General 10 4.1 Indoor hotspot 10 4.2 Dense urban 12 4.3 Rural 14 4.4 Urban macro 15 4.5 Sub-Urban macro 16 4.6 High speed 16 4.7 Extreme long distance coverage in low density area 17 4.8 Urban coverage for massive connection 17 4.9 Air-to-Ground Scenario 17 4.10 Non-Terrestrial Network 17 4.11 [Urban grid] 18 4.12 [Highway Scenario] 18

Clause 5.1 Key performance indicators of TR 38.914 ### 5.1.14 Positioning Positioning is the ability to estimate the position of connected devices. The positioning accuracy is the [90%] point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s]. Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)]. The minimum requirements for the device positioning accuracy for various [test environments] are summarized in Table 5.1.14-1. Table 5.1.14-1: Positioning accuracy [Test Environment]; Horizontal Accuracy; Vertical Accuracy. Indoor Factory - ISAC. [Urban Macro – ISAC / Urban – ISAC]. ### …… ### 5.1.17 Sensing Sensing-related capabilities are measured in terms of the following technical performance requirements: • Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. • Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. • Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

### Clause 5.4 Requirements of New and Existing Services of TR 38.914 ### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Observation 1: the use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs, are supported by 6GR and 6G RAN architecture referring to the latest TR 38.914.

Observation 2-1: For positioning, the positioning accuracy is considered as the KPI, which include Horizontal positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude) and [Vertical positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

Observation 2-2: For Sensing, Detection Probability, False Alarm Probability, Horizontal/Vertical Localization Accuracy and Velocity Accuracy are considered as the KPI.

As RAN1/RAN2 haven’t started to discuss sensing/ISAC, the modes including Monostatic, Bistatic, Multistatic should be considered from 6G study’s perspective.

Proposal 1: As RAN1/RAN2 haven’t started to discuss sensing/ISAC for 6GR, the modes including Monostatic, Bistatic, Multistatic should be considered from 6G study’s perspective.

## 3.2 Targeted Frequency

Although Frequencies from 0.5 to 52.6 GHz are the primary focus during the Rel-19 study on the channel modelling for NR ISAC.

But the following factors should be considered when the targeted centre frequency (fc) /wave length (λ=c/fc) is chosen.

1) The form factor or size of physical object to be detected vs the wave length (λ) of sensing signal.

Firstly, it should be guaranteed that the wave length (λ) of sensing signal is much less than the size (a) of object, so that the RCS (Radar Cross Section) can be located at the Optical Region. Where is in the optical region, the RCS (Radar Cross Section) of physical object will not be changes for different centre frequency. As shown below, 2πa/λ should be larger than 10 where is Optical Region. In other words, size (a) of physical object is equal to or larger than three times wave length (λ).

| Centre frequency (fc) | 2GHz | 4GHz | 6GHz | 30GHz |
| --- | --- | --- | --- | --- |
| Wave length (λ) | 15cm | 7.5cm | 5cm | 1cm |
| Minimum size of physical object | Around 45cm | Around 22.5cm | 15cm | 3cm |

For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz. For use cases about whether objects (Whether Radar), e.g. turbulence for detecting wind speed, raindrop, cloud and mist, the wave length for sensing signal should be chosen carefully considering the size of physical object to be detected.

Observation 3: For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz.

2) The Radar azimuth/elevation angular resolution vs the wave length (λ) of sensing signal.

Generally, the Radar azimuth/elevation angular resolution is relative to the wave length (λ) of sensing signal and antenna size (D). The formula is shown below.

θ3dB or φ3dB=k∙λD

Where, k is between 0.89 and 1.02 according to the experience. In other words, for the same size of antenna, the less wave length is, the less Radar azimuth/elevation angular resolution is.

Observation 4: for the same size of antenna, the less wave length is, the less Radar azimuth/elevation angular resolution is.

3) Large scale path loss vs the wave length (λ) of sensing signal.

As the centre frequency is increasing (the wave length of sensing signal is decreasing), the large scale path loss will increase.

For example, the basic monostatic (Pulse Wave) Radar Function for Point Physical Point is shown below.

The smaller wave length (λ) of sensing signal represents the smaller receiving power of echo signal.

4) Doppler frequency vs the wave length (λ) of sensing signal.

Generally, the Doppler frequency caused by relative velocity is calculated based on the following formula.

fdoppler=2vλ

Proposal 2: When the targeted centre frequency is discussed, the following factors should be considered together, e.g. the size (a) of physical object to be detected, Radar azimuth/elevation angular resolution (θ3dB or φ3dB), the Radar Function / the targeted receiving Power of echo signal and the Doppler frequency/Maximum velocity of object.

## 3.3 Bandwidth of Sensing signal

The range resolution for sensing signal is derived based on the following formula:

ΔRange=cτ2=c2B

Where, c is the velocity of light, 3×108 m/s, τ (second) is the Pulse duration for Pulse Radar or B (Hz) is the Bandwidth of sensing signal.

No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

| Bandwidth of Sensing Signal (MHz) | 5MHz | 10MHz | 20MHz | 30MHz | 50MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- |
| The range resolution (m) | 30m | 15m | 7.5m | 5m | 3m | 2m |

Observation 5: No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

## 3.4 sensing signal: Continuous Wave (CW) / Pulse Wave (PW)

According to the existing Radar system, there are two kinds of sensing signal/Radar System, i.e. Pulse Wave (PW) Radar and Continuous Wave (CW) Radar. We’d like to compare these two kinds of Radar System considering the following aspects.

|  | Pulse Wave (PW) Radar | Continuous Wave (CW) Radar |
| --- | --- | --- |
| Time Domain characteristic | Or |  |
| Frequency Domain characteristic |  |  |
| Time-Bandwidth Product (TBP) | τ×B=1 | τ×B>1 |
| Operating State of the transmitter PA | Saturation State for larger output power | Saturation State or linear State due to large TBP |
| Duplex Mode | The operation of transmission and receiving is in the Time Division Duplex (TDD). | The operation of transmission and receiving is in the Full Duplex at the same time and frequency (bandwidth). |
| Self-interference | As TDD is used for transmission and receiving, there is no self-interference issue. | There is a strong receiving self-interference coupled from transmitter. Generally, the methods of moving target detection technology should be used to eliminate the self-interference, e.g. self-heterodyne, Pulse-Doppler Method. |
| Range Blind Zone | cτ/2 | If Full Duplex Operation between transmission and receiving is used, there is no Range Blind Zone for CW Radar. |
| Pulse Compression | Not supported | Supported. For example, matched filter is used to complete the Pulse Compression for LMF signal, but the side lobe of strong Object may interfere the detection of the small Object. |

Proposal 3: the characteristics above for Pulse Wave (PW) Radar and Continuous Wave (CW) Radar can be considered as the part of RAN4 6G ISAC study.

Proposal 4: RAN4 should actively and closely work with RAN1 on design of sensing signal from system performance impacts and coexistence perspective.

# 4 Summary

2 Current status for the studies of NR sensing/ISAC

3.1 Scenario, Use case and KPI for 6G Sensing/ISAC

Observation 1: the use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs, are supported by 6GR and 6G RAN architecture referring to the latest TR 38.914.

Observation 2-1: For positioning, the positioning accuracy is considered as the KPI, which include Horizontal positioning accuracy in the horizontal plane (i.e., x/y axis, or latitude/longitude) and [Vertical positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

Observation 2-2: For Sensing, Detection Probability, False Alarm Probability, Horizontal/Vertical Localization Accuracy and Velocity Accuracy are considered as the KPI.

Proposal 1: As RAN1/RAN2 haven’t started to discuss sensing/ISAC for 6GR, the modes including Monostatic, Bistatic, Multistatic should be considered from 6G study’s perspective.

3.2 Targeted Frequency

Observation 3: For the agreed use cases of 6GR sensing , i.e. UAVs, human, vehicles and AGV, all of them can meet the RCS requirements of Optical Region for the centre frequency equal to or larger than 2GHz.

Observation 4: for the same size of antenna, the less wave length is, the less Radar azimuth/elevation angular resolution is.

Proposal 2: When the targeted centre frequency is discussed, the following factors should be considered together, e.g. the size (a) of physical object to be detected, Radar azimuth/elevation angular resolution (θ3dB or φ3dB), the Radar Function / the targeted receiving Power of echo signal and the Doppler frequency/Maximum velocity of object.

3.3 Bandwidth of Sensing signal

Observation 5: No matter what kind of Radar system we choose, the bandwidth of sensing signal will determine the range resolution of sensing signal.

3.4 sensing signal: Continuous Wave (CW) / Pulse Wave (PW)

Proposal 3: the characteristics above for Pulse Wave (PW) Radar and Continuous Wave (CW) Radar can be considered as the part of RAN4 6G ISAC study.

|  | Pulse Wave (PW) Radar | Continuous Wave (CW) Radar |
| --- | --- | --- |
| Time Domain characteristic | Or |  |
| Frequency Domain characteristic |  |  |
| Time-Bandwidth Product (TBP) | τ×B=1 | τ×B>1 |
| Operating State of the transmitter PA | Saturation State for larger output power | Saturation State or linear State due to large TBP |
| Duplex Mode | The operation of transmission and receiving is in the Time Division Duplex (TDD). | The operation of transmission and receiving is in the Full Duplex at the same time and frequency (bandwidth). |
| Self-interference | As TDD is used for transmission and receiving, there is no self-interference issue. | There is a strong receiving self-interference coupled from transmitter. Generally, the methods of moving target detection technology should be used to eliminate the self-interference, e.g. self-heterodyne, Pulse-Doppler Method. |
| Range Blind Zone | cτ/2 | If Full Duplex Operation between transmission and receiving is used, there is no Range Blind Zone for CW Radar. |
| Pulse Compression | Not supported | Supported. For example, matched filter is used to complete the Pulse Compression for LMF signal, but the side lobe of strong Object may interfere the detection of the small Object. |

Proposal 4: RAN4 should actively and closely work with RAN1 on design of sensing signal from system performance impacts and coexistence perspective.

# Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple)

[3] TS 22.137, “Service requirements for Integrated Sensing and Communication”, SA1

[4] TR 22.837, Feasibility Study on Integrated Sensing and Communication, SA1

[5] TR 38.901-v19.0.0, Study on channel model for frequencies from 0.5 to 100 GH, RAN1

[6] RP-242348, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Xiaomi, AT&T

[7] RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[8] RP-252200, SR on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, China Telecom

[9] RP-252882, TR 38.914 v0.2.0, “Study on 6G Scenarios and Requirements”, RAN