3GPP TSG-RAN WG4 Meeting #116bis                                                                R4-2513180

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Considerations on coexistence between AIoT and NR/LTE in outdoor scenarios

Agenda item:	7.11.3

Document for:	Approval

- Introduction

In RAN #108 meeting, a new SI [1] for A-IoT was set up and RAN4 work should be started in this meeting. In this contribution, we focus on the objectives in SID, share some views and proposals.

- Discussion

The objectives of the SID are shown as below, some co-existence study related items are highlighted.

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band.

According to the RAN4-led objective in the SID, RAN4 should study coexistence in outdoor scenarios. In this contribution, we propose the scenarios that should be studied in our mind.

  - Coexistence scenario

According to the SID, the objective is to study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. As NR and LTE has the same deployment scenario and the same simulation parameters, it is proposed only coexistence between A-IoT and NR is studied, co-existence with LTE can follow the results and conclusions, as in Rel-19.

Proposal 1: It is proposed only coexistence between A-IoT and NR is studied, co-existence with LTE can follow the results and conclusions.

According to the SID, Rel-20 A-IoT is operated in FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. For in-band and standalone coexistence simulation, only the assumptions for ACLR and ACS are different, the values for in-band operation are much stricter, and also in outdoor scenario, there is very difficult to allocate a separate spectrum for A-IoT, so we propose to simulate in-band operation for priority first, as shown in figure 1.

Figure 1 A-IoT R2D and D2R spectrum

Proposal 2: It is proposed co-existence study for in-band operation has priority.

2.2 Coexistence interference analysis

Figure 2  Interference between NR system and A-IoT system

RAN4 needs to study coexistence between Device 2b/Device c and NR/LTE in outdoor scenarios, and use band n8 as an example band. The deployment scenario is topology 1 with no external carrier wave, and the A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. According to the spectrum allocations of R2D and D2R, the interference case between NR system and A-IoT system are shown in figure 2 and table 1.

Table 1 Interference case between NR system and A-IoT system

| Numbering | Aggressor | Victim | Frequency | Aggressor → Victim |
| --- | --- | --- | --- | --- |
| 1 | NR BS | A-IoT device | DL | NR BS Tx → Device Rx |
| 2 | A-IoT reader | NR UE | DL | A-IoT reader Tx →NR UE Rx |
| 3 | NR UE | A-IoT reader | UL | NR UE Tx →A-IoT reader Rx |
| 4 | A-IoT device | NR BS | UL | Device Tx →NR BS Rx |

Proposal 3: The interference case between NR system and A-IoT system are shown in table 1.

2.3 Coexistence Assumptions

2.3.1 Deployment scenario

In SID, it requires A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs, below are A-IoT and NR cell layout.

Figure 3 Cell layout

SID also requires Maximum distance between Reader and Device of 50-500 m which means the ISD between BS is 50-500m. According to RAN4 previous study in Table 6.1 in [4], 500m cell range should belong to Urban Macro cells, so we suggest to reuse Urban macro cell assumptions in RAN4 co-existence study for 500m cell range. Other cell range between 50-500m is FFS.

Proposal 4：500m cell range and Urban Macro assumptions are used in RAN4 co-existence study. Other cell ranges between 50-500m are FFS.

Simulation assumptions related to network layout is captured for D4T1 in Table as below.

Table 2 Deployment parameters for D4T1

| Parameter | D4T1 |
| --- | --- |
| NR BS/ A-IoT reader deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, all the reader and NR BS are activated in one snapshot Inter-NR BS/ inter-A-IoT reader distance: 750m |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SC |
| BW for A-IOT system | 180kHz, for 15kHz SCS |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| Device distribution | 1 active A-IoT device under one reader at one drop, Minimum distance between reader and device is 1m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | MCL of 70 dB |
| Pathloss model | PLb : NLOS and LOS UMa in TR38.901 |

2.3.2	NR BS/ A-IoT reader RF characteristics

The NR BS and A-IoT reader parameters are defined as below table.

Table 3 NR BS/ A-IoT reader RF parameters

| Parameters | Values for evaluation |  |
| --- | --- | --- |
|  | NR BS | A-IoT reader |
| Tx power (dBm) | 46dBm | 38 or 33 dBm over R2D transmission BW[3] |
| Antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} Antenna gain (dBi) (including feeder loss):15 |  |
| Height of antenna (m) | 25 |  |
| Noise Figure (dB) | 5 |  |
| ACLR (RB level) | Option 1: 30dBc, Option 2: 17dBc |  |
| ACS (RB level) | [TBD] |  |

2.3.3	NR UE/ A-IoT device RF characteristics

The NR UE and A-IoT device parameters are defined as below table.

Table 4 NR UE/A-IoT device RF parameters

| Parameter | Values for evaluation |  |  |
| --- | --- | --- | --- |
|  | NR UE | Device 2b | Device c |
| TX power in dBm | -40 to 23 | -10 | 5 |
| Antenna gain (dBi) and antenna pattern | Omni direction antenna with 0dBi |  |  |
| Height of antenna (m) | 1.5 |  |  |
| Noise Figure（dB） | 9 | 15 | 13 |
| ACLR (RB level) | 30dBc | [10dBc] |  |
| ACS (RB level) | 30dBc | [TBD] |  |

Proposal 5: It is proposed to adopt simulation deployment assumptions and RF parameters in Table 2-Table 4.

- Conclusion

This contribution provides our views on coexistence in outdoor scenarios. The following proposals are concluded as follows:

Proposal 1: It is proposed only coexistence between A-IoT and NR is studied, co-existence with LTE can follow the results and conclusions.

Proposal 2: It is proposed co-existence study for in-band operation has priority.

Proposal 3: The interference case between NR system and A-IoT system are shown in Table 1.

Proposal 4：500m cell range and Urban Macro assumptions are used in RAN4 co-existence study. Other cell ranges between 50-500m are FFS.

Proposal 5: It is proposed to adopt simulation deployment assumptions and RF parameters in Table 2-Table 4.

- References

- RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

- TR 38.769

- R1-2506575, LS on BS Tx power assumption for coverage evaluation in A-IoT

- TR 36.942