3GPP TSG RAN WG4 Meeting #116bis		R4-2513238

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on enhanced BS co-location requirements

Agenda Item:	7.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#109, a new WID was approved for BS RF requirement evolution, and one of the objectives is to enhance BS co-location requirements [1]:

Enhancement of co-location requirements - Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3GHz - Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements. - Consider impact on the following OTA co-location requirements: - Transmitter OFF power requirement - Transmitter co-location spurious emission requirement - Transmitter intermodulation requirement - Co-location OOBB requirement - Enhance OTA co-location requirements - Improve the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered - Option 1: Investigate and improve CLRA concept - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS), if any

In this contribution, we present our initial views on the works for this objective.

# 2 Discussion

As stated in the WID, there are two tasks for enhancing co-location requirements for BS type 1-O for upper FR1 frequencies:

- BS-to-BS coupling loss evaluation Vs. frequency range and its impacts on the four identified OTA co-location requirements:

- Derivation of OTA co-location requirements with reasonable expected BS-to-BS isolation

For the first task, simulation or measurement campaign is expected since it is not completed in Rel-19.

For the second task, the current OTA co-location requirements are specified by introducing the Co-Location Reference Antenna (CLRA), so that the requirements are defined in terms of conducted power at the CLRA antenna port, as shown in the table below. Since the co-location requirements are specified at the CLRA conducted power, the corresponding Co-Location Test Antenna (CLTA) is defined as the practical counterpart of the CLRA, allowing some tolerance to improve availability for conformance testing.

|  | Type 1-O Co-location requirements |
| --- | --- |
| Transmit OFF | 9.5.2.2: The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz. |
| Transmit spurious co-location emission | 9.7.5.2.5: The power sum of any spurious emission is specified over all supported polarizations at the output(s) of the co-location reference antenna and shall not exceed the basic limits in clause 6.6.5.2.4 + X dB, where X = -21 dB. |
| Transmit inter-modulation | 9.8.2: Note: The transmitter intermodulation level is the total radiated power of the intermodulation products when an interfering signal is injected into the co-location reference antenna. |
| OOBB | 10.6.2.2: The interferer power levels are specified at the co-location reference antenna conducted input. Note: for non-co-location requirements, wanted signal power is in terms of EISminREFSENS + 6dB, and interfering signal is CW in terms of RMS field strength (V/m) |

Although the final enhanced OTA co-location requirements rely on the outcome of the first task, RAN4 may still start to look into potential methods assuming a reasonable BS-to-BS isolation, therefore, these two tasks can be conducted in parallel. Of course, to facilitate the discussions, two separate sub agenda items can be created from next meeting.

Proposal 1: Evaluation of BS-to-BS isolation for OTA co-location requirements and improving core requirements for upper FR1 frequencies should be performed in parallel, which requires two sub agenda items under this objective.

It is anticipated that the two potential approaches for deriving enhanced OTA co-location requirements—namely an improved CLRA-based method or one relying on existing OTA metrics—will be compared and investigated with respect to their impact on the four OTA co-location requirements. From the beginning, it should be clarified that the final four enhanced OTA co-location requirements must be defined using a single unified approach in order to reduce test costs; in other words, RAN4 should avoid a situation where some requirements are based on one approach while others are based on the other

Proposal 2: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs.

Once the final enhanced OTA co-location requirements are defined, compliance may be declared based on the vendor’s BS design.

Proposal 3: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified.

# 3 Conclusion

In this contribution, we have the following proposals for enhanced BS co-location requirements:

Proposal 1: Evaluation of BS-to-BS isolation for OTA co-location requirements and improving core requirements for upper FR1 frequencies should be performed in parallel, which requires two sub agenda items under this objective.

Proposal 2: RAN4 to agree a single unified approach to define enhanced OTA co-location requirements to avoid mixed definitions and reduce test costs.

Proposal 3: Enhanced OTA co-location core requirements for upper FR1 frequencies should be manufacturer-declared once specified.

# Reference

[1] RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei)

[2] 3GPP TR 25.942 v6.0.0, “Radio Frequency (RF) system scenarios”

[3] R4-2510227, “On improvements of co-location requirements”, Ericsson

[4] RP-252586, “CR to TR 37.941: Rel-19 CLTA-related agreements”, Huawei et., al.