3GPP TSG-RAN WG4 Meeting #116bis														R4-2513282

Prague, CZ, 13th – 17th October, 2025

Agenda Item:	8.12

Source: 	Xiaomi

Title: 	Overview for 6GR testability and OTA

Document for:	Discussion

- Introduction

According to the agreed 6G SID in [1], RAN4 is tasked to study the testability methodology framework and key assumption for 6G system. The related scope and objective is captured as follows:

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

In this contribution, our preliminary view on the testability methodology framework and key assumption for 6G system.

- Discussion

The SID of “Study on 6G Radio” defines core objectives for 6G system including:

- Immersive communication,

- Ubiquitous connectivity,

- Hyper Reliable and Low-latency communication,

- Integrated AI and communication.

- Integrated sensing and communication,

- Massive communication.

These requirements imply new demands on radio system design, including operation in new/wider frequency ranges, more complex antenna/array configurations, increased mobility/dynamics, and tighter energy constraints.

OTA testing is critical to ensuring that implementations meet these requirements in realistic, radiated environments rather than idealized conducted test setups alone.

Observation 1: OTA testing is critical to ensuring 6G commercial implementations to meet 6G core requirements in realistic.

Based on the 6G SID, we identify the following dimensions that OTA testing should be addressed or evolved in 6G:

| Dimension | Implication from RP-251881 | What OTA Testing Needs to Adapt |
| --- | --- | --- |
| Frequency / Spectrum | 6G aims to exploit higher bands (e.g. FR3 frequency range), as well as wider spectrum bandwidths. | Study the chambers, models, calibration, quiet zones to adopt wider bandwidths and new 6G spectrum. |
| Device type & New Performance Metrics | Diverse device types are supported in 6G | RAN4 needs to study new OTA performance metrics to adopt different device types; RAN4 may need to study new performance metric for new device types except the basic EIRP/TRP; |
| Non-Terrestrial and Hybrid Deployments | Integration of terrestrial and non-terrestrial networks (satellites, HAPS, drones etc.) is part of the study. | Study new channel model to simulate elevation angles, large Doppler shifts and large propagation delay to support NTN scenarios. The channel model for NGSO introduced in Rel-19 can be used as baseline. |
| ISAC (Integrated Sensing & Communication) | Integrated Sensing & Communication service is supported in 6G | Study the test method and performance metric to support ISAC service, e.g. the metric to support the performance accuracy in sensing (e.g. range, angle, Doppler, resolution) . |
| AI/ML | AI/ML-assistant RAN use cases are included in the 6G SID | Study the testability, generalization and performance metric to enable the new introduced AI/ML-assistant use case; |
| Energy Efficiency & Power Constraints | The SID stresses low energy consumption both at device and network side; efficient hardware and power usage are crucial. | RAN4 may need to study the performance metrics that show power consumption under OTA conditions, not only in conducted mode. |
| Advanced Antenna / Array / Beamforming | Use of large arrays, multi-panel UE, advanced beam control. | Study the boundary condition between near-field and far-field; Study the potential test method to support multi-panel for FR2, if needed. |

Observation 2: 6G OTA testing should evolve into a multi-dimensional validation framework that integrates new spectrum, advanced antennas, non-terrestrial networks, ISAC, AI/ML, energy efficiency, and enhanced performance metrics to accurately assess 6G devices in realistic, dynamic environments.

According to existing OTA test method/metric introduced in 5G NT, RAN4 may need to discuss and identify the gaps or issues need to be further discussed in 6G OTA testing framework to meet the new 6G requirements of the new 6G use cases and features:

- Frequency range support

  - For FR3 frequency range, it is ambiguous position between conducted and OTA testing;

- Test chamber/environment limitations: For new frequency ranges and wide bandwidths, chamber reflections, losses, and the “quiet zone” size become more challenging. Also, manufacturing tolerances and material behaviors (dielectric loss, surface roughness) start to matter significantly.

- Channel/mobility emulation: In NR, the OTA testing setups mainly assume static or slow changing environments. For NTN, or high speed use cases, dynamic channel modelling (fast Doppler, elevation angle variations, etc.) is insufficiently addressed.

- Testing method for ISAC: Existing OTA test procedures mostly target communication metrics (TRP/EIRP). There is little standardized methodology for sensing metrics (range error, Doppler resolution, angle of arrival, detection probability/false alarm etc.), especially when sensing and communication are transmitted or received simultaneously.

- Testing for AI/ML: Define what aspects of AI/ML behaviour need to be tested: performance metric under  AI/ML models, robustness to impairments, adaptation and re-training, and model stability.

- Beamforming complexity: Existing OTA test cases may cover single panel, limited beam steering, but 6G new features, e.g. RIS/LIS, may introduce new architectures whose behavior (phase continuity, mutual coupling, dynamic reconfiguration) is not fully captured in existing tests.

- Energy/Power Efficiency Metrics under OTA: the performance metrics for energy efficiency evaluation via OTA radiated levels may need to be studied in 6G. In addition, RAN4 may also need to study the power consumption vs. performance trade-offs in OTA environment under wideband/new frequency operation.

- Observation 3: 6G OTA testing should overcome critical gaps in frequency coverage, chamber capabilities, dynamic channel emulation, ISAC metrics, AI/ML validation, beamforming complexity, and energy efficiency to meet the demands of 6G core requirements.

Based on above observations, we propose that RAN4 should have further discussion on the following aspects as part of the 6G OTA Testing

- Frequency ranges for 6G OTA study

- Assumption for different device type, antenna/antenna array;

- Channel model for 6G OTA study, e.g. static or dynamic;

- Test method and metric for ISAC;

- AI/ML testing

  - OTA validation for AI/ML-assistant features;

  - Generalization and AI/ML Robustness Testing;

- Energy Efficiency evaluation under OTA performance metric;

- Chamber and Measurement Infrastructure for 6G OTA testing, e.g. chamber size, quiet-zone size, calibration, MU, etc.

Proposal 1: RAN4 should have further discussion on the following aspects as part of the 6G OTA Testing:

- Frequency ranges for 6G OTA study

- Assumption for different device type and antenna/antenna array;

- Channel model for 6G OTA study, e.g. static or dynamic;

- Test method and metric for ISAC;

- AI/ML testing

  - OTA validation for AI/ML-assistant features;

  - Generalization and AI/ML Robustness Testing;

- Energy Efficiency evaluation under OTA performance metric;

- Chamber and Measurement Infrastructure for 6G OTA testing, e.g. chamber size, quiet-zone size, calibration, MU, etc.

- Conclusion

In this contribution, we provided our preliminary view and observations on the testability methodology framework and key assumption for 6G system as follows:

Observation 1: OTA testing is critical to ensuring 6G commercial implementations to meet 6G core requirements in realistic.

Observation 2: 6G OTA testing should evolve into a multi-dimensional validation framework that integrates new spectrum, advanced antennas, non-terrestrial networks, ISAC, AI/ML, energy efficiency, and enhanced performance metrics to accurately assess 6G devices in realistic, dynamic environments.

- Observation 3: 6G OTA testing should overcome critical gaps in frequency coverage, chamber capabilities, dynamic channel emulation, ISAC metrics, AI/ML validation, beamforming complexity, and energy efficiency to meet the demands of 6G core requirements.

Proposal 1: RAN4 should have further discussion on the following aspects as part of the 6G OTA Testing:

- Frequency ranges for 6G OTA study

- Assumption for different device type and antenna/antenna array;

- Channel model for 6G OTA study, e.g. static or dynamic;

- Test method and metric for ISAC;

- AI/ML testing

  - OTA validation for AI/ML-assistant features;

  - Generalization and AI/ML Robustness Testing;

- Energy Efficiency evaluation under OTA performance metric;

- Chamber and Measurement Infrastructure for 6G OTA testing, e.g. chamber size, quiet-zone size, calibration, MU, etc.

Reference

[1]		RP-251881	New SID: Study on 6G Radio