3GPP TSG-RAN WG4 Meeting #116bis                                                R4-2513306

Prague, Czech Republic, Oct 13 – 17, 2025

Title: 	Consideration on BS RF study for 6G

Agenda Item:	8.5

Source: 	Huawei, Hisilicon

Document for:	Approval

- Introduction

SI on 6G study was formally approved in RAN#108 meeting [1]. The BS RF study for 6G has been given the scope by RAN4 chair as follows:

- non-spectrum and non-AI BS RF, MSR, BS RF related coverage and efficiency, and the coexistence study.

In this paper we briefly introduce the items which we think provide opportunity for further study for 6G in the following 2 areas.

- Potential optimization for existing FR1 bands

- RF requirements optimization for centimetre wave bands

- Discussion

2.1 Potential optimization for existing FR1 bands

Basically, for existing FR1 bands the NR/MSR/AAS requirements should be the baseline. Some potential consideration/update for 6G is discussed in the subclause.

The topic additional hybrid requirement sets for BS type 1-H/large antenna array has been in Rel-20 BS RF package [2]. The justification is to add more OTA options to avoid the over stringent conductive requirements without the verification the overall radiated performance, e.g. transmitted signal quality, co-location requirement, and transmitter intermodulation [3]. Meanwhile the situation is becoming true only for large antenna array in the future and for the legacy bands, there is no need to update since the whole ecosystem has matured.

Proposal 1: the enhancement of BS type 1-H is focusing on large antenna array case for bands above 5GHz.

For FR1 FDD bands, in order to prevent the receivers of the BSs being desensitised by emissions from a BS transmitter, requirements named protection of the BS receiver of own or different BS was defined.

TS 38.104 Table 6.6.5.2.2-1: BS spurious emissions basic limits for protection of the BS receiver

| BS class | Frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| Wide Area BS | FUL,low – FUL,high | -96 dBm | 100 kHz |  |
| Medium Range BS | FUL,low – FUL,high | -91 dBm | 100 kHz |  |
| Local Area BS | FUL,low – FUL,high | -88 dBm | 100 kHz |  |
| NOTE 1: For BS operating in band n104, the basic limit is increased by 1dB. NOTE 2: For BS operating in regions where a band is only partially allocated for NR operations (e.g. band n28), this requirement only applies in the UL frequency range of the partial allocation. NOTE 3: For BS capable of multi-band operation, Table 6.6.5.2.2-1 assumes that the supported operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special protection requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

The requirement assumes 30 dB coupling loss between BS transmitter to BS receiver of own. To meet the requirements, a large volume cavity filter with very high suppression is needed. This presents a number of obstacles to innovation in energy-efficient and minimalist deployments. Meanwhile all RX requirement is tested with TX on. Hence the requirement become to be not needed.

Proposal 2: It is proposed to study the possibility to remove the protection of the BS receiver of own BS for FDD bands.

One of the SI detailed objective is to support wider channel bandwidth (at least 200MHz) for 6G deployments at least above 2 GHz, around 7 GHz. It is expected the major workload is on the common system parameters such as spectrum utilization and sync raster design. For BS RF, as shown in Table 2.1-1, there would be a few impacts due to the introduction of 200 MHz or wider. One point need to be further discussed is operating band unwanted emission, considering that the PSD may be lower than that of small channel bandwidth.

Table 2.1-1: Initial assessment for support wide channel bandwidth

| Subject | Requirement | Initial assessment |
| --- | --- | --- |
| Transmitter | General | no change |
|  | Base station output power | no change |
|  | RE Power control dynamic range | no change |
|  | Total power dynamic range | minor change, a new row should be added for a new CBW |
|  | Transmit ON/OFF power | no change |
|  | Transmitted signal quality | no change |
|  | Occupied bandwidth | no change |
|  | ACLR | minor change, new CBW should be added |
|  | Operating band unwanted emissions | To discuss whether existing OBUE is still suitable |
|  | Transmitter spurious emissions | no change |
|  | Transmitter intermodulation | no change |
| Receiver | General | no change |
|  | Reference sensitivity level | minor change, new CBW should be added |
|  | Dynamic range | minor change, new CBW should be added |
|  | In-channel selectivity | minor change, new CBW should be added |
|  | Adjacent Channel Selectivity (ACS) and narrow-band blocking | minor change, new CBW should be added |
|  | Blocking | no change |
|  | Receiver spurious emissions | no change |
|  | Receiver intermodulation | minor change. The location of CW interfering signal shall be adjusted due to the new CBW. |

Proposal 3: It is proposed to study the corresponding BS RF requirements for support wide channel bandwidth.

For NR RE power control dynamic range is reused from LTE. RE power down requirement is limited by the EVM requirement as power de-boosting on some EPREs will lead to reduced Tx SNR which effectively requires a lower EVM requirement. When the requirement was set, 64 QAM is used as reference. Meanwhile considering that FR1 BS support 256 QAM or 1024 is already the baseline, gNBs with existing capability can support larger power reduction than those in the current 38.104 specification. For example, if considering 1024QAM as baseline, there would be about 10 dB potential extension is achievable. The RE power down extension would benefit the network power saving and reduction of co-channel interference.

Table 2.1-2: Existing RE power control dynamic range for NR/LTE

| Modulation scheme used | RE power control dynamic range (dB) |  |
| --- | --- | --- |
| on the RE | (down) | (up) |
| QPSK (PDCCH) | -6 | +4 |
| QPSK (PDSCH) | -6 | +3 |
| 16QAM (PDSCH) | -3 | +3 |
| 64QAM (PDSCH) | 0 | 0 |
| 256QAM (PDSCH) | 0 | 0 |
| 1024QAM (PDSCH) | 0 | 0 |
| NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station. |  |  |

10dB

Table 2.1-3: EVM requirements for different modulation scheme as in 38.104

| Modulation scheme for PDSCH | EVM Requirements (%) | SNR = -20*log(EVM) |
| --- | --- | --- |
| QPSK | 17.5% | 15.1 |
| 16QAM | 12.5% | 18.1 |
| 64QAM | 8% | 21.9 |
| 256QAM | 3.5% | 29.1 |
| 1024QAM | 2.5% | 32.0 |

Proposal 4: It is proposed to study the feasibility to extend BS RE power down dynamic range for FR1 bands.

2.2 RF requirements optimization for centimetre wave bands

- Tx

It is already the case that ACIR has been investigated for higher bands in the FR1 region. ACIR simulations consider both average throughput loss and 5%-tile loss. Existing AAS assumptions have shown that average power distribution results in similar results for both correlated and non-correlated performance, however higher correlated gain potentially could result in impact at the 5%-tile. Hence the impact of higher frequencies and potentially higher gain antennas needs to be assessed.

BS ACLR is related somewhat to the operating band unwanted emission (OBUE) and whilst ACLR is identified by R4 OBUE is based on a regulatory requirement. Currently ACLR is the stricter requirement. As frequency increases and ACLR is potentially lower it is possible OBUE will be the tougher of the 2 requirements and it may be necessary to discuss if it is suitable. OBUE is PSD so wider channel BW’s may be important (and potentially more complex adjacent channel interference models).

Proposal 5: for co-existence study for centimetre wave bands, it is proposed to study potential differences on the required ACIR due to large array and higher frequency effect and potentially to revisit ACLR and operating band unwanted emission.

- Rx

FR1 receiver requirements maintain links to the existing conducted requirements as such 2 sensitivity requirements were necessary to defined (Minimum sensitivity and reference sensitivity). FR2 has no equivalent conducted set of requirements so receiver requirements could be defined based on a single reference sensitivity. With the introduction of new high bands that will be based on AAS there is no need to maintain an equivalence to conducted requirements and a single OTA sensitivity can be used (based on minimum RX NF and antenna gain range assumptions). Interference requirements can then be derived based on this sensitivity declaration in a similar fashion to FR2.

Proposal 6: Since there are no existing non-AAS systems to maintain any equivalence with, it is proposed to study the possibility to remove the link to the existing conducted requirements. A similar approach as FR2 can be adopted, i.e. single declared sensitivity.

- Blocking

Conducted FR1 blocking specifies only the aggressor level without considering the victim system wanted signal level, and does so at very low probability (99.99%). In AAS system this leads to a difference in level depending on the assumptions of the receiver design i.e. if the interference is coherent or non-coherent, this makes deriving a requirement in this fashion without knowing the receiver architecture very difficult. By instead basing the requirement on the difference between the wanted signal and the blocking signal (at the victim) the correlation level does not affect the result and a single requirement regardless of system architecture and implementation) can be derived. Using a single sensitivity requirement and a wanted to interferer level blocking definition is more suitable for AAS systems in new bands.

Proposal 7: It is proposed to use the same methodology used for FR2 to derive a requirement, i.e. blocker vs wanted signal analysis.

- Co-location and co-existence

Co-location and co-existence (in same geographical area) requirements are currently based on worst case scenarios.

OTA Co-location is based on a horizontally co-located column antenna, as high band scenarios are defined it seems multi-column antennas are more likely and once introduced as the co-located system the interference scenario becomes more complex and potentially dependent on the steering direction of potentially both systems. If the scenario is updated therefore the derivation of the requirement lends itself to a statistical simulation-based approach to simulate a realistic dynamic scenario.

Co-existence (same geographical area) which is based on MCL scenario in existing requirements is also more complex when considering dynamic AAS systems. Statistical analysis currently being considered may be applied to new bands and the potential effect of high gain antennas on the requirements should be further investigated.

Proposal 8: it is proposed to study whether new requirement/metrics are adopted by considering higher frequency effect and in addition large array effect, pending on the output of Rel-20 BS RF WI.

Summary

In this paper we have identified the BS RF items under consideration for study in 6G including:

Potential optimization for existing FR1 bands

Proposal 1: the enhancement of BS type 1-H is focusing on large antenna array case for band above 5GHz.

Proposal 2: It is proposed to study the possibility to remove the protection of the BS receiver of own for FDD bands.

Proposal 3: It is proposed to study the corresponding BS RF requirements for support wide channel bandwidth.

Proposal 4: It is proposed to study the feasibility to extend BS RE power down dynamic range for FR1 bands.

RF requirements optimization for centimeter wave bands

- Tx

Proposal 5: for co-existence study for centimetre wave bands, it is proposed to study potential differences on the required ACIR due to large array and higher frequency effect and potentially to revisit ACLR and operating band unwanted emission.

- Rx

Proposal 6: Since there are no existing non-AAS systems to maintain any equivalence with, it is proposed to study the possibility to remove the link to the existing conducted requirements. A similar approach as FR2 can be adopted, i.e. single declared sensitivity.

- Blocking

Proposal 7: It is proposed to use the same methodology used for FR2 to derive a requirement, i.e. blocker vs wanted signal analysis.

- Co-location and co-existence

Proposal 8: it is proposed to study whether new requirement/metrics are adopted by considering higher frequency effect and in addition large array effect, pending on the output of Rel-20 BS RF WI.

References

[1] RP-251881, New SID: Study on 6G Radio

[2] RP-252904, Moderator's summary for RAN4 REL-20 5G-Adv: BS RF topics, Huawei

[3] RP-250284, Views on Rel-20 RAN4-led topics, Huawei, HiSilicon