3GPP TSG-RAN WG4 Meeting #116bis		   R4-2513319
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on 5G-6G MRSS

Source:	OPPO

Agenda item:	8.10

Document for:	Approval

- Introduction

In RAN #108 meeting, the study item on 6GR was approved in [1], the study aims to develop one non-backward compatible radio access technology (hereafter “6GR”) to meet a broad range of use cases. One of objectives is to study MRSS:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

  - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

  - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4]
NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study.

  - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

6G should use same numerologies as 5G especially for 5G-6G coexistence on same frequency.

In this contribution, we mainly discuss MRSS.

- Discussion

2.1 Operating frequency range for MRSS

Multi-RAT spectrum sharing (MRSS) is an important technique that allows MNO to operate different radio access technologies (RATs) on a same frequency spectrum (e.g., frequency bands, carriers or sub-bands) as a very cost-effective way to migrate users from an old RAT during a transition period to a new RAT.

To achieve smooth evolution and migration from 5G to 6G, MRSS should also be considered on those overlapping spectrums between 5G and 6G.

In 6G, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Compared 5G spectrum to 6G spectrum, 5G FR1 up to 7.125GHz and FR2-1 is overlapping with 6G spectrums as shown in Figure 1.

Figure 1 The overlapping spectrums between 5G and 6G

Since so far there is only a few countries deployed 5G in FR2 across the globe due to the limitation of the application scenarios.

Proposal 1:	5G-6G MRSS operation priority on FR1 up to 7.125GHz spectrum, FR2-1 (24.25 GHz – 52.6GHz) can be considered base on the interest of industry.

2.2 Co-existence issue

In 6G, one objective is to study 5G-6G Multi-RAT Spectrum Sharing for migration 5G-6G. therefore, 5G-6G need consider co-channel and adjacent channel co-existence.

Adjacent channel co-existence will study when define 6G RF requirements and guarantee by ACLR, so we just discuss co-channel co-existence in here.

Observation 1:	Adjacent channel co-existence between 5G and 6G will be guaranteed by ACLR, MRSS only need study co-channel existence.

In RAN1 #122 meeting, they discuss MRSS schemes, e.g.,

- NR/6GR resources are split via TDM/FDM, including rate-matching

- 6GR shares NR signals/channels

Since there is no any conclusion how the resource will share between NR and 6G, here analysis each case from RAN4 perspective.

Case 1: NR/6G resources sharing via TDM only

If NR/6G resources sharing via TDM, NR and 6G only share the time resource, they will occupy all frequency resource in whole carrier/channel separately, as shown in Figure 2:

Figure 2 NR/6G resources sharing via TDM

From both of UE side and BS side, no co-channel interference need consider since there is only one kind of signal at one time, only need consider the switching time between 5G configuration and 6G configuration.

Observation 2:	There is no co-existence issue for NR/6G resources sharing via TDM only, the switching time between 5G configuration and 6G configuration need be considered.

Case 2: NR/6G resources sharing via FDM, semi-statically

If NR/6G resources sharing via FDM, NR and 6G have fixed assigned frequency resource, and there have both of NR signal and 6G signal at one time in the carrier/channel as shown in Figure 3:

Figure 3 NR/6G resources sharing via FDM

From the UE side, regardless of a 6G UE or a 5G UE will receive both of 6G signal and 5G signal at the assigned MRSS carrier at one time.

If a 6G UE is allowed dual connection and is configured both of 5G frequency resource and 6G frequency resource, the 5G signal and 6G signal should keep orthogonality in frequency domain and to make the 6G UE could use one FFT to treat both of 5G signal and 6G signal.

If a 6G UE is only allowed to connect 6G when it is assigned MRSS carrier, the 5G signal shared spectrum with it is an interference, vice versa. To reduce the impact between different RAT on one MRSS carrier, 5G signal and 6G signal should also keep orthogonality in frequency domain.

Observation 3:	SCS alignment is necessary to avoid OFDM sub-carrier cross interference when 5G signal and 6G signal sharing frequency resource in one carrier/channel.

Case 3: NR/6G resources sharing via FDM, dynamically

If NR/6G resources sharing via both of TDD and FDM, they will dynamically share the time/frequency resource, but at one time, it still is a FDM mode sharing between NR signal and 6G signal, as shown in Figure 4:

Figure 4 NR/6G resources sharing dynamically

The mutual influence between 5G and 6G sharing the one carrier/channel also need avoid for NR/6G resources sharing dynamically. In addition, it also needs consider the switching time between 5G configuration and 6G configuration.

Observation 4:	NR/6G resources dynamically sharing not only need avoid OFDM sub-carrier cross interference when 5G signal and 6G signal share frequency resource in one carrier/channel, but also need consider the switching time.

Based above analysis, to avoid the mutual influence between 5G and 6G sharing the one carrier/channel, 5G signal and 6G signal need keep the orthogonality, therefore, 5G signal and 6G signal should adopt the same SCS.

Proposal 2:	For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel.

Proposal 3:	For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study.

In RAN#122 meeting, the following agreement was reached:

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

Compared to LTE-NR MRSS, 5G-6G MRSS need resolve some pain points that existing in the DSS design between LTE and NR:

- PDCCH capacity: In Rel-15, NR PDCCH cannot overlap with LTE PDCCH/CRS on the shared spectrum.

- Spectrum efficiency: RB-level and RE-level rate matching can be applied only to NR PDSCH to avoid overlapping with resources for LTE signals/channels.

How to result this pain points is RAN1’s work.  However, the issue of SSB conflicts needs to be taken into consideration for in-channel coexist between 5G and 6G.

As in 4G, the SSB is always at the center of CBW, to avoid conflicts, in NR the SSB can be configured within the CBW; CRS in LTE also spread in the CBW, in the MRSS, it needs to be punctured. In 5G, the CSI-RS is configurable.  Therefore, LTE SSB might interfere 5G data if overlap need to puncture these RBs.

In 5G, the SSB can be configured at different frequency points within the CBW, it is an always-on signal for all UEs. Therefore, 6G design should consider the coexist with 5G, and study how to avoid the interference from NR SSB.

Observation 5:	SSB is an always-on signal for all UEs in NR.

Proposal 4:	For MRSS operation, 6G design shall consider the in-channel coexist with 5G, and study how to avoid the interference from NR SSB.

2.3 Channel raster

6G design also need consider the coexist with 5G Within the spectrum that can be shared by 5G and 6G, most bands lower than 2700MHz used100kHz channel raster and those bands higher than 3300MHz used SCS based channel raster. Then in the case of Cell bandwidth and UE specific CBW is configured with ODD/EVEN or EVEN/ODD RBs then the alignment of the two centers is impossible for 100kHz channel raster. Therefore, the enhanced 10kHz channel raster was defined for NR operating bands with 100 kHz channel raster. Therefore, 6G channel raster design need consider the compatibility with all of these 5G channel rasters. No special channel raster issue need consider for MRSS.

Observation 6:	6G channel raster design need consider the compatibility with all of these 5G channel rasters.

Proposal 5:	For the 5G-6G MRSS bands, the 6G channel raster design shall make sure it is compatible with 5G. Besides of that, no other issue is foreseen in MRSS channel raster design.

- Conclusion

This paper discussed MRSS and proposed:

Operating frequency range for MRSS

Proposal 1:	5G-6G MRSS operation priority on FR1 up to 7.125GHz spectrum, FR2-1 (24.25 GHz – 52.6GHz) can be considered base on the interest of industry.

Co-existence issue

Observation 1:	Adjacent channel co-existence between 5G and 6G will be guaranteed by ACLR, MRSS only need study co-channel existence.

Observation 2:	There is no co-existence issue for NR/6G resources sharing via TDM only, the switching time between 5G configuration and 6G configuration need be considered.

Observation 3:	SCS alignment is necessary to avoid OFDM sub-carrier cross interference when 5G signal and 6G signal sharing frequency resource in one carrier/channel.

Observation 4:	NR/6G resources dynamically sharing not only need avoid OFDM sub-carrier cross interference when 5G signal and 6G signal share frequency resource in one carrier/channel, but also need consider the switching time.

Proposal 2:	For MRSS operation via FDD sharing, 6G should adopt the same SCS with 5G when sharing carrier/channel.

Proposal 3:	For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study.

Observation 5:	SSB is an always-on signal for all UEs in NR.

Proposal 4:	For MRSS operation, 6G design shall consider the in-channel coexist with 5G, and study how to avoid the interference from NR SSB.

Channel raster

Observation 6:	Channel raster for FR1 NR bands is defined based on 100kHz and SCS, and FDD bands with 100kHz channel raster can use enhance 10kHz.

Proposal 5:	For the 5G-6G MRSS bands, the 6G channel raster design shall make sure it is compatible with 5G. Besides of that, no other issue is foreseen in MRSS channel raster design.

- Reference

[1] RP-251881 New SID: Study on 6G Radio, NTT DOCOMO	RAN#108