3GPP TSG-RAN WG4 Meeting #116bis	R4-2513333

Prague, Czech Republic, October 13th - 17th, 2025

Source:	Nokia

Title:	RAN4 aspects for Sensing in 6G networks

Agenda item:	8.11

Document for:	Discussion

Introduction

During the last RAN4 meeting, the Chairman presented a summary of RAN4 topics in the 6G study item. Among them the sensing topic was listed, as specified in the SID [1]:

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

This contribution investigates RAN4 aspects for sensing in 6G networks based on the above objective in the SID.

Discussion

In this section, we discuss RAN4 relevant aspects for sensing in 6G networks.

Overview on sensing modes

In the Rel-19 study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, six sensing modes were considered: TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic [2]. Bistatic modes include multi-static modes according to the RAN1 study.

As shown in Figure 1, in the TRP monostatic scenario, the same BS acts as transmitter of the sensing signals and as receiver of the signals reflected from the target. Similarly in the UE monostatic sensing mode, the same UE acts as transmitter of the sensing signals and as receiver of the reflected sensing signals. In the TRP-TRP bistatic or multi-static sensing modes, a BS acts as transmitter of the sensing signals, while one or more other BSs act as receiver(s) of the signals reflected from the target. The same latter principle is used in the UE-UE bistatic or multi-static sensing modes, where a UE acts as transmitter of the sensing signals, while one or more other UEs act as receiver(s) of the signals reflected from the target. The last two sensing modes involving both the UE and the BS are TRP-UE and UE-TRP bistatic scenarios. A BS acts as transmitter of the sensing signals and one or more UEs receive the reflected sensing signals in the TRP-UE bistatic or multi-static sensing modes. A UE acts as transmitter of the sensing signals and one or more BSs receive the sensing signals reflected from the target in the UE-TRP bistatic or multi-static sensing modes.

Figure 1: Overview of the six sensing modes.

For the 6G study on sensing, RAN4 can consider these sensing modes when discussing measurements, requirements, and testability aspects. In case there will be updates from RAN1 during the study regarding the sensing modes (e.g., deprioritized scenarios and/or new modes introduced), RAN4 can take such updates into account.

RAN4 to consider the six sensing modes (i.e., TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic) at the start of the 6G study. Further updates on the considered sensing modes can be made based on input from RAN1.

Measurement and timing related procedures

Measurement and timing related procedures for 6G sensing are considered hereafter. Depending on the use cases, subject to be defined by RAN1, which may be aligned to those defined for NR sensing /ISAC in TR 38.914, measurement and timing requirements need to be studied for 6G sensing, both in terms of measurement latency and measurement accuracy.

Measurements and reporting

6G sensing should be based on the measurement and reporting framework designed for 6G. As this framework is yet being investigated as part of the ongoing 6G study item, a starting point for the 6G sensing study could be the reuse of NR framework for measurement and reporting, which is foreseen to be used for sensing / ISAC in NR.

For instance, the NR PRS framework for measurement and reporting or the NR TRS framework for measurement and reporting may be assumed starting point for evaluation of measurement and reporting delays.

RAN4 should study the framework for measurement and reporting for 6G sensing based on the framework defined for sensing / ISAC in NR.

On the other hand, the framework for measurement and reporting in NR is based on gap-assisted and gapless measurements, which both are subject to be investigated for 6G RRM procedures. Furthermore, periodic and aperiodic measurement procedures need to be considered.

Furthermore, measurements and reporting for 6G sensing should be studied for idle, inactive and connected RRC mode.

RAN4 should study measurement and reporting for 6G sensing for gap-assisted and gapless measurements, both for periodic and aperiodic measurement procedures, in RRC idle, RRC inactive and RRC connected mode.

Timing requirements

In 6G sensing, timing requirements are critical to ensure that sensing measurements do not interfere with communication functions or other sensing operations. The interruption caused by sensing must be minimized, especially in scenarios where high reliability and low latency are essential, such as UAV detection in urban macro deployments or AGV navigation in indoor factories. Sensing latency requirements vary across use cases, for example, human detection demands latency as low as 0.5ms, while UAV and vehicle detection allow up to 2 ms. These constraints imply that sensing operations must be carefully scheduled to avoid collisions with other measurements and transmissions. RAN4 must consider not only the latency but also the delay and interruption budgets when defining requirements, ensuring that sensing can coexist with communication without degrading performance.

Timing requirements for 6G sensing are tightly coupled with latency and interruption constraints, which vary significantly across use cases and must be managed to avoid collisions with other operations.

Sensing scenarios in 6G are diverse, ranging from outdoor UAV tracking to indoor human respiration monitoring, each with unique timing and accuracy demands. For instance, micro-Doppler use cases like blade rotation or human respiration require high-frequency resolution and low latency sensing, while vehicular detection in urban grids emphasizes positioning accuracy and robustness. These variations necessitate flexible timing configurations and adaptive delay handling mechanisms. RAN4 will need to account for these differences by defining scenario-specific timing profiles and ensuring that sensing measurements are synchronized with network operations. Moreover, the coexistence of multiple sensing modes, such as TRP monostatic, TRP-UE bistatic, and UE-UE bistatic, will add complexity to timing coordination, especially when UEs are involved as sensing nodes.

The diversity of sensing scenarios in 6G demands flexible and adaptive timing requirements, with RAN4 needing to tailor specifications to each use case and sensing mode.

Depending on use cases and sensing mode, the timing requirements in RAN4 are dependent on RAN1’s outcome including the signal structures, waveform, channel models, measurement configurations, and procedures for sensing transmission, reception, resource allocation, and/or synchronization mechanisms.

RAN4 should study the RAN4 impact on timing and delay requirements once RAN1 makes sufficient agreements.

RAN4 will investigate minimum requirements for sensing. However, we should also ensure that UE which performs better than minimum is also supported by the specification in order to facilitate getting the system benefit from such implementation. In particular, such UEs may distinguish from other UEs by satisfying more stringent timing requirements.

RAN4 to investigate minimum timing requirements for 6G sensing and also whether UE can support more stringent timing requirements.

RF impacts

Based on the progress in RAN1, there are some concerns that may impact the RF specification in RAN4.

- The DL waveform, i.e., DFT-S-OFDM, which may be used by UE to transmit the sensing signals. Since only CP-ODFM signal is defined in the current specification for UL, then, if UE need to use DFT-S-OFDM for sensing, the ACLR, SEM, etc. RF specification needs to be re-visited.

- UE needs to do Tx and Rx at the same time for sensing since the reflected signals can come back to UE at any time depending on the distance of the objects from UE. Therefore, the Tx and Rx power separation in full-duplex mode needs to be analysed. RAN1 is expecting 65-80 dB separation/isolation and we need to verify if that is reasonable.

- RAN1 may propose to use higher than 31 dBm signals in the sensing case. Since the highest maximum power of UE types in RAN4 is PC1 at 31 dBm, therefore, higher than 31 dBm output power will need to be analysed in terms of co-existence, ACLR, SEM, etc.

The concerns from RAN1 related to UE using DL waveform, Tx and Rx separation for full-duplex mode, and higher than 31 dBm maximum output power will impact RF.

A challenge in detecting/localizing/tracking ground targets is the environment [RP-251989]. The line-of-sight link between base stations serving as transmitter and receiver and the target may often be blocked by objects in the environment, such as buildings. To address this challenge, we can utilize a radar technique, such as sync radars, to transmit sensing signals simultaneously. Also, to improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate, we can consider modifying radar technology “chirping” in the frequency domain based on LTE and 5G technologies to be used in 3GPP.

Reusing radar technologies can help to solve the issues related to detecting/localizing/tracking ground targets, improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate.

In general, we propose to keep using the 5MHz, 10MHz, 15MHz, or 20MHz bandwidth, which is compatible to current LTE and NR OFDM signals. When reusing radar technologies, we try to reuse the same technologies used in LTE and NR to minimize the impact to the waveform, ACLR, and other RF specifications.

When adopting the benefits of other technologies, such as radar technologies, into 3GPP domain to solve sensing issues mentioned in RAN and RAN1, consider adjusting them to be LTE/NR compatible to reduce the workload and reduce the impact to RF.

RAN4 should study the RAN4 impact on RF requirements once RAN1 makes sufficient agreements.

Coexistence aspects

The SID also mentions coexistence in the responsibility of RAN4. While coexistence in RAN4 usually is regarded as coexistence of systems in adjacent spectrum, there is also the aspect related to coexistence of sensing with communication in the same spectrum.

As mentioned in e.g. RP-252174, the latter type of coexistence refers to efficient resource utilization, including resource allocation and signal design, to achieve the required sensing performance for the served use case(s) while the same spectrum is also serving 6G communication.

Therefore, we propose to distinguish between

- RF coexistence of sensing with other RATs in adjacent spectrum and

- Service coexistence of sensing with integrated communication or with concurrent positioning procedures in the 6G RAT in the same spectrum.

RAN4 should distinguish between i) RF coexistence of sensing with other RATs in adjacent spectrum and ii) service coexistence of sensing with communication or positioning procedures in 6G RAT in the same spectrum.

The RF co-existence study will be needed if the waveform, bandwidth, and power of sensing signal is different from current OFDM signals in the standard. Therefore, if there is no change for those, then no impact for RF co-existence.

However, RAN1 mentioned that UE may use a higher output power than PC1. In this case, the RF co-existence needs to be studied since a UE with higher output power than PC1 (31 dBm) has not been introduced in RAN4 spec so far.

In our view, RF coexistence of sensing with other RATs in adjacent spectrum is of higher system relevance and hence should be studied with first priority. Thereafter, service coexistence aspects, as mentioned above, should be investigated.

RAN4 to focus on sensing scenarios without communication or concurrent positioning procedures with first priority.

UE energy saving

Different sensing scenarios will be considered in the study together with different sensing modes as depicted in section 2.1. Optimization is needed regarding UE energy saving for transmission and reception of reference signals used both for sensing and positioning.

Therefore, RAN4 should consider procedures/requirements to support UE energy saving. This includes existing energy saving methods during sensing operation. Energy saving benefits may be larger for some sensing modes than for others.

RAN4 to investigate UE energy saving aspects when sensing is activated for the different sensing modes.

Testability

The integration of sensing into 6G systems is expected to support a wide range of use cases such as UAV detection, human respiration monitoring, AGV navigation, and smart transportation. These applications demand high accuracy, low latency, and robust performance under diverse conditions. To ensure the 6G system meets these expectations, it is essential to develop a comprehensive testing framework that evaluates sensing capabilities across scenarios. Testing will provide insights into the minimum achievable sensing performance and help validate system reliability. RAN4 must study performance metrics such as positioning accuracy, micro-Doppler accuracy, missed detection probability, and sensing latency. These metrics are foundational to defining the testing framework. RAN4 should also assess whether existing positioning testing frameworks can serve as a baseline. However, this reuse depends on the measurement types, procedures, and reporting mechanisms that RAN1 must first define.

A testing framework is essential for validating 6G sensing performance, and its development in RAN4 depends on foundational definitions from RAN1.

Defining the testing framework for 6G sensing requires careful consideration of the Device Under Test (DUT) and Test Equipment (TE) configurations. Each sensing scenario, whether involving UAV tracking, human motion detection, or AGV navigation, will require the UE to perform and report different types of measurements. These measurements may include micro-Doppler shifts, positioning estimates, or velocity tracking. The testing framework must be differentiated to reflect these variations, ensuring that the DUT is properly configured and the TE can simulate realistic environments and signal conditions. Existing test setups in TS 38.133 already take into account multi-cell configurations, timing alignment procedures, and measurement gap handling, which can inform future sensing test designs. RAN4 will need to extend these principles to accommodate sensing-specific requirements, such as bistatic and monostatic sensing modes, and ensure accurate emulation of real-world deployment scenarios.

Scenario-specific DUT and TE configurations are critical for accurate sensing evaluation, and existing test setups, e.g., positioning testing, can offer a foundation for future 6G sensing tests.

In the current phase, RAN4’s scope for 6G sensing is limited to areas that do not overlap with RAN1 or RAN2. This implies that RAN4 may wait for RAN1 to define the signal structures, waveform, channel models, measurement types, and procedures before it can proceed with detailed testing specifications. RAN1 is expected to define sensing modes such as TRP monostatic, TRP-UE bistatic, and UE monostatic, along with corresponding measurement procedures for micro-Doppler, positioning, and velocity estimation. Once these are established, RAN4 can begin developing test cases, performance benchmarks, and validation methods. This dependency highlights the importance of coordination across working groups to ensure timely and coherent development of 6G sensing standards.

RAN4’s progress on sensing testing is contingent on RAN1’s definition of signal, channel, and measurement types/procedures, requiring close inter-WG collaboration.

RAN4 can start the study with the testability aspects that are independent from RAN1 scope.

Conclusion

This contribution investigated RAN4 aspects for sensing in 6G networks based on the objective in the SID [1]. The following Observations and Proposals were made:

- RAN4 to consider the six sensing modes (i.e., TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic) at the start of the 6G study. Further updates on the considered sensing modes can be made based on input from RAN1.

RAN4 should study the framework for measurement and reporting for 6G sensing based on the framework defined for sensing / ISAC in NR.

RAN4 should study measurement and reporting for 6G sensing for gap-assisted and gapless measurements, both for periodic and aperiodic measurement procedures, in RRC idle, RRC inactive and RRC connected mode.

- Timing requirements for 6G sensing are tightly coupled with latency and interruption constraints, which vary significantly across use cases and must be managed to avoid collisions with other operations.

The diversity of sensing scenarios in 6G demands flexible and adaptive timing requirements, with RAN4 needing to tailor specifications to each use case and sensing mode.

Depending on use cases and sensing mode, the timing requirements in RAN4 are dependent on RAN1’s outcome including the signal structures, waveform, channel models, measurement configurations, and procedures for sensing transmission, reception, resource allocation, and/or synchronization mechanisms.

RAN4 should study the RAN4 impact on timing and delay requirements once RAN1 makes sufficient agreements.

RAN4 to investigate minimum timing requirements for 6G sensing and also whether UE can support more stringent timing requirements.

The concerns from RAN1 related to UE using DL waveform, Tx and Rx separation for full-duplex mode, and higher than 31 dBm maximum output power will impact RF.

Reusing radar technologies can help to solve the issues related to detecting/localizing/tracking ground targets, improve sensing accuracy, increase the accuracy of positioning estimate, and increase the accuracy of velocity estimate.

When adopting the benefits of other technologies, such as radar technologies, into 3GPP domain to solve sensing issues mentioned in RAN and RAN1, consider adjusting them to be LTE/NR compatible to reduce the workload and reduce the impact to RF.

RAN4 should study the RAN4 impact on RF requirements once RAN1 makes sufficient agreements.

RAN4 should distinguish between i) RF coexistence of sensing with other RATs in adjacent spectrum and ii) service coexistence of sensing with communication or positioning procedures in 6G RAT in the same spectrum.

RAN4 to focus on sensing scenarios without communication or concurrent positioning procedures with first priority.

RAN4 to investigate UE energy saving aspects when sensing is activated for the different sensing modes.

A testing framework is essential for validating 6G sensing performance, and its development in RAN4 depends on foundational definitions from RAN1.

Scenario-specific DUT and TE configurations are critical for accurate sensing evaluation, and existing test setups, e.g., positioning testing, can offer a foundation for future 6G sensing tests.

RAN4’s progress on sensing testing is contingent on RAN1’s definition of signal, channel, and measurement types/procedures, requiring close inter-WG collaboration.

RAN4 can start the study with the testability aspects that are independent from RAN1 scope.

- References

- RP-2501881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

- RP-240799, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T.

- 3GPP TR 22.870 v0.3.1 “Study on 6G Use Cases and Service Requirements”.

- 3GPP TR 22.837 v19.4.0 “Study on Integrated Sensing and Communication”.

- 3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements (Release 20)”.