3GPP TSG-RAN WG1 Meeting #116bis	R4-2513343

Prague, Czech Republic, October 13th – 17th 2025

Agenda Item:	8.10

Source:	Ericsson

Title:	Considerations on spectrum sharing for 6G

Document for:	Discussion

# 1	Introduction

At RAN4#116bis, work will commence in RAN4 on the RAN WG 6G SI. To facilitate the discussion, the chairman has allocated a number of threads for discussion as follows:

This contribution addresses the thread on spectrum sharing.

6G will be deployed widely in spectrum that is currently used for other IMT RATs, in particular 4G and 5G. Since operators provide a service to existing users with the legacy RATs, they will need to continue to allocate large amounts of this spectrum to legacy RATs well into the lifetime of 3GPP in order not to degrade the user experience of existing users. At the same time, when deploying 6G, they will need to provide sufficient spectrum that the 6G user experience is satisfying. Since the amount of spectrum available will not double, it will be necessary for users to use the same spectrum; i.e. the same carriers in the same bands for legacy RATs and 6G; at least for 5G and 6G.

It is not yet clear whether there might be any interworking between 5G and 6G. Any need for 5G/6G DC is subject to further RAN discussion and currently not in the scope of the SI. There will, though, be a need for at least 5G and 6G to share time and frequency resources, via a mechanism currently known as MRSS.

The details of how MRSS will work will depend on factors such as the design of synchronization signals, reference signals and broadcast channel and the mobility framework that are at this stage still open. For sharing time and frequency resources though, it is fairly obvious that the time/frequency grid needs to be compatible between 5G and 6G. In principle, the numerology, symbol length etc. should be at least compatible and in principle the same.

During the discussion on system parameters, take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS

A further consideration is that, if a carrier is used for both 5G and 6G then the basestation will need to transmit both types of carrier. To facilitate BS design, this implies that there should be compatibility for the RF requirements for 5G and 6G. This may not mean that the RF requirements are the same, since some 6G improvements may imply new or different RF requirements.

In terms of impact to the different types of RAN4 work, at this stage an assessment may be as follows:

UE RF:

Apart from the system parameter related considerations described above, it is not likely that MRSS will significantly impact UE RF requirements. There may be some need to ensure that the RF requirements can enable compatibility between 6GR UEs and legacy UEs operating in the same band and in the same carrier.

When designing UE RF requirements, take into account that 6GR UEs will operate on the same carrier as legacy UEs due to MRSS.

BS RF:

As described above, the BS will need to be able to transmit 6GR and 5G within the same carrier. This needs consistency between the numerologies and slot formats. The BS requirements should also be consistent and facilitate a multi-standard 6G/5G BS. The requirements may not be exactly the same, since 6G requirements should facilitate improved performance in the same RAT, but they should be compatible.

When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs.

RRM:

In legacy NR, dynamic sharing with 4G technologies is in general more cumbersome given the relatively large amount of always-on signals in 4G. Especially, in that time, RAN4 only focused on the traditional inter-RAT handover requirement. To allow a dynamic sharing of resources between 5G and 6G in MRSS, the related signal, measurements, and procedure design should be introduced, supported by the corresponding RAN4 RRM requirements. However, the detailed requirements discussions should wait for other WGs.

RAN4 should study MRSS based measurement and handover requirements if needed, taking into account the RAN1/2 design of MRSS.

Demodulation:

For 5G, DSS demodulation requirements took into account puncturing of PDSCH due to LTE CRS. There may potentially be a need for 6G MRSS related demodulation requirements, however more details on the design of SSB, RS and MRSS are required in order to decide on the need for demodulation requirements for MRSS.

Consider demodulation requirements for MRSS once the RAN1/2 design is clearer.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	During the discussion on system parameters, take into account that alignment between 5G and 6G slot length and numerology will facilitate MRSS

Proposal 2	When designing UE RF requirements, take into account that 6GR UEs will operate on the same carrier as legacy UEs due to MRSS.

Proposal 3	When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs.

Proposal 4	RAN4 should study MRSS based measurement and handover requirements if needed, taking into account the RAN1/2 design of MRSS.

Proposal 5	Consider demodulation requirements for MRSS once the RAN1/2 design is clearer.