3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513346

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	8.13

Source:	Ericsson

Title:	RAN4 operation efficiency

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the RAN4 operation efficiency according to the agenda guidance.

# 2	Discussions

The release independent specification, the coexisting framework and the specification improvement on both RF and RRM are discussed in below sections.

## 2.1	Common RAN4 topics

## 2.1.1	On release-independency feature

MCC points out the how RAN4 should handle the release independent feature in 38.307 in RAN contribution [RP-240905] and this should be clarified further in day 1 in 6G. RAN4 has added many new aspects for release independent since the first introduction of the release independent specification in UMTS time. At beginning only bands are in specification, then other features added, e.g CA combination, channel bandwidth, power class, band for different feature etc. in particular, there are some “release independent” feature which we think may not belong to 3x.307. In our opinion, these “release independent" features should be handled in RAN2 in TS 38.331 instead of RAN4 38.307. RAN2 has “early implementation” concept for release independent from Rel-N and has a rule that no interoperability issue "Implementation of this CR from Rel-N will not cause interoperability issues". Any early implementation CR agreed is tracked in annex C in TS 38.331.

Observation 1: Release independent from Rel-N in RAN2 is defined as "Implementation of this CR from Rel-N” and RAN2 also has a rule of “not cause interoperability issues”.

Below we lists some example in 38.307 to illustrate our view.

Example 1: UL 7.5KHz shift for band n40 in 38.307 and quoted below. RAN2 has CR (R2-2011066) for early implementation according to RAN4 LS.

Table 5.3-2: UL 7.5KHz shift for band n40 in FR1

| Feature | DL/UL | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the configuration was introduced) |
| --- | --- | --- | --- | --- |
| 7.5KHz UL shift for band n40 in FR1 | UL | TDD | Rel-15 | Table B.4.7-1 |

Example 2: High speed train and quoted below. RAN4 sent LS (R4-1915916) to RAN2 and RAN2 CR (R2-2006188 ) for early implementation.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |

Example 3: DMRS bundling for GSO scenario. The feature is specified in RAN1 in Rel-17 while RAN4 add the core requirement in Rel-18. This may not be in line with MCC expectation.

“F.	Where can the REL-independent approach not be used?

The REL-independent approach is strictly limited to bands and band combinations, it is not a mechanism to ignore 	the REL-freeze by squeezing whatever late features into frozen releases.

The REL-independent approach shall also not be misused to shift RAN4 work (for which there is currently no time 	available) into the next REL but still squeeze in the corresponding RAN1/2/3 work into the current REL because 	acc. to 3GPP procedures the incomplete feature (RAN1/2/3 work) has to be removed from the current REL at the 	REL-freeze as it cannot work without missing RAN4 part and RAN5 can not test incomplete features.”

Table 9.2-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| DMRS bundling for GSO scenario | Rel-17 | Table F.1-4 | Rel-18 WI NR_NTN_enh introduced phase continuity requirements for DMRS bundling for the GSO scenario: see Table C.1-1 |

Observation 2: Some “release independent” aspects in TS 38.307 overlap with early implementation of CR in RAN2 TS 38.331.

Proposal 1: Use RAN2 release independent from Rel-N with early implementation concept for “release independent” feature instead of the 3x.307 if such feature has other working group impact, e.g signalling in RAN2. Following the MCC guidance on release independent handling in RAN4, only allow the band related feature in 3x.307.

## 2.1.2	On coexisting frame work

At the beginning of 5G , there was no adjacent channel coexisting study done in FR1 (reusing the LTE coexisting) but only FR2. The coexistence study is documented in TR 38.803 for FR2. When adding the NTN feature, the new coexistence assumption is done in TR 38.863. In this TR, both 36.942 and TR 38.901 are referenced for some of the coexistence assumption parameters. For AAS BS antenna model, the TR 38.921 is referenced. In TR 38.921, the coexistence study is done for the study item on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz. The coexistence simulation work and modelling of the antenna is distributed in several RAN4 study (NTN, IMT parameter). In A-IoT study, the study is documented in TR 38.769. All the coexistence assumptions are agreed within each feature and there is no common guideline on some other assumptions and parameter. In 6G, the result of previous co-existence studies should be re-used when relevant to avoid excessive work while for some new aspects and features additional coexistence study will be highly likely needed and there is a need to agree and summarize the coexistence simulation parameters and assumptions e.g. different deployment scenario, different antenna modelling etc, one example is listed in Table 1. This would increase the efficiency of RAN4 as during 6G era, the discussion to settle simulation assumptions and parameters every time such study is needed could be avoided.

In order to improve the operational efficiency, it would be good to agree on a coexistence framework to be the basis of the coexistence studies that will be required during 6G. This would save online discussion time, allowing reusing previous relevant work, allowing for easy traceability of assumptions, reducing copy-pasting etc. As a relevant previous work, during Rel. 19, RAN4 worked towards TR 38.922, in the context of the Study Item on IMT parameters for the 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz.  TR 38.922 could be taken as a basis for this coexistence framework for different assumptions that were considered valid for different frequency ranges, including the cmWave range and another range between 1710 – 4990 MHz.

Relevant parameters that could be agreed in a coexistence framework are:

- Typical deployments and ISD

- Typical antenna architectures for different frequency ranges, for different BS classes; subarray and antenna element models.

- For the channel model RAN4 should in general refer to TR 38.901, which is well updated and maintained.

- Maximum BS and UE transmission power.

- UL power control algorithm

- Noise figures of BS and UE for different bands of interest.

- Link SNR to System Mapping

Observation 3: the Coexisting work for each feature in 5G is agreed within the feature study/working item phase and no generic guideline followed.

Proposal 2: Start 6G coexisting framework for all features with common assumptions to save future work load in feature development.

Table 1: coexisting simulation scenario summary

| RAT | TN/NTN | scenario | Frequency range | Typical frequency | Reference |
| --- | --- | --- | --- | --- | --- |
| NR | TN | Urban, Rural | FR1 | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) | TR 36.942 TR 38.863 (TN part) |
|  |  | Indoor, urban | FR2 | 28 GHz | TR 38.803 |
|  |  | Urban, dense Urban, indoor | 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz |  | TR 38.921 |
|  |  | Urban, dense Urban, indoor | 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz |  | TR 38.922 |
|  | NTN | Urban macro, rural for FR1, Urban for FR2 | FR1, FR2, Ku bands | 2GHz FR1, 17G/27GHz FR2 | TR 38.863 |

## 2.2	RF aspect of specification improvement and modernization

There is a new study item [1] to investigate the possibilities of the improving the specification and the TR 21.802 is outcome of this study item.

“The start of a new Generation provides a unique opportunity to explore potential changes to the format of 3GPP specifications, and the processes for specification version control and CRs. With 6G Study Items now underway, it is crucial to explore potential improvements in specification development before the 6G Work Item phase begins, to ensure that the most reliable, modern and efficient approach can be used for the next generation of mobile technology.

Potential improvements include, but are not limited to:

- Elimination of manual errors between CR approval and implementation of CRs in the specifications;

- Immediate availability of new specification versions at the end of each TSG plenary;

- Enhanced navigation, including cross-linking and referencing, across entire body of 6G specifications;

- Elimination of latency and stability issues;

- Easier automation of processing the content of the specifications”

Proposal 3: companies follow the SI [1] for specification modernization.

For 5G, RAN4 specified 3 specifications in RF, namely 38.101-1/2/3. The clause numbering of the requirements at the specification was aligned, e.g. Clause 6 for transmit characteristics, so RAN5 conformance test specification clause numbering could be aligned with RAN4 for all frequency ranges. This should also be used for 6GR.

Proposal 4: the clause numbering in RAN4 specification of 6GR for different FR range should be aligned.

When the 7GHz , 8 GHz and 15GHz frequency range is to be specified in 6G, it should be discussed if the conducted specification or OTA specification should be used.

Proposal 5: discuss which specification is used to specify the 7GHz, 8GHz and 15GHz.

The suffix structure of the 5G specification can be reused in 6G. However, the requirement relation between general requirement and suffix requirement is stated in clause 4:

“A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to L) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to L) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.”

The rule of suffix requirement in above clarification may need to consider more cases in 5G specification. For example, in 38.101-1 there are configured output power for CA in clause 6.2A.4 and the general clause 6.2.4.  In general clause 6.2.4, there is a ΔTIB,c in Pcmax,c formular which pointing to the clause 6.2A.4:

“∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA”

This example is not the case of the “tighter requirements” as the requirement are different for general and suffix requirements (i.e the general is Pcmax,fc and suffix requirement for total aggregated power from all carriers, Pcmax) and for CA feature, both general requirement and suffix requirement apply. When configured with CA the UE configures a maximum power Pcmax,fc per serving cell and a total aggregated power Pcmax at power prioritization according to 38.213 occurs when the UE is power limited. However, it is not easy to understand the UE power configuration behaviour as the CA suffix requirement specify the total aggregated power Pcmax from different CC and general requirement specify the signal CC output power, it is not clear to reader if the single CC power PCMAX,c would be capped when the aggregated power from each Pcmax,c exceeds the total power PCMAX. And if so, which carrier power PCMAX,c would be reduced?

Proposal 6: In case to specify the feature requirements at both general clause and suffix clause, and the requirement in general clause and suffix clause relating to each other, it is preferable to state which requirement should be met in what condition.

In 38.101-1, the different wording is used for a UE to do sth, it is summarized below:

| Next word after “UE is” | example | comments |
| --- | --- | --- |
| configured | “UE is configured” 53 occurrence | Most used, could be interpreted with RRC configuration |
| scheduled | “UE is scheduled” 32 occurrence | Most used in context of DCI scheduling from network, can be MAC signaling (MAC-CE) for e.g. ending a semi-persistent scheduling |
| Configured or scheduled | “Configured or scheduled” 2 occurrence | Either RRC or DCI, or semi-persistent scheduling |
| associated | “UE is associated” 3 occurrence | Only used in “UE is associated with a serving cell on PS carrier”, could be inter-changeable with “configured”? |
| allowed | “UE is allowed” 17 occurrence | “UE is allowed to set “ or “UE is allowed to reduce”, should follow the 21.801 drafting rule, Annex E, for permission, “may” is used and equivalent expressions for use in exceptional cases “is permitted is allowed is permissible” |
| required | “UE is required” 5 occurrence | Most used with the number of Rx antenna. “ATG UE is required optionally to be equipped with four Rx antenna ports.” should follow the 21.801 drafting rule, Annex E, use “shall” or when it related to UE capability, rewording to “if the UE indicates support of “UE capability”, it shall meet the [RAN4] requirements of [suffix or general requirement] |
| capable | “UE is capable” 2 occurrence | should follow the 21.801 drafting rule, Annex E, use “can” |
| within | “ UE is within” 4 occurrence | Only used as “UE is within the protected zone”, no ambuiguity |

For another example on consistent wording, the “antenna”, “antenna connector”(275 occurrence) and “antenna ports”(167 occurrence) are all used in requirements. the specification quality could be improved if the wording is consistent and even better if such terminology is defined in clause 3. In the clause of receiver requirements, it seems the “antenna port” and “antenna connector” can be inter-changeable, for example:

“7.2	 Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports.”

And

“Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).”

But in transmitter requirement, sometimes the “antenna port” is refereeing to antenna ports logically associated with specific physical channels/signals used in RAN1 specification, for example:

“If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2F.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.”

And below in the conext of single layer or two-layer

“The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 as specified in [38.306].”

It is less obvious if the layer information is not mentioned and both antenna connector and antenna port are used at the same time.

“In Figure 6.3A.3.3.6-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports.”

Observation 4: the wording in RF specification is not consistent in different requirement in 5G specification

Proposal 7: the wording consistency can be improved with drafting rules and clearly defined terminology.

## 3.2	RRM aspect of specification improvement and modernization

### Relation between requirements and test cases

The issue of unclear or non-straightforward relation between requirements and test cases has also been discussed under the specification improvement study for NR. As a possible solution to the issue, it was proposed to introduce a mapping table between requirements and test cases. It was realized, whoever, that such a table is expected to be very long, and also maintaining and keeping always up to date such a table will most likely require extra efforts and increase the number of CRs. Another approach could be to include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

Proposal 8: Include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

### 3.2.2	Specification structure for test cases

To make it easier and more intuitive finding test cases of interest, it can be beneficial to have the specification structure for test cases similar to the requirements structure in the main part of the specification, at least at the top level (or maybe even at the second level), e.g.:

Test cases part:

- RRC_IDLE state mobility

- RRC_INACTIVE state mobility

- RRC_CONNECTED state mobility

- Timing

- Signalling characteristics

- Measurement procedure

- Measurement performance requirements

Proposal 9: Specification structure for 6G test cases is similar to the requirements structure in the main part of the specification, at least at the top level (or maybe even at the second level), e.g.:

-	RRC_IDLE state mobility

-	RRC_INACTIVE state mobility

-	RRC_CONNECTED state mobility

-	Timing

-	Signalling characteristics

-	Measurement procedure

-	Measurement performance requirements.

Furthermore, in the test cases part of the TS 38.133 we also have some use cases at the top level, e.g., ATG, RedCap, positioning, etc. On one hand, this simplifies finding e.g. the positioning and NB-IoT test cases, but on the other hand this breaks the mapping between requirements section and test cases specification structure...

Proposal 10: Test cases for specific applications or use cases can be in a separate section, but without breaking the main structure of test cases and the mapping between the core requirements sections and sections with test cases, e.g.:

- Option 1: uses cases are added as separate sections after the main test cases hierarchy;

- Option 2: uses cases are grouped at a second level of the test cases hierarchy

### 3.2.3	Common configurations

We observed that many configuration parameters in test cases are repeated. It may simplify the specification if more such common configurations can be identified and collected in a common section, e.g., under A.3 (RRM test configurations) or similar.

Proposal 11: If common configurations can be identified for different test cases, they can be collected in a common section, e.g., under A.3 (RRM test configurations) or similar.

### 3.2.4	Common configurations

A RAN discussion has taken place in March 2025 on the principles for specification work [3]. Such principle can be very well adapted also in RAN4.

Proposal 12: Adapt an overall principle for 3GPP RAN4 specifications - creating lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified.

# 4	Summary

Observations:

Observation 1: Release independent from Rel-N in RAN2 is defined as "Implementation of this CR from Rel-N” and RAN2 also has a rule of “not cause interoperability issues”.

Observation 2: Some “release independent” aspects in TS 38.307 overlap with early implementation of CR in RAN2 TS 38.331.

Observation 3: the Coexisting work for each feature in 5G is agreed within the feature study/working item phase and no generic guideline followed.

Observation 4: the wording in RF specification is not consistent in different requirement in 5G specification

Proposals:

Proposal 1: Use RAN2 release independent from Rel-N with early implementation concept for “release independent” feature instead of the 3x.307 if such feature has other working group impact, e.g signaling in RAN2.Following the MCC guidance on release independent handling in RAN4, only allow the band related feature in 3x.307.

Proposal 2: Start 6G coexisting framework for all features with common assumptions to save future work load in feature development.

Proposal 3: companies follow the SI [1] for specification modernization.

Proposal 4: the clause numbering in RAN4 specification of 6GR for different FR range should be aligned.

Proposal 5: discuss which specification is used to specify the 7GHz, 8GHz and 15GHz.

Proposal 6: In case to specify the feature requirements at both general clause and suffix clause, and the requirement in general clause and suffix clause relating to each other, it is preferable to state which requirement should be met in what condition.

Proposal 7: the wording consistency can be improved with drafting rules and clearly defined terminology.

Proposal 8: Include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

Proposal 9: Specification structure for 6G test cases is similar to the requirements structure in the main part of the specification, at least at the top level (or maybe even at the second level), e.g.:

-	RRC_IDLE state mobility

-	RRC_INACTIVE state mobility

-	RRC_CONNECTED state mobility

-	Timing

-	Signalling characteristics

-	Measurement procedure

-	Measurement performance requirements.

Proposal 10: Test cases for specific applications or use cases can be in a separate section, but without breaking the main structure of test cases and the mapping between the core requirements sections and sections with test cases, e.g.:

- Option 1: uses cases are added as separate sections after the main test cases hierarchy;

- Option 2: uses cases are grouped at a second level of the test cases hierarchy

Proposal 11: If common configurations can be identified for different test cases, they can be collected in a common section, e.g., under A.3 (RRM test configurations) or similar.

Proposal 12: Adapt an overall principle for 3GPP RAN4 specifications - creating lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified.

# References

- SP-250802; Study on Modernization of Specification Format and Procedures for 6G

- RP-250766, Lean and Streamlined 6G Standards, Apple, AT&T, BT, CMCC, DT, Ericsson, Huawei, Intel, KT, MediaTek, Nokia, NTT Docomo, Qualcomm, Reliance Jio, Samsung, Spark NZ, T-Mobile USA, Telstra, Verizon, Vodafone, ZTE, March 2025.