# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513367

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.13.3.2

Source:	China Telecom

Title:	Discussion on RRM performance requirements for RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements for RRM impacts for SBFD operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for RRM impacts for SBFD operation captured in the WF [1] and summary [2].

- Discussion

Test cases

Status in the summary [2]:

Issue 3-3-2: Test case list - Proposals Samsung; Nokia; CTC; CMCC; E///; HW; ZTE; MTK. L1-SRS-RSRP measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. L1-CLI-RSSI measurement (delay and accuracy); Y; Y; Y; Y; Y; Y; Y; Y. SSB L1 measurement; Y; Y; Y. SSB L3 measurement; Y; Y. CSI-RS RLM; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS BFR; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-RSRP measurement; Y; Y; Y; Y; Y; Y; Y; Y. CSI-RS L1-SINR measurement; Y; Y; Y; Y; Y. CSI-RS L3 measurement; Y; Y; Y. RACH; Y. - Recommended WF - Suggest to agree to define at least following test cases - L1-SRS-RSRP measurement (delay and accuracy) - L1-CLI-RSSI measurement (delay and accuracy) - CSI-RS RLM - CSI-RS BFR - CSI-RS L1-RSRP measurement (delay and accuracy) - Discuss whether to define following test cases - CSI-RS L1-SINR measurement (delay and accuracy) - SSB L1 measurement - SSB L3 measurement - CSI-RS L3 measurement - RACH

Based on the core part discussion in RAN4 previous meetings, there are two objectives been discussed and defined, i.e., RRM requirements for UE-to-UE CLI handling and RRM impacts for SBFD operation.

For RRM impacts for SBFD operation, scenarios including CSI-RS colliding with UL transmission, CSI-RS puncturing in frequency domain in SBFD symbols and CSI-RS distributed over SBFD and non-SBFD symbols are considered in the core part discussion, and impact of SBFD operation for CSI-RS based L1-RSRP/ L1-SINR measurement and CSI-RS based RLM/ BFD/ CBD evaluation are to be defined in the core part, related test cases can be designed in performance part.

Proposal 1: It’s proposed to design the following test cases

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement

Status in the summary [2]:

Issue 3-3-4: Test case scenarios - Proposal 1a (Nokia): - RAN4 to define the test case list for SBFD in SA operation and wait for decision in RAN2 on the support of SBFD with DC. - Proposal 1b (E///): - Define test cases for single cell operation. - Proposal 2 (Nokia, CTC, HW, E///): - Define test cases for FR1 and FR2. - Recommended WF - Suggest to agree on P2, i.e. to define SBFD RRM test cases for both FR1 and FR2. - Wait for RAN2 conclusion, and further discuss whether to define test cases for EN-DC.

In our views, test cases for FR1 and FR2 need to be considered.

Proposal 2: RAN4 to define SBFD RRM test cases for both FR1 and FR2.

- Conclusion

The contribution presents our views on RRM performance requirements for RRM impacts for SBFD operation, with the following proposals:

Proposal 1: It’s proposed to design the following test cases

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement

Proposal 2: RAN4 to define SBFD RRM test cases for both FR1 and FR2.

- References

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509061, Topic summary for [116][218] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #116, 25 – 29 August, 2025.